Changing the Disc in the Background

Some developers have asked for a way to make DRM-600 series drives

change discs without requiring the normal 5-7 second wait.  This

can be done using Pioneer's CDP (Character Device Player) device

driver.  CDP is a character device driver that's built into the

DRD6XX.SYS drivers.  It is enabled with "/C:CDP" in the CONFIG.SYS

device line.

CDP allows the use of Pioneer's mnemonic commands.  Normally, CDP

waits for the return from the command, but this can be disabled by

prefixing the command with "|n"  From DOS, the command

echo 2ZS > cdp

will cause a change to disc 2 (as seen on the magazine).  It will

wait for the return from the drive.  Under DOS, there seems to be

no way to prevent the "|" from being interpreted as a pipe.  You

can, however, copy a file with the command to CDP.

Here's a sample C program:

#include <stdio.h>

main()

{

FILE *handle;

if ((handle = fopen("cdp", "r+w")) == NULL ){

printf("\n Could not open cdp");

exit(1);

}

if (setvbuf(handle,NULL,_IONBF,0)){     /* No buffering */

printf("\setvbuf failed");

}

fprintf(handle, "|n1zs\r");

printf("\n1zs issued");

getch();

fprintf(handle, "|n2zs\r");

printf("\n2zs issued");

getch();

fprintf(handle, "|n3zs\r");

printf("\n3zs issued");

getch();

fprintf(handle, "|n4zs\r");

printf("\n4zs issued");

getch();

fprintf(handle, "|n5zs\r");

printf("\n5zs issued");

getch();

fclose(handle);

}

NOTE:  Each command must be terminated with a carriage return.

Writing a new command before an old command has begun execution

will overwrite the old command.  You can see this if you remove

the getch() calls above.  In this case, the "1zs" will begin

execution.  Then the "2ZS" will be overwritten by "3ZS", "3ZS" by

"4ZS", and "4ZS" will be overwritten by "5ZS".  So you will see

the drive pull in disc 1, and then disc 5.

The CDP functionality is also available through the Pioneer-unique

WRITE CDP and READ CDP SCSI commands.