��������������������������������������������������������������Ŀ

��ı������������ı�����ı�       ��-��������������������������Ĵ TM

��ı�����������ı�������ı�     ��--��     ������������ı�����Ĵ

��ı����������ı� �������ı�   ����ı�      �����������ı  ���Ĵ

��ı���������ı�   �������ı�������ı�     ����ı������ı�����Ĵ

��ı��������ı�     �����ı�   ����ı���������ı     �--� ����Ĵ

��ı�������ı�       ��--��     ��--����������ı     �--�  ���Ĵ

��ı�������ı�       ��-��       ��-�����������ı���� �-�   ��Ĵ

����������������������������������������������������������������

F A X P a k  (tm)

Command Line Utilities

for

Intel Fax Boards

(C) 1989-1992 Jeff Hofstetter

All rights Reserved

1722 Drake Street

Longmont, Colorado 80503

USA

voice: 303.440.7683

fax: 303-651-1161

BBS: 303-772-6778

FAXPak User's Guide

TABLE OF CONTENTS

COPYRIGHT.............................................i

LICENSE...............................................i

WARRANTY..............................................i

INTRODUCTION..........................................1

FAXPak OVERVIEW.......................................2

INSTALLATION..........................................3

COMMAND SYNTAX........................................3

REFERENCE GUIDE.......................................4

FPABORT.............................................5

FPANS...............................................6

FPDEL...............................................7

FPLOG...............................................9

|             FPMON..............................................10

FPNEXT.............................................11

FPRECV.............................................12

FPRETRY............................................14

|             FPSEND.............................................16

Getting the command options from a file.........18

Manually" receiving a fax.......................19

Sending a list of files.........................19

|                Adding a FROM to your @phonelist................20

|                Net SatisFAXtion File Conversions...............20

Sending to a list of addressees.................20

Sending list of files to list of addressees.....20

Looking up phone numbers in a "phone book"......21

Process input file and send faxes...............21

|                Using an external conversion program............23

FPSTAT.............................................24

FPPRINT............................................25

FPQMOVE............................................28

FPVIEW.............................................29

Zoom levels.....................................30

Command keys....................................30

Mousing around..................................30

TECHNICAL INFORMATION................................31

FAXING FILES.........................................31

INTEL SOFTWARE and QUEUES............................31

HOW FAXPak RELATES TO THE INTEL SOFTWARE.............33

CUSTOMER SUPPORT, UPGRADES, BUG REPORTS..............34

|           FAXPak Support Bulletin Board System (BBS)...........34

|           Using multiple fax boards............................35

FAXPak Order Form....................................37

FAXPak User's Guide                                                i

����������������������������������������������������������������������

COPYRIGHT

FAXPak (tm) including all supporting documentation and programs

is Copyright Jeff Hofstetter 1989-1992. All Rights Reserved.

Copying of any of the programs or documentation is prohibited

except that you may make copies of the program disk for archive

purposes.

LICENSE

FAXPak (tm) is licensed for use on a single computer with a fax

|       board installed. A special network license is available for use

|       with Intel's Net SatisFAXtion product. This license allows all

|       users of a Net SatisFAXtion server to use FAXPak Utilities for

|       one low license fee.

WARRANTY

The programs contained in FAXPak are provided "AS IS" without

warranty of any kind, either express or implied, including, but

not limited to, the implied warranties of merchantability and

fitness for a particular purpose. The entire risk related to the

performance of the programs is on you.

In no event will I be liable to you for any damages (including

incidental or consequential damages) arising out of the use of

the included programs.

All that said, within sixty days of paying the license fee, I

will return your license fee (less any shipping charges) for any

reason. Simply return the product with a note requesting a refund

and you will receive it via return mail.

FAXPak User's Guide                                                1

����������������������������������������������������������������������

INTRODUCTION

FAXPak (tm) is a set of programs for use with the Intel

Connection Coprocessor or the Intel SatisFAXtion fax boards. The

software provided with the boards is designed for interactive

sending and receiving of faxes but doesn't handle things like

sending faxes from the DOS command line or from within batch

files. FAXPak provides these capabilities.

As a simple example, typing

"FPSEND file.dat 303-440-7683"

at the DOS command line will fax the file 'file.dat' to the phone

number shown. Other FAXPak commands allow displaying and printing

of the fax log (no 100 entry limit!), deleting entries from the

log, turning on and off the auto answer feature, etc.

FAXPak is available in three different versions:

TRIAL EDITION is the "try before you buy" version and is

available for downloading from various computer bulletin board

systems. Not all functions are available.

STANDARD EDITION is the version that contains basic features

suitable for most users.

ADVANCED EDITION provides all of the features in the Standard

Edition plus support for multiple Intel fax boards

installed in a single computer, a program for viewing PCX and DCX

files, a program to print PCX and DCX files on a Hewlett Packard

Laser printer (or compatible), plus many enhancements to the

standard commands.

The chart on the next page summarizes the major features of

FAXPak and which versions have which features.

2                                                FAXPak User's Guide

����������������������������������������������������������������������

FAXPAK VERSION FEATURE CHART

���������������������������������������������������������������Ŀ

�                     MAJOR FEATURES                � T � S � A �

���������������������������������������������������������������Ĵ

�Send files from DOS command line                   � X � X � X �

�Send multiple files to multiple addressees         � X � X � X �

�Lookup phone numbers from a "phone book"           �   � X � X �

�Send files using multiple fax boards               �   �   � X �

�Display or print the fax log                       � X � X � X �

�Delete faxes from the fax log                      � X � X � X �

�Save received faxes/files to normal DOS files      � X � X � X �

�Turn auto answer feature on and off                � X � X � X �

�List pending faxes                                 � X � X � X �

�Display status of fax board                        � X � X � X �

�Cancel a fax event before it completes             � X � X � X �

�Monitor fax transmission in real time              �   � X � X �

�Perform automatic or manual retries of failed faxes�   � X � X �

�View PCX/DCX files                                 �   �   � X �

�Print PCX/DCX files to HP compatible laser printer � X � X � X �

�Print faxes as they arrive on an HP laser          �   �   � X �

�Send faxes based on info in a message file         �   �   � X �

�����������������������������������������������������������������

T - Trial Edition, S - Standard Edition, A - Advanced Edition

FAXPak OVERVIEW

FAXPak consists of several separate programs. Here is a quick

overview of what they do:

FPABORT   cancel the current fax board operation

FPANS     control the auto answer feature of the fax board.

FPDEL     delete items from the fax log, pending faxes, etc.

FPLOG     display/print the fax log

FPMON     display fax operations in "real time".

FPNEXT    list pending faxes

FPRECV    view the receive log. Copy fax files to DOS files.

FPRETRY   retry faxes that failed.

FPSEND    fax files from the DOS command line.

FPSTAT    display info about the fax board.

FPVIEW    view PCX and DCX files

FAXPak User's Guide                                                3

����������������������������������������������������������������������

FPPRINT   print PCX and DCX files to an HP laser (or compatible).

FPQMOVE   moves pending faxes from one fax board's queue.

Note: not all programs are included with some version of FAXPak.

INSTALLATION

No special installation of FAXPak is necessary! Simply copy all

of the FAXPak files to a directory on your hard disk. For ease of

use, you may want to include the directory where you install

FAXPak in your DOS PATH statement. If you are not familiar with

the DOS PATH, any good DOS book (or even your DOS manual!) will

explain the purpose and how to use the DOS PATH.

COMMAND SYNTAX

All of the FAXPak programs are used in a similar manner:

FPname [arguments] [options]

The square brackets [] indicate optional items. Some FAXPak

programs do not use arguments, others allow multiple arguments.

Here is an example command line:

FPSEND file1.dat file2.dat 1-303-440-7683 -a "Bob Smith"

This shows the FPSEND program command line with three arguments

and one option. The arguments are 'file1.dat', 'file2.dat', and

'1-303-440-7683'. The option is '-a "Bob Smith"'. Options are

always preceded by a dash or slash (- or /). Arguments ALWAYS

come before options.

The command name is separated from the first argument (or option)

by at least one space. All arguments and options are also

separated by at least one space. You can type options in UPPER or

lower case. In the example, '-a' is the same as '-A'. When an

option requires additional information, it may be separated from

the option with one or more spaces OR it may be placed directly

behind the option. For example, the -A option (with FPSEND)

requires that you provide some text following the -A. This text

will be used as the ADDRESSEE name on the fax. You could use the

-A option either of these ways:

-A "Jeff Hofstetter"    or

-A"Jeff Hofstetter"

Note that the quotes are required in this case because the extra

information -- Jeff Hofstetter -- has an embedded space. If the

extra information doesn't have any embedded spaces, you can leave

the quotes off. For example: -A Jeff works okay, but -A Jeff

Hofstetter won't.

4                                                FAXPak User's Guide

����������������������������������������������������������������������

You can get a short list of arguments and options for any of the

FAXPak programs by typing this:

FPprogname -?   (for example: FPSEND -?)

REFERENCE GUIDE

The remainder of this Guide is a reference to each of the FAXPak

commands. Each section describes one command, which versions of

FAXPak include that tool, and how to use all of the arguments and

options.

If a feature is ONLY available in certain versions of FAXPak, the

notation [ xx ] is used where xx represents the version which HAS

the feature. For example [ SE ] would indicate that the feature

is ONLY available in FAXPak Standard Edition OR HIGHER. The

abbreviations used are: SE - Standard Edition and AE - Advanced

Edition.

|       New lines in this manual are marked with a "|" in the left margin.

FAXPak User's Guide                                                5

����������������������������������������������������������������������

COMMAND:FPABORT

VERSIONS: ALL

DESCRIPTION

Cancels the currently sending (or receiving) fax board

operation. If you submit a fax then decide to cancel it, simply

type the command FPABORT. It may take up to 30 seconds for the

operation to actually stop.

USAGE

FPABORT [options]

FPABORT -?

OPTIONS

-Q             Quiet. Don't display copyright notice.   [ AE ]

-O filename    Send program output to 'filename'.       [ AE ]

-B n           Select board n (1, 2, 3, 4).             [ AE ]

EXAMPLES

FPABORT

cancel the current operation (if any)

FPABORT -B 2

cancel the current operation for fax board #2.  [ AE ]

FPABORT -?

display short help screen.

6                                                FAXPak User's Guide

����������������������������������������������������������������������

COMMAND:FPANS

VERSIONS: ALL

DESCRIPTION

Displays and optionally changes the fax board auto answer

feature. The fax board can be set up to automatically answer

the phone line if desired. FPANS allows you to determine the

current auto answer state and change it.

USAGE

FPANS        display the auto answer state

FPANS n      set auto answer to ring 'n'

FPANS -?     display short help screen

OPTIONS

-Q             Quiet. Don't display copyright notice.   [ AE ]

-O filename    Send program output to 'filename'.       [ AE ]

-B n           Select board n (1, 2, 3, 4).             [ AE ]

EXAMPLES

FPANS

display the fax board auto answer state (ON or OFF)

FPANS 1

set fax board to answer on the first ring

FPANS 0

set fax board auto answer to OFF.

FPANS 1 -B 2

set fax board 2 to answer on the first ring.

FAXPak User's Guide                                                7

����������������������������������������������������������������������

COMMAND:FPDEL

VERSIONS: ALL

DESCRIPTION

FPDEL allows you to delete entries from any of the fax queues.

Refer to the TECHNICAL ASPECTS discussion for more information

about queues.

USAGE

FPDEL id1 id2 ...idn [options]

delete events with id numbers specified

FPDEL * [options]

delete ALL events. (Requires the -P, -R, -L, or -A option).

FPDEL -?

display list of options

OPTIONS

-P             Delete from PENDING queue only.

-R             Delete from RECEIVE queue only.

-L             Delete from LOG queue only.

-A             Delete from ALL queues.

-O filename    write output to filename                 [ SE ]

-SC            delete if status = complete              [ AE ]

-SA            delete if status = aborted               [ AE ]

-SE            delete if status = error                 [ AE ]

-N             NO PROMPT before deleting                [ AE ]

-X             Delete FILES associated with event       [ AE ]

-U text        Delete event if user field = text        [ AE ]

-D mm/dd/yyyy  Delete event if BEFORE option date       [ AE ]

-Q             Quiet. Don't display copyright notice.   [ AE ]

-B n           select fax board n (1,2,3,4).            [ AE ]

EXAMPLES

FPDEL 10355

delete event #10355

FPDEL 10355 9802 3422 1099

delete the four events listed

FPDEL * -P

delete ALL events in the PENDING queue. Before actually deleting

anything, a prompt will be displayed to verify the delete.

8                                                FAXPak User's Guide

����������������������������������������������������������������������

FPDEL * -L -N

delete ALL events in the LOG queue with no prompt to verify the

operation. (dangerous!).

FPDEL * -A -N -D 02/01/1991 -B 3

delete ALL events in ALL queues that are dated BEFORE

02/01/1991. Only delete from the queues for fax board #3.

FPDEL * -L -SC -Q -O output.dat

delete all events in the LOG queue that have a status =

complete. Do not display the copyright notice. Write the

command results into a file called 'output.dat'.

FPDEL * -P -U "DENVER" .

delete all events in the PENDING queue that have the word

"DENVER" in the user-defined field.

FPDEL * -L -SC -X

delete all events in the LOG queue that have status = complete.

ALSO, delete the files that were transmitted.

This last example bears a little explanation. Suppose that you

sent a fax to me and sent me a file called 'MESSAGE.TXT'. You

might have typed this to send it:

FPSEND message.txt 1-303-440-7683 -a "Jeff H".

If this transmission actually gets completed successfully and

you type the FPDEL command line shown above, the FPDEL command

will remove the log entry showing that the fax was completed

AND IT WILL DELETE THE FILE 'MESSAGE.TXT'.

FAXPak User's Guide                                                9

����������������������������������������������������������������������

COMMAND: FPLOG

VERSIONS: ALL

DESCRIPTION

FPLOG allows you to view or print the fax log. Unlike the Intel

software, FPLOG will display ALL events in the log, not just

the last 100.

USAGE

FPLOG [options]         display the fax log.

FPLOG -?                display list of options.

OPTIONS

-P n           display 'n' lines per page

-W             display log in WIDE format (132 columns)

-R             display in REVERSE order (newest first)

-V             Verbose. Display file names with log.

-L n           list only the first 'n' entries.

-I             include event ID in log list.

-N             NO report header.

-O filename    write output to a disk file.             [ SE ]

-U "text"      print if user field = text.              [ AE ]

-Q             Quiet. Don't display copyright notice.   [ AE ]

-B n           select fax board n (1,2,3,4)             [ AE ]

EXAMPLES

FPLOG

This displays the log on the screen in standard format.

FPLOG -P 45 -W -V -O PRN

Print the log on the printer in WIDE format, with 45 lines per

page. Include the file names in the listing.

FPLOG -L 5 -R

Display ONLY the newest 5 entries. (The -R option causes the

log to be displayed in reverse order).

FPLOG -U "FEB 1991"

Display entries from the log that contain the text "FEB 1991"

in the user-defined field.

FPLOG -B 2

Display the log for fax board #2.

10                                               FAXPak User's Guide

����������������������������������������������������������������������

COMMAND: FPMON

VERSIONS: Standard, Advanced

DESCRIPTION

FPMON displays information about the current event occurring on

the fax board. You can run FPMON to watch as faxes are sent or

received. After you are done watching, press the ESC key to end

the program.

USAGE

FPMON [options]

FPMON -?

OPTIONS

-E n           'n' is the number of seconds for timing out.

-B n           select fax board n (1, 2, 3, 4).         [ AE ]

EXAMPLES

FPMON

Watch activity on fax board.

FPMON -B 3

Watch activity on fax board #3.

FPMON -E 10

Will cause FPMON to display fax board activity until there are no

more pending faxes AND the board has not had any activity for

10 seconds.

FAXPak User's Guide                                               11

����������������������������������������������������������������������

COMMAND: FPNEXT

VERSIONS: ALL

DESCRIPTION

FPNEXT is used to display the "pending" faxes. The event ID

number is displayed with this list. Thus you can easily cancel

a fax before it goes out by using FPDEL along with the

appropriate id number.

USAGE

FPNEXT [options]

FPNEXT -?

OPTIONS

-A             display ALL pending events. Default is to

display only the NEXT pending event.

-V             Verbose. List file names as well.

|       -G             Show info about group sends.

-O filename    write output to 'filename'.              [ SE ]

-U "text"      display if user field = "text".          [ AE ]

-Q             quiet. Don't display copyright notice.   [ AE ]

-B n           select fax board n (1,2,3,4).            [ AE ]

EXAMPLES

FPNEXT

display the NEXT event to be sent.

FPNEXT -A

display ALL pending events.

FPNEXT -A -V

display ALL pending events. List files names as well.

12                                               FAXPak User's Guide

����������������������������������������������������������������������

COMMAND: FPRECV

VERSIONS: ALL

DESCRIPTION

FPRECV displays information about items in the receive queue.

In addition, you can specify that the received items be

converted to normal DOS files. Please see the TECHNICAL ASPECTS

section for information about how files are stored when

received by the fax board.

USAGE

FPRECV [options]

FPRECV -?

OPTIONS

-M             MOVE faxes to DOS files.

-F             move FILE TRANSFERS to DOS files.

-D Dirname     specify directory name to move files.

-W             OVERWRITE files with the same name.

-V             verbose. List file names.

-X             DELETE event from RECEIVE queue after move.

-O filename    write output to 'filename'.              [ SE ]

-U "text"      move if user field = "text".             [ AE ]

-Q             quiet. Don't display copyright notice.   [ AE ]

-B n           select fax board n (1,2,3,4).            [ AE ]

EXAMPLES

FPRECV

Display a list of items in the receive queue.

FPRECV -V

Display a list of items in the receive queue and list the names

of the files received.

FPRECV -M

move received faxes to DOS files.

FPRECV -M -D C:\MYFILES

move received faxes to DOS files in directory C:\MYFILES.

FPRECV -F -W

move received FILE TRANSFERS to DOS files. If a file with the

same name exists, overwrite the old file.

FAXPak User's Guide                                               13

����������������������������������������������������������������������

FPRECV -F -M -D C:\NEWFILES -X

move faxes and file transfers to normal DOS files in directory

C:\NEWFILES. After moving the files, REMOVE the event from the

receive queue.

NOTES

When FPRECV moves faxes and files to normal DOS files (see the

-M and -F options), a DOS file name is required. In the case of

received faxes, FPRECV generates a unique name in the format

nnnnnnnn.DCX. 'nnnnnnnn' is 8 digits. The file extension is

always DCX. When received file transfers are to be moved,

FPRECV uses the file name provided by the sender. If a file

with the same name already exists, FPRECV will either OVERWRITE

the old file (-W option), or will create a unique name from the

original file name. It does this by changing the LAST LETTER of

the file name (not the file extension). As an example, if you

receive a file called MYFILE.TXT, request FPRECV to move the

file, and a file with that name already exists, FPRECV will

save the file with the name MYFILA.TXT. Note that it changed

the E to an A.

14                                               FAXPak User's Guide

����������������������������������������������������������������������

COMMAND: FPRETRY

VERSIONS: Standard, Advanced

DESCRIPTION

FPRETRY scans the fax log, locates faxes that did not complete,

and retries them.

USAGE

FPRETRY [options]

FPRETRY id1 id2 ...id10

FPRETRY -?

OPTIONS

-A             automatic mode.

-AQ            automatic QUIET mode (no log display).

-E             include cancelled events as eligible.

-M n           set maximum retries to 'n'.

-G             GLOBAL. Retry all eligible events one time.

-I n           set retry Interval to 'n' seconds.

-L filename    Log results in file 'filename'.

-U "text"      retry events with user field =  "text".  [ AE ]

-B n           select fax board n (1,2,3,4).            [ AE ]

EXAMPLES

FPRETRY

Display events "eligible" to be re-tried. This display will

include the event id # which can then be used to re-try

specific events.

FPRETRY 5044 8023 6023

Retry the events shown. They will be retried immediately (or as

soon as possible).

FPRETRY -G -M 5

Perform a GLOBAL retry. Do not retry an event more than 5

times. A global retry is simply a single pass through the log,

examining each event to see if it should be retried. For

example, if you submit 30 faxes during the day some of them

probably will not get completed. You could at various times in

the day issue a global retry. Each time you do, the log will be

scanned and every event that did not complete will be re-tried.

If any failed events remain after 5 global retries, they will

not be retried again UNLESS you change the max retries setting

(-M). To perform a global retry of all failed events REGARDLESS

of how many times they have been tried set the max retries to 0

(i.e. -M 0).

FAXPak User's Guide                                               15

����������������������������������������������������������������������

FPRETRY -A

Perform an automatic mode retry. Automatic mode continuously

scans the log looking for events to retry. Since the max

retries number was not given on the command line, the default

value of 3 will be used. Since no interval was specified, the

default interval of 120 seconds will be used. FPRETRY will

continue to retry events until all events are completed OR

there are no events left which have not been retried three

times.

FPRETRY -AQ -M 5 -I 300 -L myfile.log

Perform automatic retries (in quiet mode). Set the max retries

to 5, set the retry interval to 5 minutes, write the retry

results in the file myfile.log.

NOTES:

The fax board setup allows you to specify how many dialing

retries should be done. I refer to these dialing retries as

"hardware" retries. They work as follows: the fax board dials

the number and waits some number of seconds to get an answer.

If no answer is received in the allotted time, the fax board

will re-dial the number until it has dialed the number of times

you specify. The problem with these "hardware" retries is that

they occur one after the other with no wait time in between

(other than the normal "wait for an answer" time).

It would be more useful to retry a number after some minutes

have passed. FPRETRY allows this.

FPRETRY has three distinct modes of operation. The first mode

is the "informational" mode. It merely displays failed faxes in

the log that are "eligible" to be re-tried. The "manual" mode,

simply makes one pass through the log and retries all eligible

events one time. The "automatic" mode, continuously monitors

the log and will keep re-trying failed events until all events

complete OR there are no more "eligible" events.

FPRETRY retries "eligible" events. An event is considered

eligible if it meets the following conditions: the event failed

to send successfully and the event has NOT been re-tried the

maximum number of times you allow. Normally an event that is

cancelled by you is not considered eligible, but you can use

the "-E" option to allow cancelled events to be considered.

16                                               FAXPak User's Guide

����������������������������������������������������������������������

COMMAND: FPSEND

VERSIONS: ALL

DESCRIPTION

FPSEND is the command used to submit faxes and file transfers.

Many options are available. Pay particular attention to the

version specifiers [ SE ] and [ AE ] on the various features.

USAGE

NOTE: this command can be used in a number of different

ways. There are full discussions below for some of the

unique uses.

FPSEND filename(s) phonenumber [options]

basic usage.

FPSEND M

manual mode. Receive fax NOW.

FPSEND @inpfile                                         [ SE ]

read arguments and options from a file instead of the

DOS command line.

FPSEND @filename phonenumber [options]                  [ SE ]

send list of files

FPSEND filename(s) @phone [options]                     [ SE ]

send files to a list of addressees.

FPSEND filename(s) ?abbrev [options]                    [ SE ]

look up phone number in a FAXPAK phone book.

FPSEND @filename @phone [options]                       [ SE ]

send a list of files to a list of addressees.

FPSEND filename *deffile [options]                      [ AE ]

process 'filename' according to the instructions in

'deffile'

FPSEND -?

short list of options.

OPTIONS

-D mm/dd/yyyy  specify date to send. Note that 4 digits are

REQUIRED for the year. You can specify partial

dates and the remainder of the day will be

filled in for you. For example, if you specify -

D 3/1, this would be interpreted as 03/01/1991.

FAXPak User's Guide                                               17

����������������������������������������������������������������������

-T hh:mm       specify the time to send. You must use 24 hours time

For example, to send a fax at 1pm, you would use

-T 13:00. To send one at 11:30pm, use -T 23:30

-A "text"      set the addressee (TO) name. The quotes are

REQUIRED. The maximum length for "text" is 31

characters.

-F "text"      specify the FROM name. If no FROM name is specified,

the default sender name from the Intel software

will be used.

-0             send mode zero. This is FAX FINE resolution.

This will result in the best printed results on

the receiving end, but will take longer to

transmit.

-1             send mode 1. FAX STD resolution.

-2             send mode 2. File transfer. This option is only

useful when sending to other Intel fax boards.

-3             send mode 3. This is COVER SHEET ONLY mode. If

you want to send a short message to someone, you

can send them a single page fax using this

option. See below for example.

-W             WIDE text. Normally (when faxing ASCII files)

the text font used allows 80 characters per

line. If you use this option, a smaller,

compressed font will be used. This allows 132

columns across and 88 lines on a standard letter

size page (16cpi and 8lpi).

-G             Creates an output page size of 8.5 x 14 (legal

size). Only useful in FAX modes.

|       -N x+y         Set page length to 'x' inches + 'y' eighths.

|       -K             "kill" the cover page.

|       -X             Delete files sent if fax completes successfully.

|       -XA            Delete files even if fax fails to complete.

-C filename    Filename specified is a file containing plain text.

This text will be used on the cover sheet. This

file should only contain about 36 lines of text.

-L filename    Specify the file to be used as a logo for this

transmission. If you don't specify a filename,

the default logo file will be sent. NOTE: the

logo file must be a PCX file and can only be so

large. Refer to the Intel documentation.

-O filename    write output to 'filename'.              [ SE ]

-V             verbose. Show info about each fax.       [ SE ]

-V 1           verbose. Output comma-delimited info.    [ SE ]

-V 2           verbose. Command-delimited with quotes.  [ SE ]

-V 3           verbose. Output fixed-width columns.     [ SE ]

-Q             Quiet. Don't display copyright.          [ AE ]

-U "text"      Embed "text" in the user-defined field.  [ AE ]

-Y             Convert file(s) using EP2DCX (SFAX only) [ AE ]

|       -Z "cmd"       Convert file(s) using 'cmd' line         [ AE ]

-B n           select fax board n (1,2,3,4).            [ AE ]

EXAMPLES

FPSEND ?

display short list of options.

18                                               FAXPak User's Guide

����������������������������������������������������������������������

FPSEND myfile.dat 440-7683

Fax the file myfile.dat to phone number 440-7683. The following

default values will be used: STD FAX resolution, no TO name,

FROM name will be the Intel default sender name, date and time

will be immediate, the file myfile.dat is assumed to be ASCII

only.

FPSEND file1.dat file2.dat 440-7683 -A "Bob" -T 23:30 -2

send two files to the phone number shown. Put "Bob" into the TO

field. Send at 11:30 pm (-T 23:30). Send as a file transfer (-

2).

FPSEND C:\MYDIR\*.DOC 1-303-440-7683 -a "Susan" -0

Send all .DOC files from directory C:\MYDIR to the phone number

shown. Make the addressee Susan. Send FAX FINE (-0).

FPSEND short.msg 440-7683 -3

This is the COVER SHEET ONLY mode. The file short.msg will be

read and used as the text to be included on the cover page.

FPSEND file.txt 440-7683 -C cover.txt

send file.txt. Use the file 'cover.txt' as the cover sheet

text.

|       FPSEND myfile.txt 1.303.440.7683 -N 5+4

|       This sends 'myfile.txt' and uses a page length of 5.5 inches.

|       (5+4 means 5 inches plus 4/8 of an inch). Using -N only has

|       meaning for ASCII files you send as fax standard or fine.

SPECIAL USAGE: Getting the command options from a file.

Because FPSEND allows so many options and arguments, it may be

possible to exceed the 128 character DOS command line limit. If

this is a problem, you will need to put your command arguments

and options into a disk file. To do this, use the form

FPSEND @filename

The file you specify must be a plain ASCII file with the

arguments and options specified exactly as if you were to type

them. You can put the arguments/options on separate lines or

combine them in any way you like. Here's an example:

C:\MYDIR\*.DOC C:\MYDIR\*.TXT

D:\DIR2\*.PRN

1-303-440-7683 -A "Nelson Foundry"

-F "Jeff Hofstetter"

-2

-D 03/01/1991 -T 23:30

This example shows putting the arguments and options in various

groupings. The only requirement is that the arguments (files to

send and phone number) must come before the options.

FAXPak User's Guide                                               19

����������������������������������������������������������������������

SPECIAL USAGE: "Manually" receiving a fax.

If you share your fax with a regular phone, you may receive

calls that are for your fax machine. If that happens, simply

type (at the DOS prompt):

FPSEND M .

This will cause your fax board to take over the phone line and

attempt to receive a fax on the line.

SPECIAL USAGE: Sending a list of files.

If you frequently send the same list of files, you can prepare

a plain text file containing one file name per line. Then, you

use this form of the FPSEND command:

FPSEND @listfile phonenumber [options]

FPSEND then reads your list of files and sends them as if you

had typed them on the command line. This feature may also be

useful if you are using FPSEND from within another program. You

could use the other program to generate a list of files to

send, then provide FPSEND with this file name.

SPECIAL USAGE: Sending to a list of addressees.

If you need to send the same file (or files) to several

different phone numbers, you can use this form of FPSEND:

FPSEND filename @phonelist [options]

"phonelist" is the file name of a specially prepared plain text

file that contains the phone numbers and names of where to

send. This file must be created with one addressee per line and

must be formatted as follows:

phoneNumber | ToName | time | date

The time and date entries are optional, but the phone number

and to name fields are required. You also MUST use the vertical

bar character "|" to separate the fields. Spaces are removed

from the beginning and end of each field during processing so

you can put spaces in your file to aid in readability. Here is

a short piece of a sample phone input file:

1-303-440-7683 | Jeff Hofstetter

555-4321       | Bill McHale

212-444-7654   | NY Times Editorial Staff

20                                               FAXPak User's Guide

����������������������������������������������������������������������

|    ADDING a "FROM" name to your @phonelist

|

|       A minor change was made to the format of the @phonelist

|       operation to allow a from name to be added. FPSEND has been

|       modified to allow a TO NAME to be used instead of the "time to

|       send" entry. FPSEND distingueshes between the two types of

|       entries by ASSUMING THAT AN ENTRY THAT CONTAINS A ":" IS A TIME

|       TO SEND. An entry without the ":" is assumed to be a FROM NAME.

|       Here's an example:

|

|         1-303-440-7683 | Jeff Hofstetter    *** no TIME or FROM NAME

|         1-303-555-1234 | Mike Smith | Allen Brown  *** FROM NAME

|         1-303-555-1234 | Mike Smith | 23:30        *** TIME TO SEND

|

|

|    Special Usage: Net SatisFAXtion File Conversions         [ AE ]

|

|       Intel's Net SatisFAXtion product performs two additional file

|       conversions not done by the non-network product. FPSEND will

|       now identify these two file types by file extension as follows:

|

|          .PCL will identify a file as being an HP PCL4 file.

|          .FX  will identify a file as an EPSON FX85 file.

|

|       For example:

|

|           FPSEND MYFILE.PCL ...

|

|       Will mark "myfile.pcl" as an HP PCL 4 file when it is submitted

|       to the Net SatisFAXtion CASMGR function.

|

|           FPSEND MYFILE.FX ...

|

|       will mark the file as an EPSON FX85 file type.

|

|       When Net SatisFAXtion faxes these files, it will translate the

|       file before faxing it.

SPECIAL USAGE: Sending list of files to list of addressees.

You can combine the above two special uses to allow sending a

list of files to a list of addressees like this:

FPSEND @filelist @phonelist [options]

The same file rules apply as above.

FAXPak User's Guide                                               21

����������������������������������������������������������������������

SPECIAL USAGE: Looking up phone numbers in a "phone book".

If you have a number of places that you frequently send faxes,

you can create a special file that contains the names and phone

numbers plus an abbreviation to use to "look up" the correct

number. This file MUST BE CALLED FAXPAK.PH and must either be

in the current directory (when FPSEND is run) or be locatable

via the DOS PATH. This file also must be formatted in a special

way which is described below. First, here is the proper usage:

FPSEND files ?abbrev [options]

The 'abbrev' must follow the ? with no spaces. The abbreviation

is looked up in FAXPAK.PH. If it is found, the fax is transmitted,

otherwise and error message is displayed. The search method used

to locate your abbreviation is very simple. First, upper and lower

case are ignored. Second, the FIRST entry that matches the

abbreviation you provide will be used. This would mean that if

your FAXPAK.PH file as entries for "John Smith" and for "John

Jones", if you provide the abbreviation "John", then FPSEND will

match the first "John" that it finds in the file.

FAXPAK.PH must be formatted as a plain text file with one line

for each entry as follows:

abbrev | phone number | To name

Here's a short sample:

me      | 421-0331 | M - E Engineers, Inc.

ghp     | 421-6655 | Gerald H. Phipps, Inc.

bobw    | 440-7683 | Bob Wilson

Spaces at the beginning and end of each field are removed so

you can spread things out in your file to make it more

readable.

SPECIAL USAGE: Process input file and send faxes.

If you have a plain text file containing numerous individual

messages containing a fax number, you can provide FPSEND with

the name of that file plus the name of a "definition" file that

describes how to process the file. FPSEND can then process your

message file and send a fax for each message in the file.

As usual, there are some rules that must be followed. But

first, the form of the command is:

FPSEND msgfile *definition [options]

The asterisk (*) is REQUIRED. 'msgfile' is the file name of the

plain text file containing the messages. 'definition' is the

file name of the "definition" file that controls how FPSEND

processes the file.

22                                               FAXPak User's Guide

����������������������������������������������������������������������

DEFINITION FILE.

As usual, the definition file is a plain text file consisting

of one or more lines. Each line is formatted as follows:

Keyword = value

The valid keywords are

MsgStart   = text

FromField  = text

ToField    = text

PhoneField = text

MaxDigits  = text

FileExt    = text

The only REQUIRED entry is the "PhoneField=" one.

FPSEND uses these entries to control how the message file is

divided into individual messages (MsgStart=) and how to fill in

the normal From, To and Phone number entries on the fax. The

MaxDigits= entry allows you to control whether certain types of

phone calls are permitted based on the number of digits in the

phone number (more later). The "FileExt=" entry allows you to

specify the file extension that will be used on the temporary

files that are created that contain the individual messages.

This may allow easier system administration.

Here's a sample message file that will be used in the rest of

this section:

==============================================================

02-15-91

Msg#: 10954

TO:         Bob Olson

FROM:       Jeff Hofstetter

SUBJECT:    :: 440-5555

Bob, give me a call when you get back!

==============================================================

02-16-91

Msg: 10955

TO:         Groucho Marx

FROM:       Harpo

SUBJECT:    :: 1-212-555-9876

HONK, HONK.

<< END OF SAMPLE MESSAGE FILE >>

FAXPak User's Guide                                               23

����������������������������������������������������������������������

For this particular message file, the definition file would look

like this:

MsgStart   = ==========

FromField  = FROM:

ToField    = TO:

PhoneField = SUBJECT:    ::

MaxDigits  = 7

FileExt    = FTM

Note the "PhoneField =" entry. It specifies the text "SUBJECT:

::". What FPSEND will do is scan each line in the message file

and when it finds a line that STARTS WITH the text "SUBJECT:

::", it will then consider everything AFTER the text to be the

phone number to use. This same method is used to locate all of

the fax information.

The MaxDigits entry can be used to control access to long

distance calling. By setting MaxDigits to 7 (as in the example

above), only LOCAL calls (in the U.S.) would be permitted. Using

8 digits would allow LOCAL and INTRA-AREA calls (e.g. 1-555-5555)

to go through.  If a phone number fails the MaxDigits test, the

fax is not generated.

In the example message file, the fax from Harpo would not be sent

since it fails the MaxDigits test.

Other important issues, when matching the definition entries,

upper and lower case are treated the same BUT any blank spaces

included in your definition file must be matched exactly. In

these next few example, spaces are shown as _ for clarity.

PhoneField = "SUBJECT:____::" will NOT match the following:

SUBJECT:__:: 555-1234 -- not enough spaces.

|    Using an External Program to Convert Files before Faxing [ AE ]

|

|       The new -Z option allows you to specify the name of an external

|       program you have that can convert files from one format into a

|       DCX format suitable for faxing by an Intel fax board.

|

|       When you use the -Z option you specify the name of the external

|       program, any required command options, plus two special place

|       holders form the input and output file names.

|

|       For example: (assumes the external program name is CONVERT.EXE)

|

|           FPSEND myfile.pcl 1.303.440.7683 -z "CONVERT $I $O /Q"

|

|       The "$I" holds the place of the "input" file name and the "$O"

|       for the output file name. Note that FPSEND creates the output file

|       name. The quote marks are required and any other text is passed to

|       the external program (like the /Q shown above). You can place the

|       $I and $O tokens anywhere they are required for your external

|       program.

FAXPak User's Guide                                               24

����������������������������������������������������������������������

COMMAND: FPSTAT

VERSIONS: ALL

DESCRIPTION

FPSTAT displays a screen of information about your fax board

and some other useful information.

USAGE

FPSTAT [options]

OPTIONS

-B n           select fax board n (1,2,3,4).            [ AE ]

EXAMPLES

FPSTAT

display a screen of fax board status information.

FPSTAT -B 3

display info about fax board 3.

25                                               FAXPak User's Guide

����������������������������������������������������������������������

COMMAND: FPPRINT

VERSIONS: ALL

DESCRIPTION:

Prints PCX or DCX files to a Hewlett Packard (or close

compatible) laser printer.

Prints faxes as they are received by the fax board to a Hewlett

Packard laser printer. [ Advanced Edition only ]

USAGE:

FPPRINT filename.DCX [options]

FPPRINT *.PCX [options]

FPPRINT -a [options]                                    [ AE ]

FPPRINT -?  -- display short information screen.

OPTIONS:

-I             display info about files (doesn't print).

-F n           first page to print = 'n'                [ SE ]

-L n           last page to print = 'n'                 [ SE ]

-O device      send output to 'device'                  [ SE ]

-O filename    send output to filename                  [ SE ]

-N             do NOT scale pages                       [ SE ]

-Q             Quiet. Don't display copyright info      [ AE ]

-P DL          printer options. d=duplex, l=legal       [ AE ]

-A             auto print incoming faxes.               [ AE ]

<< following options for use with -A only >>

-B nnnn        print faxes from boards n, n, n, n       [ AE ]

e.g.: -b 13 only prints faxes received on boards

1 and 3.

-G filename    loG results in filename                  [ AE ]

-X             delete faxes after printing.             [ AE ]

-M dirname     save faxes to directory 'dirname' after  [ AE ]

printing.

-M:n dirname   save faxes for board 'n' to directory    [ AE ]

dirname after printing.

-O:n device    send output for board 'n' to device.     [ AE ]

-E             exit to DOS after all existing faxes     [ AE ]

have been printed.

EXAMPLES:

FPPRINT myfile.dcx.

prints the specified file to LPT1 which is assumed to be an HP

compatible laser printer.

FAXPak User's Guide                                               26

����������������������������������������������������������������������

FPPRINT *.DCX.

prints all DCX files in the current directory

FPPRINT *.DCX -i

display info about each file (length and number of pages).

FPPRINT myfile.DCX -f 1 -l 1

print ONLY page one of the file.

FPPRINT myfile.DCX -o LPT2

print to LPT2 instead of LPT1

FPPRINT myfile.DCX -o myfile.pcl

print to a disk file instead of LPT1.

FPPRINT myfile.DCX -n

print the file but don't scale it. It will print approx 2/3

size if no scaling is done. However, it will print faster.

FPPRINT *.DCX -p dl

print all the .DCX files in the current directory. Print on

both sides of the page (this works ONLY on HP printers that

have this capability!). Use LEGAL SIZE paper if the fax is

longer than 11 inches. Note: This feature is probably only

useful on printers that have more than one paper feed tray.

FPPRINT issues page size commands to the printer based on the

actual size of the fax. If the printer receives a request to

print on a page size that is not currently loaded, it will stop

printing, display a message, and wait for user interaction.

FPPRINT -a -x

print faxes as they are received. After the fax is printed,

delete the fax files. This process will run until you stop it.

FPPRINT -a -b 23 -o:2 LPT1 -o:3 -LPT2

print faxes as they are received on boards 2 and 3 only. Print

faxes from board 2 to LPT1. Print faxes from board 3 to LPT2.

FPPRINT -a -e

If there are any received faxes, print them, then exit to DOS.

FPPRINT -a -m C:\FAXES

print faxes as they are received. Save the faxes as DCX files

in the directory C:\FAXES.

NOTES:

Most important to remember is that FPPRINT works ONLY with

Hewlett Packard laser printers. It should work with printers

that emulate HPs, but this has not been tested. You must have

adequate printer memory to print graphics pages. Often, the

minimum memory is not adequate. If you get printer errors, it

may mean that you need more memory.

27                                               FAXPak User's Guide

����������������������������������������������������������������������

Paper sizes. The common laser printers can use either letter

(8.5x11) or legal (8.5x14) paper. FPPRINT can use either size

but there are some other considerations. As each fax is

received, it is examined to determine the size of the fax

image. If the fax was transmitted in letter size, then FPPRINT

will send printer commands to select that size paper. If the

fax was sent legal size, FPPRINT will send commands to the

printer to select legal size IF YOU INCLUDE THE "-P L" option.

If you don't include this option, then commands are sent to

select letter size. The fax will be printed on 8.5x11 paper

with the extra text from the legal size page printed on a

second sheet.

Another possible situation occurs when someone with a fax

machine TAPES several pages together and faxes it as one long

page. When this happens, the fax is received as a single page

of many inches. FPPRINT splits this type documents into pages

(either letter size or legal size if you use "-P L").

Printing on both sides of the page. If your printer supports

two-sided printing (called duplex), you can turn this feature

on by using the "-P D" option with FPPRINT. In this case, a

print "job" is considered to be all of the pages from a single

fax. Each job is started on a new piece of paper.

Printer not ready. Laser printers are often slow to feed to the

next page, if you get "Not ready...abort retry..." errors, you

can instruct DOS not to report these errors and keep trying by

typing this (at the DOS prompt):

MODE LPT1,,P

Change LPT1 to match your printer port.

FAXPak User's Guide                                               28

����������������������������������������������������������������������

COMMAND: FPQMOVE

VERSIONS: Advanced

DESCRIPTION:

Allows you to move faxes from the pending queue of one board to

another (or to others).

USAGE:

FPQMOVE srcQ# destQ# [destQ# destQ#] [options]

OPTIONS:

-N             move only 'n' faxes to the destination

-Q             Quiet. Don't display copyright info.

EXAMPLES:

FPQMOVE 1 3

move all pending faxes from board 1 to board 3

FPQMOVE 1 2 3 4

move all pending faxes from board 1 to boards 2, 3, and 4. The

faxes will be divided among the 3 boards more or less evenly.

FPQMOVE 1 2 -n 10

move only 10 faxes from board 1 to 2.

29                                               FAXPak User's Guide

����������������������������������������������������������������������

COMMAND: FPVIEW

VERSIONS: Advanced

DESCRIPTION:

Allows viewing of PCX and DCX fax files. NOTE! FPVIEW only can

view PCX/DCX files that are "fax" files. The requirements (in

general) are: black and white image, 200 dot per inch

resolution, video type 640x200. Many programs create PCX files

but MOST are not designed for use with the Intel fax boards.

Only files designed for these fax boards may be viewed with

FPVIEW.

USAGE:

FPVIEW file(s) [options]

OPTIONS:

-A             Advanced mode. No menus no MOUSE!

-M             Don't use mouse even if one is found.

-Zn            Set initial zoom scale to 'n'.

-Vc            Force video mode to 'c' (C=CGA, E=EGA, V=VGA,

H=HERC).

EXAMPLES:

FPVIEW myfile.DCX

View the file shown.

FPVIEW myfile.DCX -A

View the file but in ADVANCED mode. No menu is displayed and

the mouse will not be used.

FPVIEW *.DCX -Z5

View all of the DCX files (one at a time!). Start viewing at

zoom level 5.

NOTES

FPVIEW uses expanded memory (if available) or the disk. If you

have little or no expanded memory, you will find that FPVIEW is

slow at displaying files. A future version may help this

problem.

Zoom levels. FPVIEW can display a fax page at 5 zoom levels.

Level 5 is the largest display and will display one dot for

each dot in the fax. Zoom level 1 is the smallest display and

FAXPak User's Guide                                               30

����������������������������������������������������������������������

will display an entire page on the screen. This generally

results in an unreadable display!

Command keys. Once a fax is displayed, the following keys may

be used:

PGUP           move toward top of page

PGDN           move toward bottom of page

Ctl+PGUP       display previous page (if any)

Ctl+PGDN       display next page (if any).

Ctl+RightArrow scroll toward right edge of page.

Ctl+LeftArrow  scroll toward left edge of page.

Q              quit displaying this fax and display the next

one (if any).

+              zoom IN closer to page

-              zoom OUT from page.

1              set zoom factor to 1

2              set zoom factor to 2

3,4,5          set zoom factor to 3, 4, or 5.

ESC            exit from FPVIEW.

F1             display short help screen.

Mousing around. For those so inclined, a crude mouse menu is

available at the top of the screen. Just click on the desired

function.

31                                               FAXPak User's Guide

����������������������������������������������������������������������

TECHNICAL INFORMATION

The following information is not required to use FAXPak, but it

may be helpful is solving problems that develop or in just better

understanding how your fax board does what it does.

FAXING FILES

The Intel boards can FAX only certain types of files: ASCII, PCX,

or DCX files. Trying to fax other types of files will result in

strange results on the receiving fax machine! Note, however, that

you can transfer ANY type of file from one Intel fax board to

another. This includes word processing files, executable

programs, graphics files, etc.

ASCII files are "plain text" files which contain only the normal

alpha-numeric characters plus the extended IBM characters (the

line drawing characters are acceptable). Each line in the ASCII

file should be ended with a carriage return/line feed pair. The

Intel fax process will automatically break pages as necessary AND

if it finds a form feed character (decimal 12) in the file. Only

PORTRAIT pages can be sent. Note that you can send a page in

compressed text (132 columns) if you specify that option.

PCX files are graphics images that conform to certain standards.

In general, the file must be created at 200x200 dots per inch

resolution and MUST be black and white.

DCX files are multiple PCX files "glued" together into one big

file. When an Intel fax board receives a fax (not a file

transfer!), the file is saved as a DCX file.

The SatisFAXtion board comes with software that can convert a

file which contains EPSON FX-85 printer control information to a

DCX file. This facility allows some flexibility in faxing

documents that contain different fonts and character attributes.

See the Intel documentation for additional information.

Certain third party software allows converting files from one

format to another which may be useful depending on your

requirements. The key factor in faxing is that you may ONLY fax

the three types of files listed above. If you wish to fax a

document that is NOT one of the three, you must use a program to

convert it!

INTEL SOFTWARE and QUEUES

When you install the Intel software, you perform a setup

operation which copies the software from the installation

diskettes to your hard disk. You also specify which directory to

use. In the case of the Connection CoProcessor (CCP), the default

directory is C:\CONNECT. The SatisFAXtion (SFAX) setup default

directory is C:\FAX.

FAXPak User's Guide                                               32

����������������������������������������������������������������������t

The Intel installation creates some directories underneath this

main directory:

CCP: C:\CONNECT\QUEUE, C:\CONNECT\FONTS, C:\CONNECT\MAIL

SFAX:C:\FAX\QUEUE, C:\FAX\FONTS, C:\FAX\INBOX, C:\FAX\OUTBOX

The primary Intel fax software is CCAM.EXE for the CCP or

CASMGR.EXE for the SFAX. This program is ordinarily placed in

your AUTOEXEC.BAT so that it automatically runs each time your

start your computer. Its job is to manage the hardware activities

in the background while your computer does other things. It

automatically receives any incoming faxes (if you have set the

board to answer automatically) plus it maintains a list of faxes

that are waiting to be sent. When the fax board finishes sending

one fax, the CASMGR feeds the next fax to the hardware when the

time is right.

In the terminology of the fax board, the CASMGR manages 'events'.

An event is the sending or receiving of a fax. It manages these

events by creating and maintaining three lists of events (called

queues): a PENDING queue (sometimes called the TASK queue), a

RECEIVE queue, and a LOG queue. The PENDING queue maintains a

list of events that are waiting to be transmitted. When the

scheduled time for an event rolls around, CASMGR submits the

event to the hardware. The RECEIVE queue is used to maintain a

list of received events. The LOG queue is a "history" list. Every

event will eventually result in an entry in the LOG queue. When

you send a fax, it first gets placed in the PENDING queue. When

it is submitted to the hardware, it is removed from the PENDING

queue. When the fax is completed - either successfully or un-

successfully, a record of the event gets put into the LOG queue.

When a fax is received, it gets put into the RECEIVE queue AND

into the LOG queue.

Entries in the RECEIVE queue and the LOG queue remain until you

take some action to remove them. You can use the Intel software

(CONNECT or FAX) to remove them, or you can use FAXPak. It is

important to realize that as these queues become "filled up", the

operation of the software becomes more and more sluggish. For

this reason, it is good practice to remove entries from the

queues as soon as practical. You can print out the log for future

reference then clear it. Received faxes should be saved to normal

DOS files (see the discussion below about received faxes) then

the entry in the receive queue should be deleted.

When a fax is received by the fax board, it saves the received

information in the receive queue. At this point, you can only use

the Intel software to view the received information. By using

FAXPak or the Intel software, you can move this information into

a normal DOS file. Received faxes that are moved to normal DOS

files get saved as a "DCX" file. Received file transfers get

saved in whatever format they were sent. For example, if someone

sends you a word processing document called LETTER.DOC, when you

save it, it will be saved just as it was sent. You may be able to

re-name the file when you save it if you use the Intel software.

33                                               FAXPak User's Guide

����������������������������������������������������������������������

FAXPak saves files using their original file name unless the name

already exists. In that case, you can specify that the old file

be overwritten, or FAXPak will create a name from the original

name but modified slightly.

HOW FAXPak RELATES TO THE INTEL SOFTWARE

In general terms, FAXPak does not directly access the Intel fax

board. Instead it issues requests to the Intel software (CASMGR

or CCAM) for specific functions. For example, the FAXPak program

FPSEND is used to "submit" faxes. FPSEND does not actually

transmit the fax, instead it builds an information file

containing the names of the files to be sent, the phone number,

etc. It then issues a request to the Intel software to actually

transmit the fax described in the information file.

FAXPak User's Guide                                               34

����������������������������������������������������������������������

CUSTOMER SUPPORT, UPGRADES, BUG REPORTS

Registered users of FAXPak may obtain technical assistance by

calling 303.440.7683 during normal business hours (USA Mountain

Time Zone).

You may also send a fax (or fax board file transfer) 24 hours a

day to 303-651-1161.

Registered users will be notified of new versions of FAXPak and

will be eligible to upgrade at reduced rates. Upgrades from the

Standard Edition to the Advanced Edition can be made for just the

difference is cost between the two.

If you discover any bugs please notify me via phone or fax. Every

effort will be made to solve the problem quickly and a fix

provided via FAX DELIVERY.

FAXPak Support Bulletin Board System (BBS)

The FAXPak BBS operates 24 hours a day at 303-772-6778. You can

leave technical questions there, download files, and contact other

users. An Intel 14.4EX modem can handle modem baud rates from 1200

to 14,400 (v.32bis).

35                                               FAXPak User's Guide

����������������������������������������������������������������������

|   USING MULTIPLE FAX BOARDS IN A PC

|

|       Intel provides information about installing multiple Intel fax

|       cards in a single PC. At the time of this writing, information

|       was available for the Connection Coprocessor and the "Classic"

|       SatisFAXtion card. This information was NOT YET AVAILABLE for the

|       newer model 200/400 SatisFAXtions. Contact Intel customer service

|       for the latest information. You MUST obtain the multiple board

|       instructions from Intel before you can use FAXPak with multiple

|       boards!

|

|       The software provided with your fax board (FAX.EXE, FAXPOP, etc)

|       will NOT work with multiple boards. But since most multi-board

|       faxing is a high volume, command line operation, FAXPak Utilities

|       fills the need.

|

|

|    HARDWARE REQUIREMENTS

|

|       In general you will want to have a 386-based computer or better.

|       The faster the better! Although you can probably run the fax

|       process in only 640k of memory, you will find things go MUCH

|       BETTER if you have enough memory to set up a good size disk

|       cache. Thus, 4 megabytes of RAM is a realistic minimum.

|

|       You should not have any other "strange" devices in the PC to

|       minimize the possibilities for interrupt conflicts, etc.

|

|       The PC MUST be dedicated to the fax process. Trying to use

|       multiple boards implies that you have a lot of faxing to do and

|       attempting to use that PC for other functions (while you are

|       faxing) simply increases your risk of problems.

|

|       You will want a FAST hard disk probably no smaller than 40

|       megabytes (these days, 80 mb is reasonable). This will depend on

|       your volume of faxes and the size of the files you will be

|       faxing.

|

|

|    DISK CACHE

|

|       Get yourself a good disk cache program. There are quite a few

|       available and most of the well-know ones are perfectly

|       acceptable. Some examples: Norton disk cache, PC Tools cache,

|       Hyperdisk, Microsoft SmartDrive, and the list goes on. It is very

|       wise to be sure that you use the NEWEST version of any cache you

|       choose. For example, Microsoft has been providing SmartDrive for

|       a number of years. The version provided with Windows 3.1 is a

|       very good cache. Earlier versions (like the one provided with DOS

|       5.0) are not so good.

FAXPak User's Guide                                               36

����������������������������������������������������������������������

|    KEEPING THINGS GOING

|

|       If you do high volume faxing, you are putting considerable stress

|       on your PC's resources. The hard disk is especially hard hit

|       (thus the reason for the disk cache). With four fax cards

|       chugging away, your PC is asked to handle lots of interrupts,

|       disk accesses, etc. On "marginal" equipment, you will probably

|       experience various kinds of problems. Use the best PC you can!

|

|       The fax "queues" hold information about faxes waiting to be sent

|       and log information about the results of faxes. Each fax that you

|       send creates (at least) one disk file in the queue sub-directory

|       on your hard disk. If you let these queue files accumulate, the

|       entire fax process starts to bog down as the fax software has to

|       wade through all of the files to get to the ones it needs.

|

|       You will have a much smoother operation if you can limit the

|       number of files in these queues. In general, you do this by only

|       "submitting" to the fax boards a "reasonable" number of faxes at

|       a time. You then wait for these to finish before submitting more.

|       Of course, the big question is "what is reasonable". No hard and

|       fast rules! My personal opinion is that you should try to submit

|       less than 100 faxes at a time to any one board.

|

|       After the faxes have been "sent", they are recorded in the log

|       queue. You should also attempt to keep this log queue as clear as

|       possible. This means that you should run FPLOG and save the

|       output to a disk file to record the fax results permanently. Then

|       you should use FPDEL to delete the files in the log queue.

|

|       You can use DOS BATCH language to write batch scripts that will

|       break your faxing up into manageable chunks.

|

|

|    DE-FRAGMENTING YOU HARD DISK

|

|       With all of the disk activity caused by the fax process, your

|       hard disk will become fragmented fairly quickly. Invest in a good

|       disk de-fragmenter program and run it frequently. Again, there

|       are lots of this type program available.

|

|

|    WHEN THINGS GO WRONG

|

|       Using multiple boards to do high-volume faxing is NOT a start it

|       and forget it operation. You will need to monitor the process

|       closely because there will be problems. Something will happen

|       that causes your faxes to quit being sent, some strange equipment

|       interaction will cause a high number of failed faxes, etc.

|

|       If you find that your faxes just don't get sent any more, your

|       queue files may be corrupted. If that happens, all you can do is

|       DELETE ALL OF THE FILES in your QUEUE directories and re-boot the

|       PC. Any pending faxes will have to be re-submitted.

FAXPak Order Form

Send order and make checks payable to

Jeff Hofstetter

FAXPak Utilities

1722 Drake Street

Longmont, CO 80503

Ship to

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

phone number _____________________________________

Qty  Item Description                          Amount

---  --------------------------------------  ----------

___  FAXPak Standard Edition   ($39.00)      __________

___  FAXPak Advanced Edition   ($79.00)      __________

___  FAXPak Advanced Edition Network license

(per Net SatisFAXtion server) ($199.00) __________

___  FAXPak NetShare           ($79.00)      __________

(Colorado residents add sales tax)      __________

___  Shipping/handling per item ($2.50)      __________

___  non-USA shipping per item  ($7.50)      __________

Total Order                             __________

Disk size: [ ] 3.5 inch   [ ] 5.25 inch