--------------------------------------------

DOSMODEM

--------------------------------------------

Donavon Kuhn                     Jon Niedfeldt

4518 Beau Monde Drive #202       1595 Waldorth Ct

Lisle, IL  60532                 Wheaton, IL  60187

ID0007 (Gene Plantz's BBS)

--------------------------------------------

You may use these programs for whatever purpose you like.

Distribute them freely and include them with other packages.

I'm not even going to lower myself by asking for donations

like some other people do.

--------------------------------------------

o DTR allows you to raise and lower DTR on any of the active

communications ports.  Use the form:

DTR [COM1:|COM2:|COM3:|COM4:] ON|OFF

The default is COM1:.  DTR must be active for your modem

to accept commands.

o AT allows you to send commands to your Smartmodem.  Make

sure to set the baud rate on the serial port selected

with the DOS command MODE.  The default settings for each

communications port are 2400,N,8,1.  Use the form:

AT [COM1:|COM2:|COM3:|COM4:] command string

The default is COM1:.  Ex:  'AT Z' will cause the modem

to reset.  Also note you will not see the 'OK' response

from the modem.

o The following batch file will cause the modem attached to

COM1: to reset.

DTR ON

AT Z

DTR OFF

o The following batch file will call Sportsline here in Chicago

(so you can check up on the Cubbies).

DTR ON

AT DT9761313;

Later the command: 'AT H0' will hang up the phone.