CD SHARE

CD ROM NETWORK SHARING SOFTWARE

Version 1.1 User's Manual

Digital Solutions Pty. Ltd.

P.O. Box 178

Margate

QLD, Australia. 4019

Ph. 07 8831851 Intl. 617 8831852

Fax 07 2831217 Intl. 617 2831217

TABLE OF CONTENTS

CONTENTS                                                    PAGE

1.0 BACKGROUND                                               1

1.1 THE CD-ROM SOFTWARE INTERFACE                        1

1.2 CD-SHARE - WHAT IT IS AND WHY WE NEED IT             2

2.0 CONFIGURING A CD-SERVER                                  5

3.0 CONFIGURING A CD-WORKSTATION                             5

4.0 COMMAND LINE SWITCHES                                    9

4.1 CD MANAGER COMMAND LINE SWITCHES                     9

4.2 CD REDIRECTOR COMMAND LINE                          11

4.3 CD-SERVER COMMAND LINE SWITCHES                     11

5.0 CACHING YOUR CD-ROM DRIVES - CD-CACHE                   13

6.0 ADVANCED TOPICS                                         15

6.1 MEMORY USAGE                                        15

6.2 OPTIMIZING CD-SERVE PARAMETERS                      16

FOR A NON-DEDICATED SERVER

6.3 OPTIMIZING CD-CACHE PARAMETERS                      17

6.4 RUNNING CD-SERVE AND CD-CACHE IN DEDICATED MODE     19

7.0 QUESTIONS AND ANSWERS                                   20

Page 1

1.0 BACKGROUND

1.1 THE CD-ROM SOFTWARE INTERFACE

When you purchase a CD-ROM drive  and controller card for your PC

you will usually also receive a disk containing a software device

driver  for the  drive and  a program  written by  MicroSoft Corp

called the "MS-DOS CD-ROM Extensions" (MSCDEX.EXE).

Device drivers are programs that  are loaded by including them in

a line  in the CONFIG.SYS  file on  your DOS boot  disk. A device

driver is a software program  with a standard DOS interface which

allows DOS  to talk  to hardware  devices which  it knows nothing

about. Although you don't see it DOS actually talks to your disks

via an internal device driver !

The device driver  supplied with your  CD-ROM drive(s) allows DOS

to interface  to the  CD-ROM drive  without actually  knowing the

hardware specifics of the drive. This is how DOS can interface to

various  different  brands  of  drives  with  different  hardware

interfaces -  it simply  talks via  the device  driver which does

know about the hardware.

There are two different classes of device drivers - character and

block device drivers. An example of a character device driver are

the internal DOS  devices named CON (your  console) and PRN (your

LPT1) since they work a character at a time. Your disk drives are

block  devices since  they transfer  data to  and from  memory in

blocks (e.g. a typical  cluster (block) size for  a disk might be

2K bytes.  Character device drivers  and addressed  by name (e.g.

CON,  PRN,  COM1,  etc) whereas  block  type  device  drivers are

allocated a drive letter by DOS in the range A: to Z:.

A single device driver can control a number of drives, in fact up

to 255. Since DOS  only allows block device  drive names A: to Z:

this limits the practical total number of drives to a total of 26

minus  any  physical  hard  and   floppy  disk  drives  that  are

installed.  If  you  had  two  floppy  disks  and  one  hard disk

installed (A;, B: and C:) and a device driver named 'DEVICED' was

installed  controlling 3  block devices  the  system would  be as

follows :

����� A:

�

����� B:

�

����� C:

DOS�����Ĵ   �����������Ŀ    ����� E:

�   �           �    �

���Ĵ  DEVICED  ���������� F:

�           �    �

�������������    ����� G:

Fig. 1

Page 2

DOS automatically  gives block  devices the  next available drive

letter(s) when the  device driver is  installed in the config.sys

file.

If CD-ROM's were normal block device that would be the end of the

story, however a single  CD-ROM can hold a  massive 550 MBytes of

information  - far  more than  DOS  was designed  to manage  on a

single block device. To  overcome this limitation MicroSoft wrote

some extensions  to DOS known  as the  "MS-DOS CD-ROM Extensions"

(MSCDEX). MSCDEX  uses the networking  drive redirection features

built into DOS versions  3.1 and later to  create a logical drive

for each  drive attached to  each device driver  specified on the

MSCDEX command  line when the  extensions are  started. MSCDEX is

not a device driver, it is  a "Terminate and Stay Resident" (TSR)

program which runs  and returns to  DOS but remains  in memory to

it's work in the background.

So that CD-ROM device drivers won't  be installed by DOS as block

devices  the  device  driver  header  tells  DOS  that  they  are

character  devices  (even  though  they  aren't).  CD-ROM  device

drivers are block device drivers  that appear to DOS as character

devices but  have a  special signature  so that  MSCDEX and other

software wishing to talk to the  CD-ROM via the device driver can

recognize them as  CD-ROM devices. If  our previous example (Fig.

1)  of DOS  with a  three  drive device  driver were  replaced by

MSCDEX and a  device driver (CDDRIVER)  driving three CD-ROMS the

layout would now be as follows :

����� A:

�

����� B:

�

����� C:

DOS�����Ĵ   ����������Ŀ  ����������Ŀ  ����� E:

�   �          �  �          �  �

���Ĵ  MSCDEX  ��Ĵ CDDRIVER �������� F:

�          �  �          �  �

������������  ������������  ����� G:

Fig. 2

When MSCDEX or any other software calls a CD-ROM device driver it

specifies which drive (or unit) it wishes to access by specifying

a number beginning with zero. It  figure 2 the logical drives E:,

F: and G: would be drives 0,1 and 2 to CDDRIVER.

1.2 CD-SHARE - WHAT IT IS AND WHY WE NEED IT

As well as making CD-ROM's appear as normal (large capacity) disk

drives,   MSCDEX  supplies   a  number   of  other   services  to

applications  software  that   calls  MSCDEX  directly.  Although

networks such as ArtiSoft's LANtastic can redirect a CD-ROM drive

so that it  appears as an  extra drive on  a network workstation,

Page 3

CD-ROM software that  requires MSCDEX to run  won't work. To make

matters  even   worse  some   CD-ROM  software   bypasses  MSCDEX

completely and talks directly to the CD-ROM device driver.

This is where  CD-SHARE comes in.  The CD-SHARE software contains

CD-Server  and CDWORK/CD-Redirector.  CDServer  is a  TSR program

which is run on the network  computer which has the CD-ROM drives

fitted  and  the  appropriate  CD-ROM  device  drivers installed.

CDWORK.SYS and CD-Redirector are run on workstations which wishes

to access CD-ROM  devices on CD-Servers.  CDWORK.SYS appears as a

local CD-ROM device driver, however it passes and device requests

onto  CD-Redirector  which in  turn  passes the  requests  to the

appropriate CD-Server for processing.  The results of the request

are passed back to CD-Redirector/CDWORK.SYS  and then back to the

application  which  called   CDWORK.SYS  .  CDWORK.SYS  therefore

appears to  applications programs  (including MSCDEX)  as a local

CD-ROM device driver.

The CD-Manager program  allows you to alter  the number of drives

that CDWORK.SYS appears to be controlling, and to connect each of

those  logical drives  to  a CD-ROM  drive  on a  network machine

running CD-SERVER.

Because CD-ROM  seek times  and data  transfer rates  are so slow

sharing a single CD-ROM amongst  a number of users usually causes

applications on workstations to run unacceptably slow. Because of

this the CD-SHARE software suite  also includes CD-CACHE which is

a  CD-ROM  caching   program  written  specifically   to  run  in

conjunction with CD-SHARE. When  CD-CACHE running, when a request

to  read CD-ROM  data occurs  CD-CACHE  checks to  see if  it has

already been read. If  not then the data  is read from the CD-ROM

and  stored in  the cache  as well  as being  passed back  to the

calling software. If a  request for data which  is already in the

cache occurs  then the data  is returned directly  from the cache

without any access  at all to the  CD-ROM. The CD-CACHE algorithm

ensures that the most frequently read data (directories, etc) are

always kept in the cache,  this improves response times even when

only one users is running  programs from the CD-ROM. CD-CACHE can

use conventional or extended memory up to 32 MBytes.

The networking version of figure 2 using CD-SHARE would appear as

follows :

����� A:

�

����� B:

�

����� C:                                    ��������Ŀ

DOSĴ ��������Ŀ ��������Ŀ �����Ŀ     �����Ŀ �        ���� 0

�Ĵ MSCDEX �Ĵ CDWORK �Ĵ CDR �-----� CDS �ĴCDDRIVER���� 1

���������� ���������� �������  ^  ������� �        ���� 2

�          ����������    ^

Network Connection ���           Drive Nos.����

Fig. 3

Page 4

Where :

1. CDWORK is the CD-Share CD-ROM device driver

2. CDR is the CD-Redirector

3. CDS is the CD-Server

Note that figure three is  a very simple configuration. Each copy

of  CDS which  is started  on a  CD-Server is  started up  with a

unique name and can be attached to multiple device drivers on the

server  computer. Using  CD-Manager (CDM)  each logical  drive on

CDWORK may be connected to any CDServer\Device\Drive. For Example

SERVERS         DEVICE DRIVERS       DRIVES

����������Ŀ      ������������Ŀ  ������  0

+-------� CDSERV-1 ������Ĵ   DRIVER1  ��Ĵ

|       ������������   �  ��������������  ������  1

|                      �  ������������Ŀ  ������  0

|                      ��Ĵ   DRIVER2  ���������  1

|                         ��������������  ������  2

|

|       ����������Ŀ      ������������Ŀ  ������  0

+-------� CDSERV-2 ������Ĵ   DRIVER1  ���������  1

|       ������������      ��������������  ������  2

|

|       ����������Ŀ      ������������Ŀ  ������  0

+-------� CDSERV-3 ������Ĵ   DRIVER1  ��Ĵ

|       ������������   �  ��������������  ������  1

|                      �  ������������Ŀ  ������  0

|                      ��Ĵ   DRIVER2  ���������  1

|                      �  ��������������  ������  2

|                      �  ������������Ŀ

|                      ��Ĵ   DRIVER3  ���������  0

|                         ��������������

|

|

|       WORKSTATION

|

|       DEVICE DRIVER       ATTACHED TO        DRIVES

|

|                         ��CDSERV-1\DRIVER2\2 =  0

|       �����Ŀ ��������Ŀ�

+-------� CDR �Ĵ CDWORK ���CDSERV-3\DRIVER3\0 =  1

������� ���������ٳ

��CDSERV-1\DRIVER1\1 =  2

Fig. 4

Note that since CDWORK simulates a CD-ROM device driver, there is

no need to run MSCDEX on workstations running software that talks

directly  to  the  CD-ROM  device  driver.  Also  CD-Server  is a

non-dedicated server, this means that  CDR and CDWORK may also be

run on the  server which may  then log into  and use devices made

networkable  by  it's  own  copy of  CD-Serve  as  well  as other

CD-Servers.

Page 5

2.0 CONFIGURING A CD-SERVER

1.  Install  the  CD-ROM  drives,  controllers,  etc.  and device

drivers  then test  run  the drives  and  programs as  if the

system were  not to be  networked to make  sure everything is

okay before adding the CD-Server.

2.  If you have run  MSCDEX in step 1,  then re-boot the computer

and DON'T  run MSCDEX. Copy  the CDServer  (CDS.EXE) from the

CD-Share distribution disk onto your system boot disk.

3.  Make  sure your  network  NETBIOS driver  has  been installed

(with LANtastic this is  AILANBIO, LANBIOS, or LANBIOS2). You

won't  need  any  other   networking  software  on  a  CD-ROM

workstation  or  a CD-Server  in  order to  run  the CD-Share

programs.   (You   can   still   run   network  re-directors,

non-dedicated servers, etc. if need be).

4.  Assuming you wish  to allow five logged  in users, have named

your CD-ROM  device drivers  DRIVER1, DRIVER2,  etc (You will

have specified the driver names on the "DEVICE=" line for the

drivers in the  config.sys file. Typical  names take the form

MSCD001,  MSCD002, etc)  and we  wish to  call our  CD server

cdserver1 enter the following command line :

CDS CDSERVER1 device=DRIVER1 device=DRIVER2 verbose users=5

The "device=DeviceName" section  may be repeated  up to eight

times. CD-Serve can handle up  to 32 drives (e.g. four device

drivers  each driving  eight  drives, or  eight  drivers each

driving four drives, one driver driving eight drives and four

drivers driving six  drives, or any  mix). The verbose option

is optional and causes CDS to indicate the names of connected

device  drivers  as they  are  attached to  confirm  that the

command line  was accepted  correctly. The  "device=" section

may  be  abbreviated to  "d="  to  fit more  switches  on the

command line.  CDS has more  command line  switches, they are

covered in the "Advanced Topics" section.

Note  that  after  CDS has  been  run,  all  connected device

drivers are effectively  removed from the  DOS device list so

they can't be accessed by any  other software. If you need to

access  the device  drivers in  order to  gain access  to the

CD-ROM(s)  from software  running locally  on the  server you

will have to configure the server as a workstation as well.

3.0 CONFIGURING A CD-WORKSTATION

1.  Copy  the programs  CDWORK.SYS, CDR.EXE  (CD-Redirector), and

CDM( CD-Manager) onto the workstation disk. CDWORK.SYS should

be in the root directory, CDR.EXE  and CDM.EXE should be in a

directory which is on the PATH.

Page 6

2.  Insert a  line similar  to that  following in  the config.sys

file on the workstation boot disk :

DEVICE=CDWORK.SYS [DEVICE_NAME]

Where DEVICE_NAME is the name that MSCDEX or any other CD-ROM

software  will  use  to  access  the  CD-ROM  driver. Typical

examples might be :

DEVICE=CDWORK.SYS MSCD001

OR

DEVICE=CDWORK.SYS CDWORK

We  will  assume  that  the name  CDWORK  is  used  in future

discussion. Whatever you use, make sure that the name doesn't

conflict  with a  name  you have  given  to any  other device

drivers (e.g. if you are setting up a non-dedicated CD-Server

you will have both CDWORK.SYS  and real CD-ROM device drivers

installed in the config.sys file).

3.  You will now have to  re-boot the computer so that CDWORK.SYS

is installed. CDWORK.SYS will sign on with a line similar to

CDWORK Installed - Copyright Digital Solutions

during the boot sequence.

4.  Make  sure  that   your  network  NETBIOS   driver  has  been

installed. Assuming you wish to call your CD Workstation CDW1

and  you  wish  to  redirect  to  a  maximum  of  two CDROM's

simultaneously enter the following command line :

CDR CDW1 2

The actual syntax for the CDR command line is :

CDR [Station Name] {Maximum Drives}

where []  indicates that  the parameter  is mandatory  and {}

indicates that the switch is optional. If "Maximum Drives" is

not specified  it defaults to  one. This is  all that usually

required since CD-Manager (CDM) can be used to disconnect the

redirected  CD-ROM  drive  from  any  CD  Server  device  and

re-connect to another at will. Also MSCDEX uses an additional

4K of memory for each logical drive on a device driver.

5.  You can now use  CDM to connect to  devices and drives on any

CD-Servers that are on  line. To log into  and connect to our

CDSERVER1, DEVICE1,  DRIVE 0 that  we set up  in the previous

section, enter the following command line :

CDM V IN:ON +:0=\\CDSERVER1\DEVICE1\0

Page 7

After  CDM connects  output similar  to the  following should

appear on the screen :

C:\CDM V IN:ON +:0=\\CDSERVER1\DEVICE1\0

DEVICE1            :           Units            =           2

CD - REDIRECTOR STATUS....

Connected                                           servers..

\\CDSERVER1

Drives..

Drive 0: Connected to \\CDSERVER1\DEVICE1\0

Drive 1: NOT Connected

Total Servers  Attached :  1

Total Drives   Attached :  1

Maximum Drives Attached :  2

Activity  Indicator is  : ON at Row 1, Column 80

C:\

Fig. 5

Let's analyze  the command line  to set what  we've done. The

first switch  V{erbose} tells  CDW to  work in  verbose mode.

This is why the line

DEVICE1 : Units = 2

appeared,  and why  the CD  Redirector  status was  listed in

full.

The next switch

IN{dicator}:ON

tells CDM  to turn on  the CD  Redirector activity indicator.

This  causes  a  small activity  indicator  to  flash  on the

workstation as CD-ROM activity is in progress. By default the

position of this is  row one, column 80  on the screen (upper

right corner), see "CDM Command Line Switches" for details on

how to change this position.

The next switch requires a little more description :

+:0=\\cdserver1\device1\0

The + option tells CDM to  add a new connection to CDWORK, in

Page 8

this case to a  drive 0, on a device  named "device1" on a CD

Server named "cdserver1". The actual syntax is as follows :

+:[Local Drive Number]=\\[Server Name]\[Device Name]\Drive

Where a device named "Device Name" be attached to a CD Server

named "Server Name" and that  device must have a drive number

"Drive".

If you  now run the  Microsoft extensions  on the workstation

specifying  the  name  that you  gave  CDWORK.SYS  (we'll use

CDWORK as the name) you should get the following response :

C:\mscdex /d:cdwork

MSCDEX Version 2.10

Copyright  (C) Microsoft  Corp. 1986,  1987, 1988,  1989. All

rights reserved.

Drive D: = Driver CDWORK unit 0

Drive E: = Driver CDWORK unit 1

C:\

Fig. 6

Note that your drive letters  may not necessarily begin at D:

depending upon the drives installed  in your computer and any

redirected network drives you may have.

If you now type :

DIR D:

You should now get a  directory listing of the root directory

of the disk in your CD-ROM drive.

Note that if you now type in

DIR E:

You will  get an error  message from MSCDEX  stating that the

drive  is  not  ready,  this is  because  drive  E:  which is

attached to DRIVE  1 on CDWORK  is not connected  to a server

device (see Fig. 5).

Now that  MSCDEX has  been run  you may  refer to  the CDWORK

drives by their drive number, or by their MSCDEX drive letter

when using CDM.

The - CDM command line switch allows a drive connection to be

removed e.g.

CDM -:0, or CDM -:D

Would cause CDWORK drive 0 (Fig. 5) to be disconnected, or

Page 9

CDM LOGOUT:CDSERVER1

Would cause all connections to CDSERVER1 to be removed.

4.0 COMMAND LINE SWITCHES

4.1 CD MANAGER COMMAND LINE SWITCHES

The syntax of the CD Manager (CDM.EXE) command line is as follows

CDM {switches}

Where {switches} represents  any of the  following switches, each

separated by a space. Note  that any characters between {} braces

are optional, the {}  characters do NOT form  part of the switch.

Square braces  [] indicate  that the words  between the  [] are a

name  and should  not be  entered as  shown. i.e.  The name  of a

Device, CD Server, or Drive  Number should be substituted for the

[name]  entry  (the  []  characters  are  not  required.  e.g. To

indicate  that a  device name  is required  we might  say [Device

Name], when CDM is  actually run we might  use DEVICE1 instead of

[Device Name]. In all cases a  = character may be substituted for

the : character in the following list.

Switch        Function

V{erbose}     Causes verbose  operation of  CDM. CDM  will report

various activities  as they are  processed and will

cause CDM  to show  a CD  Redirector status listing

after all other switches have been processed.

Sh{ow}        Causes CDM  to display  CD Redirector  status after

all other switches have been processed.

Logo{ut}:     Syntax

Logout:Server_Name

Causes all  CDWORK drive  connections to  CD Server

"Server_Name" to  be deleted and  the connection to

"Server_Name" to be removed.

+:            Syntax

+:[Local Drive]=\\[ServerName]\[DeviceName]\[Drive]

Attaches the CDWORK drive  [Local Drive] to [Drive]

on a device named [DeviceName] on a CD Server named

[ServerName].  For [Local  Drive] either  the local

CDWORK drive number or the drive letter may be used

if MSCDEX has been run.  [Drive] may be replaced by

ALL if you  want all of  the drives on [DeviceName]

to  be  attached to  CDWORK  beginning  with CDWORK

Page 10

drive [Local Drive]. e.g.  to Connect local drive 1

to   [Drive]   2   on   [DeviceName]   MSCD001   on

[ServerName] CDSERVER1  the following  switch would

be used

+:1=\\CDSERVER1\MSCD001\2

If  MSCDEX  had  been run  and  Local  Drive number

corresponded to drive E:  then the following syntax

could also have been used

+:E=\\CDSERVER1\MSCD001\2

-:            Syntax

-:[Local Drive]

Disconnects  [Local Drive]  from  the device  it is

currently attached to. [Local  Drive] may be either

the  CDWORK  logical  drive  (unit)  number  or the

MSCDEX drive name if MSCDEX has been run.

Dr{ives}      Syntax

Drives:[New Number of Local Drives]

Alters the number of logical drives that CDWORK can

redirect.  This  switch  will  not  be  accepted if

MSCDEX has already been run.

In{dicator}   Syntax

Indicator:[On|Off]{@Row,Column}

Turns the  CD Redirector  activity indicator  On or

Off depending upon whether  On, or Off is specified

after the : . If On is specified you may optionally

specify the screen coordinates by following On with

@Row,Column ; where valid ranges for row and column

are 1 to  25 and 1 to  80 respectively. The default

position for the indicator  is 1,80. For example to

turn  the indicator  on at  row  25, column  80 the

following switch would be used

in:on@25,80

or to turn it off

in:off

Note  that  if  in:on is  specified  without  a new

position,  then  the previous  position  setting is

retained.

Page 11

4.2 CD REDIRECTOR COMMAND LINE

The CD  Redirector (CDR.EXE) command  line must  contain a unique

name for  the CD Redirector  session and can  contain an optional

number of Local Drives specifier for CDWORK.

CDR [Unique Name] {Number of Local Drives}

e.g.

CDR STN1 1

The  number of  local drives  defaults to  one if  not specified.

Since MSCDEX uses an extra 4K of ram per local drive, a figure of

1 is recommended  if MSCDEX is  to be used.  Since it is unlikely

that  a  workstation  will  be  accessing  more  than  one CD-ROM

simultaneously this  does not impose  a limitation as  CDM can be

used to alter the actual CD-ROM drive that the single local drive

is connected to at any time.

Note that  providing MSCDEX hasn't  been run CDM.EXE  can also be

used to alter the number of local drives.

CDR can be  logged into a maximum  of five CD  Servers at any one

time. Local Drives can range from one to twenty five.

4.3 CD-SERVER COMMAND LINE SWITCHES

The syntax of the CD Server (CDS.EXE) command line is as follows

CDS [Server_Name] device=device_name {switches}

[Server_Name] a  unique name  which is  used to  represent the CD

Server on the network. At least one device must be specified.

Where {switches} represents  any of the  following switches, each

separated by a space. Note  that any characters between {} braces

are optional, the {}  characters do NOT form  part of the switch.

Square braces  [] indicate  that the words  between the  [] are a

name  and should  not be  entered as  shown. i.e.  The name  of a

Device, CD Server, or Drive  Number should be substituted for the

[name]  entry  (the  []  characters  are  not  required.  e.g. To

indicate  that a  device name  is required  we might  say [Device

Name], when CDS is  actually run we might  use DEVICE1 instead of

[Device Name]. In all cases a  = character may be substituted for

the : character in the following list.

Switch        Function

D{evice}      Syntax

Device:[Device_Name]

Page 12

where [Device_Name] is the named of a CD-ROM device

driver already installed on the server. This switch

may be used a maximum  of eight times connecting to

a  maximum of  32  drives per  server  (each device

driver can  handle multiple  drives -  the total of

these drives must not exceed 32). Device drivers to

which CDS attaches are  removed from the DOS device

list  so other  programs can  no longer  access the

device(s).

Us{ers}       Syntax

Users:[Maximum Number of logged in CD users]

Note that  a NETBIOS  session is  required for each

logged  in  user.  CDS  DOES  NOT  check  that your

NETBIOS   can  support   the  number   of  sessions

corresponding  to your  Users specification,  it is

the  responsibility  of the  network  configurer to

ensure  that   enough  netbios   sessions  will  be

available. (Up to 254 with LANtastic)

Bu{ffer}      Syntax

Buffer:[Buffer Size in sectors]

e.g. Buffer:4

Specifies the  size of the  network transfer buffer

used to  pass CD ROM  data to  CD workstations. The

actual  buffer  size  is  approximately  2K  by the

specified buffer  size. The default  Buffer Size is

three  sectors (about  6K). Larger  buffers improve

performance  but  use  more   memory  when  CDS  is

installed leaving  less room for  other programs to

run (This is not a  consideration when running a CD

Server in dedicated mode).  Note that one buffer is

allocated PER network task (see "Tasks")

Ta{sks}       Syntax

Tasks:[Number of Network Tasks]

e.g. Tasks:3

The  network  tasks   figure  determines  how  many

network   CD   requests  the   server   can  handle

simultaneously.   The   default   is   one.   On  a

non-dedicated server the law of diminishing returns

is reached at  a figure of  about five tasks. Three

tasks  is usually  adequate, use  two if  memory is

tight.

5.0 CACHING YOUR CD-ROM DRIVES - CD-CACHE

Page 13

CD-CACHE is  a caching program  specifically designed  to work in

conjunction with CD-Serve on a CD-ROM server. Because of the slow

access times of CD-ROM drives use of CD-CACHE is almost essential

if three or  more than three  users will be  accessing the CD-ROM

simultaneously.  In fact  using  CD-CACHE will  usually  speed up

response times even when only one user is accessing the CD-ROM.

CD-CACHE can use conventional,  extended, expanded, or XMS memory

for caching purposes  CD-CACHE will  cache any accesses to any of

the CD-ROM drives  (i.e. up to  32) that are  being managed by CD

Serve (CDS.EXE).

Like CDS.EXE CD-CACHE (CDC.EXE) is a TSR program. This means that

when you run CDC from the command line it will install itself and

return to the DOS prompt while  remaining in memory to perform CD

caching as necessary. CDC will not install unless CDS has already

been installed.

CDC has a number of command line switches, the two that will most

commonly be used are those that specify the type of memory CDC is

to use for caching  purposes and the amount  of memory to use for

caching.

The syntax of the CD Cache (CDC.EXE) command line is as follows

CDC {switches}

Where {switches} represents  any of the  following switches, each

separated by a space. Note  that any characters between {} braces

are optional, the {}  characters do NOT form  part of the switch.

Square braces  [] indicate  that the words  between the  [] are a

name and should not be entered  as shown. i.e. A number of KBytes

of memory to be used would  be substituted for the [Cache Size in

Kbytes] entry  (the [] characters  are not required.  When CDC is

actually run we might use 1000 instead of [Cache Size in KBytes].

In all cases a = character may be substituted for the : character

in  the  following  list.  Command  line  switches  are  not case

sensitive, i.e. TYPE is the same as Type or typE.

Switch        Function

Type          Syntax

Type:{Conv|Ext|Ems|Xms}

e.g.

TYPE:CONV  means to  use conventional memory (below

the 640K boundary) for caching.

TYPE:EXT for Extended Memory

TYPE:EMS for Expanded memory

TYPE:XMS for XMS memory

Page 14

The default  is Conventional  if no  TYPE switch is

used.

Size          Syntax

Size:[Cache Size in KBytes]

e.g. type:1000 for  a 1MByte cache.  The default is

to use ALL of the specified type of cache memory if

no size switch  is found. Note that  if you use all

of conventional memory as cache it is preferable to

use the Dedicate CDC switch  to cause the CD Server

to  run  dedicated,  this will  always  use  all of

conventional   memory  for   caching  as   well  as

improving performance dramatically  and leaving you

the  option of  unloading CDC  at any  time without

re-booting the  CD Server.  See the  "Dedicate" CDC

switch  for more  detail. If  you run  CD-CACHE not

dedicated and use all available conventional memory

for caching  there will  be NO  MEMORY LEFT  TO RUN

OTHER PROGRAMS.

Ded{icate}    Causes CD Cache  to run the  CD-Server in dedicated

mode.    This   improves    CD-Server   performance

dramatically  (5  to  10  fold)  and  will  add any

remaining  conventional memory  available  after CD

Cache has loaded  to the cache  memory. E.G. if you

have specified  1MByte of extended  memory as cache

and  use the  dedicate switch,  and CD  Cache finds

that 500K  of conventional  memory remain available

after it has  loaded - your  total cache size would

be  1.5MBytes. When  CD Cache  is run  in dedicated

mode pressing the Esc key on the CD-Server keyboard

will unload CD Cache and  return to the DOS command

line,  leaving the  CD  Server as  a non-dedicated,

non-cached  (as  far  as  CD  ROM's  are concerned)

server.

Buff{er Sectors}

Syntax

Buffer:{Buffer Size in Sectors}

When CD-Cache  performs any CD-ROM  reads it ALWAYS

reads the same number  of sectors into cache memory

then returns the requests sectors from the cache to

CD Server. For optimum  performance the Buffer Size

set for CD-Cache  should be a  multiple of that set

for CD  Serve. The  default for  CD Serve  is three

sectors and the default for CD-CACHE is six. As you

can see the  default of six for  CD-Cache will be a

multiple of Buffer  settings for CD  Server of one,

two,  or  three sectors.  The  maximum  buffer size

setting is 25 sectors  (about 64KBytes). The buffer

Page 15

size you choose will be determined by the amount of

conventional and  cache memory  you are  willing to

let CD-Cache  use and  the performance  you require

from the server. See  the "Advanced Topics" section

for  information on  how to  make decisions  on the

best  combinations  of  options  to  select  for CD

Server  and CD  Cache  to optimize  performance for

your application.

An example  command line for  running CD Cache  in dedicated mode

using 2 MBytes of extended memory would be as follows :

CDC Type:Ext Size:2000

6.0 ADVANCED TOPICS

6.1 MEMORY USAGE

The amount  of conventional memory  used by the  various CD Share

programs and MSCDEX after being loaded is listed below.

NOTE: 1K = 1024 Bytes

PROGRAM       MEMORY USED

CDWORK.SYS    < 650 Bytes

CDR.EXE       8 KBytes

CDS.EXE       12K + Users * 106 + Tasks * (BufferSecs*2532 + 225)

e.g. 10 Users, 2 tasks, 5 sectors per buffer =

12K + 10 * 106 + 2 * ( 5 * 2532 + 225)

= 39118 Bytes

= 38.2 K

CDC.EXE       Conventional Cache :

12K + Size of Cache

Other               :

Buffer Size Bytes(BSB) = BufferSectors * 2532

Buffers in Cache (BIC) = CacheSize/BSB

Conventional Memory used = 12K + BIC*34 + BSB

e.g. Cache Size = 2MBytes of Extended Memory

BufferSectors = 6

BSB = 6 * 2532 = 15192 Bytes

BIC = (2000 * 1024)/15192 = 134(.8)

Page 16

Conv Mem Used = 12K + 134*34 + 15192

= 31012 Bytes

= 30.3 K

MSCDEX.EXE    22K + 4K per logical  drive (e.g. one device driver

controlling 2 drives  would use approximately 30K).

This will vary slightly according to the version of

MSCDEX.

6.2 OPTIMIZING CD-SERVE PARAMETERS FOR A NON-DEDICATED SERVER

For a non-dedicated CD  Server we are assuming  that you want the

server to use as  little memory as possible.  As with most things

in real life you rarely get  something for nothing - the more you

prune back  the resources  available to  CD Serve  the poorer the

performance will  be. The trick  is to strike  a compromise which

gives both acceptable memory usage and acceptable performance.

As  you can  see  from the  CDS.EXE  section in  section  6.1 CDS

conventional memory usage is governed by three parameters

1. Users

2. Buffer Sectors

3. Tasks

Obviously you should only specify the number of CD users that you

actually want the CD  Server to be able  to have connected at any

one time.

The "Buffer  Sectors" parameter  defines the  size of  the memory

buffer that CDS  uses to return  CD ROM data  to a CD Redirector.

The larger this parameter  is made (Max 25)  the larger the block

of data that the server can send to the CD Redirector in one hit.

However the maximum figure of 25 sectors amounts to a buffer size

of about 64K and there is  one buffer allocated per network task,

as  you can  see  allocating large  buffers  and tasks  can quite

quickly gobble up huge amounts of memory.

The number of tasks determines  how many CD network requests that

the  CD Server  can  process at  any  one time.  Performance will

improve  as  the  number  of  tasks  is  increased.  The  law  of

diminishing returns  is reached for  a non-dedicated  server at a

figure of about five tasks.

The question then  arises as to  which combination is  best for a

non-dedicated server. The  answer of course  lies in the question

"how much memory do you want left over after CDS has been run and

what  level  of   performance  are  you   going  to  consider  as

satisfactory ?". For minimum memory usage both the Buffer Sectors

and  Tasks should  be reduced  to a  minimum. The  default buffer

sectors setting  is three,  a setting of  two is  probably a good

starting  point if  you  a trying  to  minimize memory  usage and

Page 17

maintain reasonable performance. A Tasks setting of two will give

a  fairly  substantial  improvement  over  the  default  of  one,

improvements will be gradually decreasing  as the number of tasks

is increased through three, four, and five.

With  this  background  experimentation   is  probably  the  best

indicator of what  will be acceptable  for your application. When

evaluating   performance  it   should  be   noted  that   CD  ROM

applications  usually  have  long   periods  of  disk  inactivity

interspersed  with short  bursts  of activity  when  programs and

images are  loaded from  the CD ROM.  A simple  method of gauging

performance is to create a batch file on a number of workstations

which  reads a  large  number of  files  from the  CD-ROM  into a

temporary file  on a hard  disk, deletes the  temporary file then

loops to the beginning - timing  the loop will give an indication

of performance. If you have this  arrangement set up and you wish

to re-boot  the server  in order  to alter  CDS parameters  it is

quite acceptable to simply re-boot the server. Each CD Redirector

will  eventually report  an error  and MSCDEX  will give  you the

options Abort, Retry, or Ignore. After CDS has been re-run on the

CD  Server  simply  select   Retry  and  the  CD-Redirector  will

re-connect to the server and continue.

6.3 OPTIMIZING CD-CACHE PARAMETERS

We will assume here  that for a non-dedicated  CD Server CD Cache

will always  be run  using cache  memory other  than conventional

memory;  since you  would  have to  use  all of  the conventional

memory in order to make  the cache of any use  and if you do this

you might  as well run  the CD  Server in dedicated  mode and get

some real performance improvements !

As you  can see  from the  calculations for  CDC.EXE conventional

memory usage  when using  extended memory,  the additional memory

that CDC uses beyond it's fixed code  and data size is made up of

two components

1. A  single  buffer  the  size   of  which  is  set  by  the

"BufferSectors" command  line switch.  The default setting

for this is 6 sectors or about 15 KBytes.

2. Cache Buffer management  memory which amounts  to 34 bytes

for each time the buffer allocated at step 1 will fit into

the cache memory size.

From step 2  we can see that  for a given  amount of cache memory

the amount of  cache management memory  required increases as the

"Buffer Sectors" setting decreases. In the example calculation of

section 6.1 for 2MByte  of cache memory and  6 sectors per buffer

we used  134 *  34 Bytes  = 4556  bytes for  cache management and

15192 bytes for the buffer  for a total extra conventional memory

usage  of 19748  Bytes. If  we  halved the  buffer size  to three

sectors we could used  269 * 34 =  9146 Bytes for management plus

7596 bytes for buffering  for a total of  16742 bytes. As you can

Page 18

see we have  only reduced the  consumption of conventional memory

by 3K  but we  have more  than doubled  the fragmentation  of the

cache.

As a general rule, the larger  the Buffers per Sector setting the

better as  it allows  the CD-ROM  to read  more information  in a

single disk read thus eliminating a seek from another position on

the  disk  when  sequential sectors  are  read  from  the server.

However a quick glance at some files on a CD-ROM will reveal that

many of  the files  (and the directories  if you  could see their

size) are actually less than 20K in length. This means that since

there is  usually very  little file  fragmentation many  of these

files could be  read with a  single access from  the CD ROM. Note

that although CD-CACHE allocates 2532 bytes per sector so that it

can cope with CD-ROM applications that request sector data in RAW

mode (2532 bytes per sector) most applications read the CD-ROM in

cooked mode which returns  2048 bytes per sector  (2K). Thus a 12

sector buffer would  generally read 24K  bytes per read. Although

CD-CACHE will accept buffer sizes  up to 25 sectors, using values

above 12 decreases  the granularity of the  cache for very little

improvement in performance.

For  optimum  performance the  Buffer  size  should be  set  to a

multiple of the BufferSectors setting  used with CDS. The default

CD-CACHE  buffer size  is  set to  six  sectors, this  is  a good

compromise between conventional memory usage and performance, and

is also a  multiple of the  default CD-SERV buffer  size of three

sectors (Also of 2, and 1 !).

You should choose your buffer  sectors setting with the following

things in mind:

1. If  you  are  caching a  large  number  of  drives running

different  applications,  then  for  a  given  cache  size

smaller buffers  are going to  increase buffer granularity

and therefore more  likely to keep  more of the frequently

used sections of each CD-ROM in the cache.

2. If  you don't  have  much memory  to  use as  cache memory

smaller buffers  are better as  this will  help to improve

the granularity.

3. If you  have plenty of  cache memory in  comparison to the

number  of CD-ROM's  that are  being  cached then  you can

increase the buffer size to improve performance and reduce

CD-ROM  drive wear  and  tear while  still  achieving good

granularity.

4. If you are running CD-CACHE in non-dedicated mode then you

should choose a buffer size which results in an acceptable

compromise   between   conventional    memory   used   and

granularity

Page 19

6.4 RUNNING CD-SERVE AND CD-CACHE IN DEDICATED MODE

If you  want the ultimate  in performance -  dedicate your CD-ROM

Server !

When using CD-CACHE you  can optionally tell it  to run CD Server

in dedicated mode. This will  give an immediate speed improvement

of  usually at  least five  hundred percent  as well  as allowing

CD-CACHE to add  all remaining conventional  memory to it's cache

buffers.

As a bonus  you can unload  CD-CACHE and return  the CD-Server to

non-dedicated/non-cached operation at the  touch of a key without

disturbing  and other  network users  using  the CD-ROM's  on the

server.

To run in dedicated mode  simply include the "Dedicate" switch on

the CD-Cache command line as follows :

CDC Type:Extended Size:2000 Dedicated

CD-Cache will respond with a  message saying that it is installed

and running CD-Serve in dedicated mode.

To  unload  CD-Cache  and   return  to  non-dedicated  CD  Server

operation simply press  the Esc key. Network  users logged in and

using the CD-ROM at this time will notice nothing excepting for a

dramatic decrease in performance.

If a computer is being used specifically as a dedicated CD Server

in order to share multiple CD ROM's with maximum performance then

the  memory  constraints  which  are  important  when  running in

non-dedicated  mode lose  there importance.  The CD  Serve buffer

size should be increased to about  six sectors and tasks to about

five. The  CD-CACHE buffer  size should also  be set  to at least

twelve sectors (you should install plenty of cache memory).

Read sections 6.2 and 6.3 (if you haven't already done so) to get

a good understanding of  the performance consequences of altering

the CD-Serve and CD-Cache buffer sizes.

Page 20

7.0 QUESTIONS AND ANSWERS

Question :    Does  a network  computer need  to be  set up  as a

network  server  or  have  a  redirector  or MSCDEX

installed in order to be used as a CD-Server.

Answer   :    No - All  that is required of  the network in order

to run CD Serve on  a computer is a NETBIOS network

driver.

Question  :   Do I need to run the network redirector on a CD ROM

workstation.

Answer    :   No -  All that  is required  on the  workstation is

that CDWORK.SYS is included and given a device name

in the CONFIG.SYS file on the workstation boot disk

and   that  network   NETBIOS  network   driver  is

installed before running CDR.EXE.

Question  :   Do I need to run MSCDEX on a CD-ROM workstation.

Answer    :   Only  if your  CD-ROM application  requires MSCDEX.

Many CD-ROM applications  install themselves on the

workstation hard disk then  interface to the CD-ROM

directly via the device  driver (CDWORK.SYS in this

case)  -  these applications  usually  only require

MSCDEX for  the initial installation  (i.e. to copy

the  program  files  from the  CD-ROM  to  the hard

disk).

Question  :   Can I  use a  CD-ROM workstation  to perform normal

network   activities   by   running   the   network

redirector.

Answer    :   Yes - the CD Redirector and CDWORK appear simply as

another device  driver to  both the  network and to

DOS.

Question  :   Can I  run CD  Serve in  dedicated or non-dedicated

mode on a normal non-dedicated network server (e.g.

LANtastic)

Answer    :   Yes,  although the  activities  of the  two servers

will serve to  reduce the performance  of both when

running under heavy load conditions.

Question  :   I cannot run my  normal network redirector up after

running CDR and MSCDEX

Answer    :   MSCDEX  is  implemented as  a  network re-director.

Your network redirector sees MSCDEX as a previously

installed   network  redirector   and   refuses  to

install. The solution  is to run  MSCDEX after your

Page 21

network  redirector  (and  before  your  server  if

running the  machine up as  a non-dedicated network

server e.g. LANtastic's SERVER.EXE).

Question  :   After running MSCDEX I  don't have enough memory to

run my CD-ROM program.

Answer    :   MSCDEX uses an  extra 4K of  memory for every drive

on every device driver it is attached to. Make sure

that when running CDR.EXE or CDM.EXE that you don't

set the number of drives supported by CD Redirector

to more than one. You can then use CDM.EXE to alter

this one  logical CD-ROM drive  to different CD-ROM

drives on any CD Servers on the network as and when

required.

Question  :   After I run MSCDEX I get devices with names like

G.\A.

H.\B.

I.\C.

In   my  network   connection  manager's   list  of

redirected   drives   and   I   can't   remove  the

connection.

Answer    :   MSCDEX functions  in exactly  the same  way as your

network redirector  - it  uses the  DOS redirection

functions and  therefore the  CD-ROM logical drives

appear in  the list  of redirected  drives. Because

your network redirector  doesn't recognize these as

it's own  re-directed drives  it won't  alter these

connections.

Page 22