AutoCAD DXF to 3DVector Converter. V0.1

Options:

DXF23DV inputname outputname [-s# -x# -y# -z# -mfilename -u# -v# -w# -l -n -q]

-x  x translation for object (before scale) - can be floating point, +z = up

-y  y translation for object (before scale) - can be floating point, +z = up

-z  z translation for object (before scale) - can be floating point, +z = up

-s  scale factor                            - can be floating point, +z = up

-u  x translation for object (after scale)  - integer only, +y = down

-v  y translation for object (after scale)  - integer only, +y = down

-w  z translation for object (after scale)  - integer only, +y = down

-m  materials list filename (corresponds to layer names)

-l  selective layer processing (only process layers that are found

in materials file)

-n  output true calculated surface normal (otherwise 0,0,0)

-q  negate Y axis

I you have trouble assigning a material to a line it may be because the

layer that the line is on has the shading option.  Lines  do  not  have

surface normals and therefore  cannot  have  the  shading  option  set.

If DXF23DV finds this occurance, it will insert  a default texture that

has no shading texture.  eg:  0,0,colour,0

If your a total knumbskull (like me) and  you  get  ACAD's  co-ordinate

system messed up (like me) you can use the -q option to  negate  the  Y

axis and reverse the orientation of the polygons.

Do not use punctuation in your material names or layer names.

A # sign in the materials means a comment

Example materials file:

# This is a comment

# There must be 5 fields in each assigned texture!!

CONSTANT     0,0,0,colour0,0

SHADED       0,shade,0,colour1,0

SINE         0,wavey,0,colour2,0

BOTHSIDES    both,0,0,colour3,0

BOTHSHADE    both,shade,0,colour4,0

DOUBLESIDED  double,0,0,colour5a,colour5b

DOUBLESHADE  double,shade,shade,colour6a,colour6b

1SHADE2SINE  0,shade,wavey,colour7a,colour7b

1SINE2SHADE  0,wavey,shade,colour8a,colour8b

2SINE        double,wavey,wavey,colour9a,colour9b

SAMPLESIDE   this is,an example,of what,you can,do!

Material names can be as long as ACAD allows!

Currently there is a limit of 3000 points and 2000 surfaces to any DXF file.