============================================================================

CalcuFret v1.0          c1994 Trevor Boicey

============================================================================

ONE LINE DESCRIPTION:

CALCUFRT.ZIP : Computes fret scales for musical instruments.

REQUIREMENTS:

An MSDOS compatible machine or emulator. Very little memory is

needed, I can't imagine any machine that WON'T run this program.

A printer is also handy for making hard copies of data to

keep beside you in the workshop. Lots easier than doing

precision woodworking on your computer table.

WHAT THIS PROGRAM DOES:

This program is for building and working with guitars, basses,

and other musical instruments.

You provide the program with data about the instrument you

are working with, and the program will provide the precise

locations of all the frets for that particular instrument.

CALCUFRT can be used to find the fret slots to build a

guitar from scratch, or to install a new fretboard, or

for fret crowning, or any time you need to mathematically

locate the frets of an instrument accurately.

This program is also useful for first time fretless or

orchestral musicians. With the help of this program, you

can "find" the notes on your instrument, where you can

then place stickers or nail polish to learn where the

notes are.

Although most luthiers will only use 12 frets per octave for

"western" music, the program can produce charts for any number

of frets per octave, which may be useful to luthiers producing

experimental or traditional instruments.

USING THE PROGRAM:

To run the program, type "CALCUFRT" at the DOS prompt.

The program will bring up a screen with two requesters,

labelled "SCALE LENGTH" and "FRETS PER OCTAVE". This is

all the information that CalcuFret will need to perform

the calculations.

When the program is first run, the FRETS PER OCTAVE will

show 12, and the requester will be open in the SCALE LENGTH

entry. Use the up and down cursor keys to select the

scale length of the instrument you are working with. Common

values for six string guitars are 24-26 inches, for electric

basses, 30-36 inches, etc.

The "SCALE LENGTH" is officially the distance between the

nut and the bridge, before intonation is considered. In the

real world of course, the intonation will necessitate this

distance be a bit longer. But the scale length is this

nut-bridge distance, or equivalently, twice the distance

between the nut and the octave fret. (usually 12th)

For example, to fret an electric bass, you might

desire a 34 inch scale. You would then use the up and down

arrow keys until the highlighted line in the requester was

over 34.

To change the number of FRETS PER OCTAVE, simply press the

right arrow key. The scale requester will close, and show

the scale selected. The FRETS PER OCTAVE requester will

then open, and the desired value can be selected like with

the scale length. You can also use the left/right arrow

keys to toggle between the two requestors until you are

satisifed with both settings.

( Do not confuse frets per octave with total frets. Calcufret

will show the first 48 frets regardless of this setting, which

is more than enough for any instrument I've ever seen. You

need only use the number of frets you wish to install on

your instrument. Therefore for a 22 fret guitar, FRETS PER

OCTAVE must still be at 12, and you would only use the

info provided for the first 22 frets. )

When you are satisfied that both requestors contain the right

information for your instrument, pressing RETURN accepts the

data and will cause CalcuFret to make the calculations on

those values.

OUTPUT SCREEN:

The output screen basically shows the results of all the

calculations. It contains all the data you should need

to fret an instrument.

To make a hard copy of this screen, have your printer

attached and configured normally, and press SHIFT-PrtSc

to print the screen in the usual DOS way.

To Re-Run the program, perhaps to fine-tune the input,

or experiment with new values, press SPACE.

To exit the program, hit ESC.

INTERPRETING THE RESULTS:

The output screen shows three colums of data, each column has

16 frets, for a total of 48 frets. Again, unless you are using

24 frets per octave or more, this is probably way more than

you will need.

Each fret number has two measurements beside it. One is the

width of that fret from the previous one, the other is

the cumulative distance of that fret from the nut.

Example:

Select a scale length of 34 inches, twelve frets per octave.

The output will be similar to:

Fret   Width   Cumul.

1    1.908   1.908

2    1.801   3.709

3    ....etc

Therefore, you would measure 1.908 inches from the nut,

and scribe a line. This will be the first fret.

Next, measure 1.801 inches from the scribed line

where the first fret will be. Scribe a line, this

is fret two. Now measure from the second fret to

the nut, and check that the distance is 3.709 inches.

If so, the fret is correct, you can now move on until all

frets are marked.

LEGAL INFO:

- This program can be used free of charge, and distributed

by most any method, as long as no fee is charged. Feel

free to send this program to FTP sites, BBSs, etc.

All I ask is that the archive is kept together, so that the

program and this doc file are always present.

- This program, as far as I know, works great. The results

it produces collaborate with tables from books on

guitar making that I own.

*BUT*

I am in no way responsible for the accuracy of this information.

Use at your own risk.

OTHER INFO:

- The "SCALE LENGTH" need not be in inches, if you prefer

to work in something else, like centimeters. The results

are in the same units that the input is given in.

If you wanted an instrument with a scale of 90cm, just

enter 90 as scale length, and read the results in

centimeters as well.

- I can be reached via email at "ae718@freenet.carleton.ca"

for at least a while. It's mid 1994 right now, if you get

this much later than that, I don't know where I'll be.

Feel free to email me about this program. If you can't get

it to work, tell me a bit about your machine, I'll see what

I can do.

If you have anything in mind that should be added to this

program, by all means get in touch. I'd like to expand it

to use non-tempered fret scales or other uncommon scales

as well, but frankly I don't have the mathematical info

myself. If you do, email me, and I'll make CalcuFret be

able to work with these as well.

- Enjoy!

============================================================================