Modem Test Pack Manual

Record Full Modem Configuration

Test Capabilities

Release 2.01

New England Research & Design Software

Modem Test Pack is a trademark of New England Research & Development Software.  All materials (c) Copyright, New England Research&Design Software, 1992-1994.
  All Rights Reserved.  Patents Pending.

New England Research&Design Software

Gulf & Merrifield Roads

West Chesterfield, New Hampshire

Voice/Fax: (603) 256-8050   Modem: (603) 256-8055

The following provision does not apply to any country or locality where such provisions are inconsistent with local law. New England Research & Development Software provides this publication on an "as-is" basis without warranty of any kind, either express or implied, including, but not limited to, the implied warranties of merchantability or fitness for a particular purpose.  Some localities do not allow disclaimer of express or implied warranties for certain transactions, therefore this statement may not apply to you.

# Introduction

## OverviewXE "Overview"§

The programs that display modem profiles and status lack a great deal in completeness and in the ability to record information in a fashion that permits later comparisons.  Modem Test Pack (MTP) fills this gap.  MTP also permits link testing and verified disconnects.

## Intended AudienceXE "intended audience"§

The intended audience of this manual is for professional support personnel or PC mavens who are not offput by terms like Baud and Data Terminal Emulation.  If you are the type who will, when faced with a Com-link problem, groan and pull out the modem's technical manual to start working through AT codes and data to find the problem; then you will likely get a lot of useful and some new information from Modem Test Pack, the program and the manual.  If the previous sentences seem to you to be just a lot of technoid-gibberish (not necessarily an unhealthy viewpoint), please discard or pass on this product.

End users who are having problems with their modems should consult with a trained support person to resolve them.

## System RequirementsXE "system requirements"§

IBM - PC or any compatible computer, 8088 through 80486, Pentium.

MS-DOS 3.3 or later or any fully compatible DOS or NOS environment.

Hayes compatible modem (that is AT command set).

This version does NOT support networked modems (Int 14), or modems under Windows.  If you are diagnosing modem problems it is best to get Windows, anti-virus TSRs, and other TSRs off the system.  Windows is not very good for high speed communications anyway, use DOS for serious communications.

## Files XE "files "§on the Product Diskette or ZIP

The files on your product disk and their purposes are listed in the file FILES.TXT.

## LicensingXE "licensing"§

See the file LICENSE.TXT for licensing information.  DO NOT COPY THE PRODUCT AROUND.

## Release ControlXE "release control"§ Information

See file WHATIS.NEW for release control information.

## InstallationXE "installation"§

Please see the file INSTALL.TXT for installation instructions.

µIntroduction	i

Overview	i

Intended Audience	i

System Requirements	i

Files on the Product Diskette or ZIP	i

Licensing	ii

Release Control Information	ii

Installation	ii

Chapter 1 - Syntax for MA	1

1.1	Modem Audit (MA) Syntax	1

1.2	The MODEM AUDIT Process	2

1.3	MA Displays	2

1.4	MA File Output	2

1.5	MA Input File	3

Chapter 2 - COMMAND.DAT Input File	4

2.1	File COMMAND.DAT Format	5

2.2	Identify Section	5

2.3	Scan Section	6

2.4	AT Command Section	6

2.5	SReg Section	7

2.6	Identify Bits Section	8

Chapter 3 - The MA Display	10

Chapter 4 - The Output File - COMMAND.LST	11

Chapter 5 - Save Status, P2A	13

5.1	Saving Profiles to ASCII	13

Chapter 6 - A2P ASCII to Profile	14

6.1	Restoring a Profile to a MODEM	14

6.2	A2P Command Syntax	14

6.3	A2P Output	14

Chapter 7 - Submit an AT Command - AT.EXE	16

7.1	AT Command Syntax	16

7.2	Examples	16

Chapter 8 - HangIt - Disconnect Modems	17

8.1	HangIt Process	17

8.2	HANGIT Command Syntax	18

8.3	Reset vs. Power OFF/On	18

Chapter 10 - Stress Test Overview	19

10.1	Four Parts to Modem S/T	19

10.2	Quick Start	20

Chapter 11 - STS - Stress Test Send	21

11.1	The STS Process	21

11.2	STS Command Syntax	22

11.3	STS Display	25

11.4	STS.LOG File	26

11.5	STRESS.TAB File	26

11.6	Compression and STS Patterns	27

11.7	Jitterbugging	28

11.8	Timing and STRESS.TAB Output	28

11.9	Accuracy and Timing Errors	29

11.10	Buffers and Timing	29

Chapter 12 - Analyzing Results - STRESCAL.EXE	30

12.1	STRESCAL Results.	30

12.2	STRESCAL Graphs	32

Chapter 13 - STR - Stress Test Receive	36

13.1	The STR Process	36

13.2	STR Command Syntax	37

13.3	STR Display	38

13.4	STR.LOG File	38

Chapter 14 - Errors and Common Problems	39

14.1	No Response to STS	39

Chapter 15 - Suggestions	40

15.1	Modem Handling	40

15.2	Timeout - AT\T1	40

Chapter 16 - Anomalies	41

16.1	Anomalies	41

Chapter 16 - Questions and Answers	43

16.1	Change Control	43

16.2	Editing a Status File	43

Appendix 1 - Hayes ATI4 Specification	44

# Chapter 1 - Syntax for MA

## 1.1	Modem Audit (MA) Syntax

Modem Audit has one required parameter and two optional ones.

MA com_port [time_delay]  [mdmconfig.fil]

replace:

com_port	with the number of your COM Port (1 through 8).  This release of Modem Audit supports the standard COM assignments:

ISA     IRQ UART

COM1:    4  3F8

COM2:    3  2F8

COM3:    4  3E8

COM4:    3  2E8

Micro Channel

IRQ UART

COM1:    4  3F8

COM2:    3  2F8

COM3:    3 3220

COM4:    3 3228

COM5:    3 4220

COM6:    3 4228

COM7:    3 5220

COM8:    3 5228

For non-standard COM port assignemnts use the DOS environment parameter COMA (for COM Assignment).  Set it to UART Address, IRQ, as in:

SET COMA=3e8,4

You can test to see which COM ports have UARTSXE "UARTS"§ using TESTCOMXE "TESTCOM"§, a program name with many implementations available from many bulletin boards.

If you have MS-DOS 6.0 you may use MSD to establish which COM ports are usable.  There are similar routines from Quarterdeck and other after-market suppliers.

time_delay	with a number from 1 to 99, optionally.  The default is 3, and this is the number of  twentieths of a second to delay before submitting the next command to the modem.  Most modern modems run fine with a 1, but modems using older designs and circuitry may require additional delays.  We have seen some which require 7, particularly in new 486 lap tops with built in modems.

mdmconfig.fil
The name of the modem configuration file.  The default is COMMAND.DAT.

Note: Be sure that your modems are Hayes command compatibleXE "Hayes command compatible"§ and that echo responses is ON, E1; responses are verbose, V1; and extended verbose is also set if you have it, \V2.

DTE Note: Some PCMIA and portable modems cannot handle 2,400 Baud, which is the default for AT, HANGIT, MA, and A2P.  These will timeout on the very first command.  You may select another DTE for these by setting Environment parameter DTE to the DTE rate, as in:

SET DTE=19200

Use one of 2400, 4800, 9600, 19200, 38400, 57600 and 115200.  You may have to use the MODE command from DOS to set the port as well.

## 1.2	The MODEM AUDIT ProcessXE "MODEM AUDIT Process"§

When Modem Audit is run, Modem Audit:

1.	Opens the indicated com port.  If the com port is not available or dedicated to a mouse or printer, the results may be unpredictable, usually Modem Audit just gives up after about 20 seconds.

2.	Sets the Modem to V1 Q0, which in English is "send back verbal responses", like OK or ERROR instead of 0 and 131.

3.	Submits an AT&V command to have the modem list its version of the status or profile.

4.	Runs the ATIx commands 0 through 8 and analyses the responses.

5.	Queries S registers 0 to 128.  Those that return a valid response have their data listed.

6.	Creates a short list of odd items in the S registers.

7.	Submits Commands AT0 through ATZ and analyses the responses.

8.	Submits the 0 through Z commands for patterns AT%x, AT&x, AT\x, AT+x, AT%x,  AT-x, and AT"x, and analyses the responses.

9.	Lists the expected commands that returned "ERROR"

10.	Summarizes the features of this modem indicated by the ATI4 responses.

## 1.3	MA Displays

MA displays messages to the screen, primarily to inform the user that work is progressing.  Actually there is a lot of compressed information in the displays, but the real output is sent to a file.

## 1.4	MA File Output

The output goes to a file named COMMAND.LST.  Any previous file of the same name in the default directory (or through certain append modes) will be replaced.

COMMAND.LST has everything that we were able to deduce about your modem, from S register interpretations to hidden S registers, to commands, their explanations and the commands that worked but were not documented, hidden commands.  We will go into details on the output file later.

## 1.5	MA Input File

MA expects an input file, COMMAND.DAT.  COMMAND.DAT has the information about what commands to expect the modem to honor.  We will describe this file in detail in Chapter 2.

# Chapter 2 - COMMAND.DAT Input File

COMMAND.DAT is an ASCII input file that defines the expected modem commands and registers' usage.  You might ask why we need the flexibility of an editable input file for modem specifications, well consider ...

Firstly, there are no supervised standards for how computers talk to modems.  There is a de facto command set, the AT commands, that came out with the first SmartModems from Hayes which broke open the inexpensive, reliable and usable modems in the early '80s.  No one in their right mind would try to market a modem for PCs without the AT command set.  However, every time there is an advance in data compression or error correction the leading manufacturers jump right in with proprietary code and algorithms, and, naturally new and different AT commands.

Secondly, much of what the AT commands do is manipulate one byte (8 bits) registers called S Registers.  Again no standards.  Again progress forces diversification, followed by later re-standardization ... but later isn't now.

Thirdly, there is an hodge podge of possible methods of communications from the computer to the modem, and from the modem to another modem (V.22, V.32bis, DCD, LAPM, MNP, CTR, DSR, ...).  We fondly hope that this manual will clarify some of these for you.  Anyway, not all modems support all the features.

There is a feature called fallback, where an supposedly advanced modern modem can talk to an older, slower one using a more primitive protocol.  The rude truth is that fallback often simply bloody doesn't work even with modems from supposedly quality manufacturers like Intel, which had (perhaps still has) great difficulty getting some of their high speed modems to answer 2400 baud calls.  Furthermore, the fallback methods are not standardized.  For example after failing to connect at 14,400 Baud LAPM, one modem might fallback to 12000 Baud, reliable, MNP-5 while another model or one from another manufacturer is falling back to 14400 baud.

This is due to fallback design dating back ten years.  When there were only two ways to communicate, 1200 Baud and 300 Baud, it seemed OK to try 1200 and if that didn't work fallback to 300.  Nowadays there are 42 different standard ways of communicating - 7 speeds, reliable or not, MNP or LAPM, and more if one counts manufacturer specific protocols.  Instead of fallback, modems should connect at 2400 baud, normal and start negotiating what improvements they can make in the protocols.

Fourthly, a recent survey, from IDC of Waltham Mass., as we recall, indicated that there are more than 400 firms manufacturing modems, many with several models.  This is particularly amusing because the modem manufacturers had agreed to leave an identifying byte for the "manufacturer code".  Unfortunately a byte only holds up to 255 codes, meaning that there are at least 145 manufacturers incorrectly identified.

With all this technological change and lack of standardization, confusion is impossible to eliminate.  Even Intel has given up as indicated by the fact that their own program, TESTCOM, running on one of their own modems does not lay out what the S Register bits are for, it just says "(Varies)" for some of the bits.

Sorry to put you through all that, but now you know the need for COMMAND.DAT.

## 2.1	File COMMAND.DAT Format

COMMAND.DAT is an ASCII file.  An piece of one follows:

ATA  A Manual Answer

ATB    Select CCITT or Bell protocols for 300/1200 baud

ATD  A Dial out

ATE    Echo modes

ATH    Hang up (On Hook)

ATI    Product codes

ATL    Speaker volume control

·   ·   ·

You may use any ASCII editor such as EDIT or BRIEF to edit this file.

Lines starting with a semicolon are comments and otherwise ignored..

Null lines (only a carriage return), are ignored.

There are sections.  The sections have headers, the headers are:

Identify Section

Scan Section

AT Command Section

SReg Section

Identity Bits Section

There are two key phrases.

Dumpport dumps some information about the port.

Scan Sregs 0-128  indicates that Sregs 0 to 128 are to be checked for existence.  The second parameter (128) must be greater than the first(0).  The first should be zero or greater.  The second may be up to 4096.  The two parameters should be separated by a dash and no spaces.

## 2.2	Identify Section

The items in the identify section are submitted to the modem and the results are reported.  The intent of this section is to list the ATI commands.  Line items are just AT commands, up to 40 characters, as in:

identify section

ATI1

ATI2

ATI3

ATI4

ATI5

You may include setup code in this section, such as:

AT&F

You may also include other commands such as AT%L to report on line quality.

## 2.3	Scan Section

The scan section defines series of commands to be checked with the modem.  An example is:

scan section

;23456789012345

at         0-Z

at&        0-Z

at%        0-Z

at\        0-Z

Positions 1 to 6: the template of the command.

Position 12: The start character for the scan.  It does not have to be 0 but it must be lower in the ASCII collating sequence than the end character for the scan.

Position 13: dash, -.

Position 14: The end character for the scan.

For example the line AT      0-Z will submit the commands AT0, AT1, AT2, ... ATZ.  There is nothing sacrosanct about 0-Z.  You may scan, for example A-Z, or B-B.

You may use as many, or as few line items as you wish.

It is important to be aware of the "assumed" attribute in the AT COMMAND SECTION which prevents a command from actually being submitted.  For example you do not really want an ATA submitted, for if it is submitted, the modem will go off-hook.

## 2.4	AT Command Section

This section documents the known commands, a small section is:

AT command section

;2345678901234567890...

ATA  A Manual Immediate Answer

ATB    Select protocols 1>Bell 212A for 1200

ATD  A Dial out

ATE    Echo modes 0-off    1-on

ATF    Autodetect Mode 0 - 10

ATH    Hang up (On Hook) 0-=Off Hook  1-On Hook (hung up)

Positions 1 to 4: the command.

Position 6: A if the command is to be assumed.  Assumed commands are not submitted to the modem.

Positions 8 to 79: Explanation for the command.

AT Command Section line items may have comments attached.  These are listed along with the command on the output.  For example, if you wish to be more verbose, you could change the ATB line as in:

ATB    Select protocols for 1,200 baud

C       0 sets CCITT

C       1 sets Bell 212A

Comments

Position 1: C

Positions 2-13: not used.

Positions 14 - 79: the comment.

Comments may total up to 1024 characters per line item.  Each line of comments adds 16 characters to the total for spacing and formatting.

This means that you may use up to about 12 lines of comment per line item,  more if the comments are short.  This ought to be plenty.

## 2.5	SReg Section

This section documents the S Register usage.  For example:

Sreg section

;2345678901234567890 ...

S000        000 Answer on Nth ring.

S001        000 Current ring count.

S002        043 Escape sequence character.

· · ·

S013        000 Reserved

;2345678901234567890 ...

S014:0      000 Autodetect 0>disabled  1>enabled. F.

S014:1      001 Echo (1) enabled to save line time, E.

S014:2,6    000 Result codes, enabled (0) Q.

S014:3      001 Result codes, verbal (1) V.

S014:5      000 Pulse (1) or Tone dialing.

S014:7      001 Originate (1) or answer mode.

Position 1: S

Positions 2-4: the register number.

Positions: 5-12: for bit definitions (see below).

Positions 13-15: the usual value for the the register.

Positions 17-79: the explanation for the register.

S Register line items may have attached comments just as for the AT Command Section.

For S registers that are bit value registers, the format is:

Position 1: S

Positions 2-4: the register number.

Positions: 5: Colon, :.

Positions: 6: The bit, or start bit number.

Positions: 7:  
blank if this is a single bit switch, as in:

S014:1      001 Echo (1) enabled to save line time, E.

Dash - if this is for a sequence of bits

S021:3-4    000 AT &D  DTR options

Comma , if this is for a pair of bits

S014:2,6    000 Result codes, enabled (0) Q.

For either sequences or pairs, the first bit is low order, which is the conventional way so that 0-7 has its usual meaning.

Positions: 8: The second or end bit.

Positions 9-12: reserved for further confusion from modem manufacturers.

Positions 13-15: the usual value for the the register.

Positions 17-79: the explanation for the register.

## 2.6	Identify Bits Section

The command ATI4 responds with several lines.  For example a Zoom VFP28 responds:

ATI4

a007880284C6002F

bC60000000

r1005111151012004

r3000111170000000

Important points to be made is that most manufacturers are reluctant to give out information. there is a Hayes "Standard", most manufacturers do not follow it, but they do come up with something like it.  A copy of a piece of the Hayes "Standard" is in Appendix 1.

In the example above, it looks like Zoom tried to follow the Hayes spec. but blew it by ignoring byte D6 and wound up one byte short with all the bits offset from D7 on.  Maybe there is some other explanation, they aren't saying.

If you look in Appendix 1, there are four strings, a, b, r1 and r3.  Given this, the lines in the Identify Bits Section are understandable, for example:

Identify bits section

;234567890123456789 . . .

a  03 3 Smartmodem 1200 FE Commands

a   3 2 Smartmodem 2400 FE Commands

· · ·

a  16 0 S95:0 CONNECT XXXX (for DCE rate)

;b line

b   1 3 V.42 Alternate Protocol (MNP)

b   1 2 V.42 LAPM

; speed strings  r1 and r3

r1  2 0 45.45

r1  2 1 50

· · ·

r1 15 3 100,800

r1 16 0 105,600

r1 16 1 110,400

r1 16 2 115,200

r3  2 0 45.45

r3  2 1 50

· · ·

Positions 1-2: The line identifier.

Positions 4-5: The byte offset, or for the a line, the D number.

Positions 7: The bit offset, 0 to 3.

Positions 9-79: The bit explanation.

For example, the line:

a  03 3 Smartmodem 1200 FE Commands

means in the 'a' line, use the D3 byte, which is the fourth, and then select the third bit. and if it is on, the modem supports something like the Hayes Smartmodem 1200 FE commands.  Since the bytes are ASCII representations of Hex nibbles, this means that the 1200 FE commands are supported if D3 is one of { 8, 9, A, B, C, D, E , F }, because that means that bit 3 of 0-3 is ON.

Since D3 is 7, for a Zoom VFP28. that means that it does not support 1200 FE commands.  Since bit 2 is on, the VFP28 does support 2400 FE commands.

These formal definitions are always more tedious than the actuality.  Look at some of the included DAT files and it should be relatively easy and obvious, particularly if you waded through the introduction of this chapter and thusly realize that we may have to be able to define all the commands and any bit of the S registers differently for any particular modem.

# Chapter 3 - The MA Display

The display primarily keeps moving to let the user know that it is still running.  It is informative nonetheless.  The registers are displayed as in:

S001 S002 S003...

When the AT commands are sent to a modem there are four possible modem responses.

1.	"ERROR" is assumed not to be a command.  The command terminating character is displayed.

2.	"OK" is returned for a good command.  An o is displayed.

3.	Timeout is assumed to be a partial command and therefore legitimate.  A t is displayed.

4.	The command was "assumed" with an "A" in the command definition line, position 6.  An a is displayed.

When trying commands the display looks like:

Checking  AT commands...

0 1 2 3 4 5 6 7 8 9 o t < t > o @ a a C o t F G t t J K t t N a t t R a t  U a t t t

So the sequence  @ o t o D E F in the AT\ area means:

AT\@ returned ERROR.

AT\A returned OK.

AT\B returned timeout.

AT\C returned OK.

AT\D E and F  returned ERROR.

No assumed commands were in this sequence.

The a returned for ATO indicates that ATO was assumed, which is a good idea because the ATO command says Go OnLine Immediately, which we don't really want to do.

The display may be re-directed to a file.

# Chapter 4 - The Output File - COMMAND.LST

The COMMAND.LST file is a complete list of your modem's status.  First is the ordinary AT&V output:

ACTIVE PROFILE:

B1 E1 L2 M5 T Q0 V1 X4 Y0 &C1 &D2 &G0 &L0 &P0 &Q0 &R0 &S0 &X0 &Y0

\A3 %A013 \C0 %C1 %E1 \G0 \J0 \K5 \N3 \Q3 \T00 \V2 \X0 -J1 "H3 "S0 "O250

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:030

S08:002 S09:006 S10:014 S11:070 S12:050 S18:000 S25:005 S26:001 S31:000

STORED PROFILE 0:

B1 E1 L2 M5 T Q0 V1 X4 Y0 &C1 &D2 &G0 &L0 &P0 &Q0 &R0 &S0 &X0

\A3 %A013 \C0 %C1 %E1 \G0 \J0 \K5 \N3 \Q3 \T00 \V2 \X0 -J1 "H3 "S0

S00:000 S07:030 S11:070 S18:000 S23:029 S25:005 S26:001 S27:064 S31:000

STORED PROFILE 1:

B1 E1 L2 M5 T Q0 V1 X4 Y0 &C1 &D2 &G0 &L0 &P0 &Q0 &R0 &S0 &X0

\A3 %A013 \C0 %C1 %E1 \G0 \J0 \K5 \N3 \Q3 \T00 \V2 \X0 -J1 "H3 "S0

S00:000 S07:030 S11:070 S18:000 S23:029 S25:005 S26:001 S27:064 S31:000

TELEPHONE NUMBERS:

&Z0

These manufacturer supplied dumps are usually too compressed to be informative.  They are also often incomplete, missing registers and commands.

We list the registers' values, and an explanation:

Registers

S00=  000  Answer on the 0th ring.

S01=  000  Ring count - count for S0, above.

S02=  043, 2Bx, +  Escape (Off line) character. Above 127-> no Escape.

S03=  013, Dx      Carriage Return character.

S04=  010, Ax      Line Feed character.

·  ·  ·

S12=  050  Expect 50 50ths of seconds of silence after the escape sequence.

S13=  000  Undocumented register

S14=  138, 8Ax Bit Map  bits 0, 4, 6 reserved.

1:1   Echo  0=off 1=on.

2:0   Result codes  0=enabled  1=disabled.

3:1   Result codes  0=Numeric 1=Verbal.

5:0   Dialing  0=Tone 1=Pulse

7:1   Mode  0=Answer  1=Originate

·  ·  ·

When we run through the ATI, or Identification commands.  Although we have only seen AT0 through AT4 documented so far, we scan from AT0 to AT8:

Product Identification/Checksums

ATI0    149

ATI1    000

ATI2    ROM Checksum OK

ATI3    DOWNLOAD.400-3.00

ATI4    007040C004000

When we list the AT commands which returned an "OK", a timeout, or were assumed.  "ASSUMED" is appended to the explanations for assumed commands.  We put a blank line in before commands which seem legitimate but which were not described in COMMAND.DAT, undocumented commands.

AT Commands

AT=  Undocumented Command

AT?  Undocumented Command

ATA  Manual Answer ASSUMED

ATB  Select CCITT or Bell protocols for 300/1200 baud

ATD  Dial out ASSUMED

ATE  Echo modes

ATH  Hang up (On Hook)

ATI  Product codes

ATL  Speaker volume control

ATM  Speaker on/off control

ATO  Return to on line mode ASSUMED

ATP  Undocumented Command

ATQ  Enable/disable result codes

ATS  Read and write S registers ASSUMED

You can see that MA gives you the most complete listing of a modems status.

NOTE:  Modem Audit may change the modem's profile.  For example to test the ATE command, as we just submit ATE if your modem had been set up as ATE0, it will now be set up as the ATE default.  However you may use the A2P program to restore the modem from a COMMAND.LST file.  See below.

# Chapter 5 - Save Status, P2A

## 5.1	Saving Profiles to ASCII

P2A used to copy the profile from the modem to the display.  If the output is re-directed to a file, the modem's profile is saved so that it may be re-applied to the modem at any later time, or applied to a similar modem elsewhere.  We may use A2P to restore a modem's profile from the output of P2A.

P2A has been eliminated.  It is just as easy to us AT as in

AT com_port &V

# Chapter 6 - A2P ASCII to Profile

## 6.1	Restoring a Profile to a MODEM

A2P submits all the pieces of an ASCII profile to the modem as a command.  Almost all, we skip the phone numbers and the stored profiles.

## 6.2	A2P Command Syntax

A2P needs to know the file to use for input and the com port to which the modem is attached.  We also use a LOG file.  The same details about com_port as for MA apply.

A2P com_port filename  logfilename [DEBUG]

filename	 is the drive, path and file name of the file.  The file should have output re-directed from P2A.

logfilename	 is the drive, path and file name of the file to which a log of the actions taken DialUp be directed.  This file will be created and any file of the same specification will be destroyed.

DEBUG	is an optional DialUp which will display the commands as they are sent to the modem.

## 6.3	A2P Output

P2A displays, or sends to a file, output similar to:

Setting Q0 V1  so that we have verbal responses.

However the LOG file, A2P.LST, is very useful, particularly if something seems to be going wrong:

ATB0

OK

ATE0

OK

ATL0

OK

ATM0

OK

·   ·  ·

OK

ATS26=001

OK

ATS31=000

OK

A2P accepts the MA output file, COMAND.LST for setting up the modem.  It will also accept the results of AT port &V, see below.

DTE Note: Some PCMIA and portable modems cannot handle 2,400 Baud, which is the default for AT, HANGIT, MA, and A2P.  These will timeout on the very first command.  You may select another DTE for these by setting Environment parameter DTE to the DTE rate, as in:

SET DTE=19200

Use one of 2400, 4800, 9600, 19200, 38400, 57600 and 115200.  You may have to use the MODE command from DOS to set the port as well.

# Chapter 7 - Submit an AT Command - AT.EXE

AT is a simple little but useful program to jam a command at a modem.  It is most useful to hang up a modem, or to reset one or several profile parameters.  It was just meant to be a simple test and diagnostic tool but it got so useful that we decided to include it.

## 7.1	AT Command Syntax

AT requires the port number and then the command to submit.  AT assumes an AT at the beginning, so do not include that portion of a command.  AT uses up to 9 parameters, P1 through P8.

AT com_port [p1] [p2] [p3] [p4] [p5] [p6] [p7] [p8]

com_port is the usual com or serial port parameter.

p1 through p8 are optional parameters which must be AT commands.

## 7.2	Examples

Reset the modem to Manufacturer settings:

AT 3 &F

Hang up the modem:

AT 3 H

Shut the speaker off:

AT 3 M0

You may use several parameters, as in the following which turns the speaker ON and sets it to low:

AT 3 M1 L1

Warning: Because of a peculiarity in DOS commands in order to perform an AT command with a quote, precede it with a backslash.  Particularly whenever you use a quote check the displayed output for correctness.  To submit AT"H0 for example, submit:

AT 3 \"H0

DTE Note: Some PCMIA and portable modems cannot handle 2,400 Baud, which is the default for AT, HANGIT, MA, and A2P.  These will timeout on the very first command.  You may select another DTE for these by setting Environment parameter DTE to the DTE rate, as in:

SET DTE=19200

Use one of 2400, 4800, 9600, 19200, 38400, 57600 and 115200.  You may have to use the MODE command from DOS to set the port as well.

# Chapter 8 - HangIt - Disconnect Modems

## 8.1	HangIt Process

HangIt sends an ATH to the modem.  If the modem is dis-connected it will respond OK.  If HangIt gets an OK it terminates.

If the modem is busy chatting to no one, it will not respond with an OK.  Then HangIt sends the modem a string of +++.  This is supposed to tap the modem on the shoulder and say, "Hang on a minute, I need to talk to you.".  HangIt waits a bit and then tries another ATH.

If we have gotten this far, probably the modem is jammed open, however, if we get an OK, we've successfully dis-connected the modem and HangIt goes away.

If HangIt does not get an OK, it tries the +++, wait, ATH sequence ten times, each time bleeping the user and asking that the power be cycled.  the <ESC> key will interrupt the program.

If after ten times it still cannot dis-connect the modem, it ends anyway, but with a return code of one instead of zero, so you may set up a nasty little batch loop as in:

:loop

hangit 3

if errorlevel 1 goto loop

Then your computer will keep on beeping at you until you turn it off.

## 8.2	HANGIT Command Syntax

Note: Be sure that your modems are Hayes command compatibleXE "Hayes command compatible"§ and that responses is ON (Q0) and responses are verbose (V1).

## XE "MODEMTST Command Syntax"§

The formal syntax for HANGIT is:

HANGIT com

where:

com is a number from 1 to 8 indicating the com port to use.  This release of HANGIT supports the standard COM assignments:

## 8.3	Reset vs. Power OFF/On

You might get lazy and use the reset button on your computer instead doing Power OFF/ON.  Well some modems do not correctly re-set when the bus reset line is raised.  The bus reset line (a.k.a. Master Reset) is one of the gold tabs on the card that you put into the bus slot.  "raising" the line means putting a few more volts through it so that the board knows to do something.

Well, what the board does is determined by the firmware that you load into the modem, and not all the manufacturers get it right.  We will not keep a list of who does it wrong, because they might get it right next time they upgrade the firmware, or ones that have it right may screw it up.  Yes, indeed it is hard to believe that the modem manufacturers could screw up something so basic, but the computer on which this is being typed has just such a modem.

So, if you want to be sure, turn the power off, wait three seconds and then turn th power on again.

DTE Note: Some PCMIA and portable modems cannot handle 2,400 Baud, which is the default for AT, HANGIT, MA, and A2P.  These will timeout on the very first command.  You may select another DTE for these by setting Environment parameter DTE to the DTE rate, as in:

SET DTE=19200

Use one of 2400, 4800, 9600, 19200, 38400, 57600 and 115200.  You may have to use the MODE command from DOS to set the port as well.

# Chapter 10 - Stress Test Overview

Manufacturers of modems say a lot of things about their modems.  A lot are not true, or truth has to be bent so far that it meets itself coming back in order to have what the manufacturer says be "true".  For instance one advertises MNP and does it in software, how regressive.  Another doesn't answer anyone elses' V.42 and claims that it does, and if it doesn't they are all otu of spec.  On it goes.  So we wrote a stress tester.

## 10.1	Four Parts to Modem S/T

There are three pieces to the Modem S/T.

STS (Stress Test Send) is a program which calls another system and sends a known set of records for testing the outer limits of the modem's performance.  You must call a FactsBack/SAFE system or one running STR.

STR (Stress Test Receive) is a program which answers another system and receives a known set of records for testing.  It accumulates results and then sends them to the caller.

STRESCAL (Stress Calculate) is a program which calculates and displays statistics from a test run including some informative charts.

The basic Modem S/T procedure is to have STS call a STR or FactsBack/SAFE system.  The SAFE system receives a test set of records and takes precise measures of when they are received  The timing measurements are stored in a file named STRESS.TAB and which is then sent to the calling system.

STRESCAL interprets the results, giving you data rates for compressible and incompressible data, averages and maximum transfer rates.  It also displays some interesting graphs which can be used for better understanding of the results:

## 10.2	Quick StartXE "quick start"§

On the dialing or calling computer run the batch file:

STS 3 57600 5 1-603-256-8055

STRESCAL

replacing:

3	with the number of your COM port;

57600 with the maximum baud expected (after compression).  Actually DTE, see below.

5	with the number of "records" to use (5 is a good number for initial tests)

1-603-256-8055 with the phone number of the receiving or answering system.

Note: Be sure that your modems are Hayes command compatibleXE "Hayes command compatible"§ and that echo responses is ON, E1; responses are verbose, V1; and extended verbose is also set if you have it, \V2.

# Chapter 11 - STS - Stress Test Send

## 11.1	The STS ProcessXE "MODEMTST Process"§

When STS is run, STS:

1.	Dials the FactsBack/SAFE system, the receiver.

2.	Checks to see if the sender and receiver agree on how many patterns to exchange.  This can be any number from 0 to 500.  Each pattern is a set of 3Kbytes of highly compressible data and 3Kbytes of incompressible data.

Both the sender and receiver allow up to ten seconds for each 1K byte block.  If this limit is exceeded the line is considered too noisy for stress testing.  Even at 2400 baud non-compressed a 1K block should only take 5 seconds.

3.	Transmits the patterns to the receiver, which checks the patterns for correctness.

4.	Turns the line around 100 times.  We call this jitterbugging.

5.	Receives the results file using the Zmodem protocol.

6.	Hangs up and ends the program.

STS needs to know:

1.	Which COM port to use.

2.	What DTE rate to use.

3	How many patterns to use for testing.

4	What phone to call.

Not surprisingly, a STS command may look like:

STS 3 57600 5 1-603-256-8055

Note: Be sure that your modems are Hayes command compatible and that echo responses is ON, E1; responses are verbose, V1; and extended verbose is also set if you have it, \V2.

## 11.2	STS Command SyntaxXE "MODEMTST Command Syntax"§

The formal syntax for STS is:

STS comXE "com"§ dteXE "dte"§ npatXE "npat"§ phone [LOCAL] [DEBUG]

where:

com is a number from 1 to 8 indicating the com port to use.  This release of STS supports the standard COM assignments:

dteXE "dte"§	Data Terminal Emulation, the characters per second rate for between the CPU and the modem.  As was noted before, this should be higher than the Baud rate (the Modem-Modem rate).  If you expect MNP-5, use twice the baud rate.  When you expect LAPM (Link Access Procedure for Modems), also known as (sort of) CCITT V.42bis, use a DTE rate that is four times the baud rate.

Supported DTE rates are (do not enter the comma):

DTE rateXE "Baud rate"§s

1,200

2,400

4,800

9,600

14,400

19,200

38,400

57,600

115,200

npatXE "npat"§	is the number of patternsXE "patterns"§ to use for testing.  We cover this in detail below.  5 is a good number to start with.  npat must be from 0 to 500.

phonenumberXE "phone number"§:	This is the phone number to be dialed.  Phone number may be up to 40 characters.  Your modem documentation will have the characters you may use.  Generally the Hayes standard set will be available which includes:

-	for documentation, as in 387-2763

,	for a two second pause

0-9	for dialing

As this is a command line parameter, no internal blanks are permitted.  Do not use something like "1 603 256 8055", instead use "1-603-256-8055" (quotation marks added for "clarity").

If your entry includes the AT characters, STS assumes you are taking control of the entire setup and transmits the entire command to the modem.  This is how to do pulse dialing or to enter odd commands, as in:

STS 3 57600 5 ATDP1-603-256-8055

or

STS 3 57600 5 AT&FS31=5DT1-603-256-8055

Do not use blanks in the AT command.

Finally, you may use LOCALXE "LOCAL"§ for the phone number, in which case STS will attempt to establish a local connection (two computers, two modems and a modular phone line between them without the assistance of your local phone company):

STS 3 57600 5 LOCAL

DEBUGXE "DEBUG"§	is an optional parameter which increases the text sent to STS.LOG during a run.

## 11.3	STS DisplayXE "MODEMTST Display"§

A successful STS run will display results like:

STS,(c)Copyright New England Res.&Des.Software,1993.All Rights Reserved.

ATH\T1DT387-2763

Dialing.

Baud:14400  DTE:57600   8N1  DTR:1  RTS:1  Rel.:1  Comp.:1  MNP:0  LAPM:1

Sending 5

Receiving 5

GOTCOMP

5 records or 15360 bytes  0:04.06   APPROXIMATELY: 3783 bps

Scrambles.

GOTRAN

5 records or 15360 bytes  0:10.22   APPROXIMATELY: 1502 bps

Jitterbugging...

FINI

OPENING FILE STRESS.TAB

File STRESS.TAB updated.

The display lines are explained below:

ATH\T1DT387-2763

Dialing.

This shows you the AT commandXE "AT command"§ used and lets you know that communications with the local modem have begun and the remote connection is being attempted.

Baud:14400  DTE:57600   8N1  DTR:1  RTS:1  Rel.:1  Comp.:1  MNP:0  LAPM:1

BaudXE "Baud"§ and DTEXE "DTE"§ we have covered in detail.  This line give the specifics for this connection.

8N1	XE "8N1"§	stands for the usual 8 bits, no parity, one stop bit.

DTRXE "DTR"§		is "1" if data terminal ready is set, else "0".  It will usually be "1".

RTSXE "RTS"§		is set to "1" if Ready To Send is set.  We onlys support modems with RTS/CTS flow control.

Rel.XE "Rel."§		is set to "1" if a reliable connectionXE "reliable connection"§ has been established.

Comp.	is set to "1" if compressionXE "compression"§ is taking place

MNP		is set to "1" if MNPXE "MNP"§ compression is being used.

LAPMXE "LAPM"§	is set to one if Link Access Procedure for Modems is being used.

Sending 5

This line says that you are sending 5 records.  Your system is the sender.  The other system will receive and log times for the records, and then send you a file of the results.

The sender and receiver must agree on the number of patternsXE "number of patterns"§XE "patterns"§ to be sent.  If the sender  asks for less than the receiver was expecting, the number of patterns is reduced.  If the more are requested the session is terminated.

GOTCOMP

5 records or 15360 bytes  0:04.06   APPROXIMATELY:3783 bps

This indicates that the compressible data has been received and verified (GOT COMPressible).  The number of bytes is shown, the time used as seen from the sender's end and is only a guess at the bytes per second.  The receiver has the accurate timing and will send along the data later.

Scrambles.

GOTRAN

5 records or 15360 bytes  0:10.22   APPROXIMATELY:1502 bps

This indicates that the incompressible data has been received and verified (GOT RANdom).  The number of bytes is shown, the time used as seen from your end and a guess at the bytes per second.

Jitterbugging...

The line is being turned around 100 times.  Tests are complete.

OPENING FILE STRESS.TAB

File STRESS.TAB updated.

The sender has received the file STRESS.TAB using ZModemXE "STRESS.TAB"§.

## 11.4	STS.LOGXE "MODEMTST.LOG"§ File

STS keeps a log of each run in a file named STS.LOG.  This file is appended to each time, and so if you do not edit and delete it once in awhile, it will grow large.  the optional debug switch adds to the information stored in STS.LOG.

## 11.5	STRESS.TABXE "STRESS.TAB"§ File

The file STRESS.TAB is teh file that STR sends to STS.  It has the start time and the time each 1024 byte block is received.  It is used by STRESCAL.EXE, which is described later.

## 11.6	CompressionXE "compression"§ and STS Patterns

Most modern modems support data compression, MNPXE "MNP"§ (Microcon Network Protocol) or V.42bisXE "V.42bis"§.  The way they generally work is that the modem receives some data.  It stores it in a buffer.  These buffers are generally 128 to 256 bytes but before 1995 we will have modems with 4Kbyte buffers.

OK, now we have the modem with some data in a buffer.  Then it looks for internal redundancy.  If it finds it, it can compress the data.  The modem does so and then sends it along the line.  MNP compression can reduce the amount of data transmitted to about 1/2 of the original.  LAPMXE "LAPM"§ (Link Access Procedure for Modems, LAPM can be thought of as V.42bis, if that helps) can reduce the data to as little as 1/4 of the original if there is a lot of internal redundancy.

Since we want to exercise the modems at maximum compression and also at minimum compression we need two record types, compressible and incompressible.  For compressible we use three patterns, each 1024 bytes long.  The first is all ASCII "1" characters, the second "2"s and the third is "3"s.  So for each time we do a record, we do 3072 compressible characters of compressible data.

For the incompressible characters we use a pseudo-random number generator to get us three records of 1024 bytes each for which each number is unlike the previous number, or very little internal redundancy.  This is a reasonably good simulator of transferring binary files and testing shows that it forces the modems right down to near the baud rate, which means that little compression is going on.  Furthermore, it is quite reproducible so the sender can generate what records that the receiver can easily check.  Then the receiving system may ask for a re-transmission if there is even a single bit wrong.

## 11.7	JitterbuggingXE "Jitterbugging"§

Another important aspect of high speed modem transfers is turnaround timeXE "turnaround time"§.  If an asynch line is going in one direction, that is data is flowing from modem A to modem B, if B then sends a character to A most communications lines take awhile to turn the line around and send the character.  In theory this doesn't happen on full duplex lines.  In practice fast modern modems still take about 70 milli-seconds to turn around.  Nowadays this may not really be line time, rather the amount of time a smart modem waits to see if there are more characters for compression.  The result is the same, however, it slows down your communications and means that you want to minimize the number of times the "receiver" has to talk to the "sender".

The jitterbugging section has modem A send a character to modem B, and then wait for a character from B.  It repeats this process 100 times.  Naturally the characters sent are checked in case line noise trashed a record.  If that happens, the jitterbugging test is abandoned.

## 11.8	TimingXE "timing"§ and STRESS.TAB Output

Timing is always done at the receiver's end.  This is because it really doesn't matter how fast the sender thinks it is going.  The sender has inaccurate measurements.  The only thing that counts is how fast the characters arrive at the other end.

At the receiver end, the time that each 1,024 byte block is received is stored in memory.  To assure that disk I/O does not affect the results no disk I/O is written until after the end of testing.  Furthermore there is a slight pause between each test (not each record) and the receiver's clock is started only when the first character of a block is received from the sender.  All this means that the results that you get are very accurate.

Anyway the timing marks are stored in memory.  When testing is complete the receiver writes file STRESS.TAB and then sends it to the system running STS.

## 11.9	AccuracyXE "accuracy"§ and Timing Errors

The measurements are good to within one PC click (an 18.5th of a second, or about 0.05§5 seconds).  for timing small amounts of time PC clock is only capable of returning times of .055, .11, .165 ... seconds.  At 57,600 data bits per second (14,400 baud with LAPM) a 1K block takes 0.14§ seconds.  because of the granularity of the measuring we will get some transfers recorded at .11 seconds and some at .165 seconds, but this is as good as one can do on a PC.

This is why the average is more important than the absolute maximum transfer rate and one of the reasons why there are "variations" in transfer rates for individual blocks.

## 11.10	Buffers and TimingXE "buffers and timing"§XE "timing"§

You should be more concerned about averages than minor variations for individual blocks.  Remember that there are many stages between when the sender sends a character before a character is available for the receiver program.  Any of these can slightly increase the amount of time for a particular block:

1.	The sender program "sends" a character (or block), that is it informs the lower level software that a character or region is ready to be transmitted.

2.	The character is moved into a RAM buffer for the com port.

3.	The character is moved into the modem's buffer through the bus.

4.	The modem may wait a bit for further characters for compression.

5.	The data or a data packet is sent from the sender modem to the receiver modem.

6.	The data is checked, if it is not correct a re-transmit is requested at the modem-to-modem level.

7.	The data is un-compressed if it was compressed.

8.	A character is made available to the receiver's bus and a receive interrupt submitted.

9.	The receiver's CPU gets around to noticing, picks up the character and moves it into the communications buffer in RAM.

10.	The low-level communications software picks up the character and delivers it to the receiving "application" program.

If disk or keyboard I/O is occurring at the "same" time, steps 1-3 and 9-10 may be delayed.  If  the delay is too much, the character may be damaged or lost (PARITY or FRAMING errorsXE "PARITY errors"§XE "errors: PARITY"§XE "FRAMING errors"§XE "errors: FRAMING"§).

If the CPU is too slow, as with an old 8088 at 4.7 MHertz or when running Windows (joke), step 10 may not be done in time.  Then the low level software's buffer may fill and overflow.  These are OVERRUN errorsXE "errors: OVERRUN"§.

STS will flag these if they occur.

# Chapter 12 - Analyzing Results - STRESCAL.EXE

## 12.1	STRESCALXE "STRESCAL"§ Results.

When the run is complete there is a file named STRESS.TAB on each computer, with timing ticks for each 1,024 byte transfer.  This raw data needs a bit of interpretation for us humans.  Program STRESCAL reads STRESS.TAB and prepares some useful information.

First it prints the average and maximum transmission rates for both the compressed and incompressible data, such as:

Compressed: 6,400 5,086.1 12 Un-compressed: 1,862 1,648.2 5

Line turnaround time  73.1 msec  or 421.1 chars at 14,400 baud

Word Processing files: secs per page 0.94 Pages per minute  63.834

and then draws two sets of graphs, one for compressed and one for incompressible data, showing the milli-seconds per 1K block and the bytes per second for each 1K block:

Compressed: 6,400 5,086.1 12 Un-compressed: 1,862 1,648.2 5

These are the maximum bytes per second for a 1K block (e.g. 6,400), the average (e.g. 5086) and the coefficient of variation(cov).

The cov is a statistic which shows how much variability is in the statistics.  If there is a lot of line noise, the cov will be higher.  This may be useful for comparing runs on problem or noisy lines.  If you are familiar with some statistics, it is just a fancy standard deviation, sort of.

Line turnaround time  73.1 msec  or 421.1 chars at 14,400 baud

The line turnaround time is from 100 samples.  Many communications consultants will tell you that the turnround time is zero for a full duplex line.  They are right about the line but horrendously wrong when it comes to two computers, two modems and a line.

The characters listed is to demonstrate how shatteringly bad turn-around may be as compared to full streaming.  This is why ZModem is faster than Xmodem.  This is also why we need a ZModem II for multiple file transfers.

Word Processing files: secs per page 0.94   Pages per minute  63.834

The word processing files is an estimate using the average compressed speed and assuming 2,500 bytes per page of word processing documents.  If you research your DOC files you will discover that this is not a bad estimate for documents like contracts that do not have a lot of graphics in them.  For items like this manual or first pages of letters with graphic letterheads there will be more bytes per page.

Nevertheless, one realizes that the minute per page that facsimile transmission takes is disgustingly inefficient with your phone time.

## 12.2	STRESCAL GraphsXE "STRESCAL Graphs"§

When you run STRESCAL first it shows you a histogram of the milliseconds for the transfer.  The histogram is useful because you can see at a glance how many records were slow, how many fast, ...

On this run, for the compressed records on the left, we had sixteen records at 160 and the same at 170 milliseconds, ten at 220 milliseconds, one at 270 ms, and 2 at 330 ms.  This is a rather normal distribution of records for a clean line.  There will always be a slow one or two at the beginning and then the transmission settles in to sending records at two or three millisecond splits.  Remember the granularity of the PC timing and all the buffering going on.

On non-compressible records we get less of a variation because most of the compression related buffering is eliminated.  Unless there are noise spikes, we will see the cov lower for non-compressible records on every run.

This histogram display is particularly useful for runs with a large number of records.  Note that we have used a power of two log scale on the y-axis.  For example, for a run of 150 1K blocks we might get the following results.

Using the histogram we can easily see the results for runs of up to 500 patterns, or 1,500 1K blocks.  The per-record graphs of the second page are not so good for large runs, unless one just wants to see where noise spikes were.

The graphs on the second frame are pretty much self explanatory.  Each 1K block is plotted, both by the number of milliseconds the block took, and the bytes per second for the block.  For a run of 50, we see graphs like:

The first one or two blocks of 1,024 characters always take a little longer.  Then the buffers and whatever seem to settle down and the timing seems to cycle between two or three measurements.  No doubt the vagaries in the measurements are primarily from the poor resolution of the PC clock and the variations introduced by the clock ticks hitting partially complete buffers sometimes, just complete buffers sometimes and sometimes buffers about to be filled.

A sample from a run of five on two systems with Intel SatisFAXtionÔ Intel and a straight phone cord between them (LOCAL connection).

A sample from a run of three hundred on two systems with Intel SatisFAXtionÔ Intel and a straight phone cord between them (LOCAL connection).  This is the maximum for STS and goes beyond the resolution of the graphics.  The only reason we put the maximum as high as 300 patterns (900 Kbytes of each of compressible and incompressible is that then using the log one can prove intermittent errors from the re-transmission requests).

A sample from a run of thirty patterns on two systems with Intel SatisFAXtionÔ Intel and a straight phone cord between them (LOCAL connection).  The sender has the DTE limited to 19,200 so the Bytes per second is way down.

# Chapter 13 - STR - Stress Test Receive

## 13.1	The STR ProcessXE "MODEMTST Process"§

When STR is run, STR:

1.	Prepares the modem for answering.  After connect,  STR does . . .

2.	Checks to see if the sender and receiver agree on how many patterns to exchange.  If the sender requests fewer patterns than the receiver has set for maximum, the receiver agrees.  If the sender requests more than the set maximum, the receiver tells the sender to lower the number of patterns.

3.	Recieves and checks the patterns.  If one is in error, a "REDO" is reuqested.  Each time that a 1K block is successfully received, a timing tick is stored in RAM.

4.	Turns the line around 100 times.  We call this jitterbugging.

5.	Writes the timing ticks to a table on diosk, a file named STRESS.TAB.

5.	Sends the results file using the Zmodem protocol.

6.	Hangs up and ends the program.

STR needs to know:

1.	Which COM port to use.

2.	What DTE rate to use.

3	The maximum number of patterns to allow.

Not surprisingly, a STS command may look like:

STR 3 57600 50

## 13.2	STR Command SyntaxXE "MODEMTST Command Syntax"§

The formal syntax for STS is:

STR comXE "com"§ dteXE "dte"§ maxpatXE "npat"§ [LOCAL] [DEBUG]

where:

com is a number from 1 to 8 indicating the com port to use.  This release of STS supports the standard COM assignments:

dteXE "dte"§	Data Terminal Emulation, the characters per second rate for between the CPU and the modem.

maxpatXE "npat"§	is the maximum number of patternsXE "patterns"§ to use for testing.

DEBUGXE "DEBUG"§	is an optional parameter which increases the text sent to STR.LOG during a run.

LOCAL	is an optional parameter that must be used for local or direct hookups.

## 13.3	STR DisplayXE "MODEMTST Display"§

A successful STR run will display results like:

STS,(c)Copyright New England Res.&Des.Software,1993.All Rights Reserved.

ATH\T1DT387-2763

Dialing.

Baud:14400  DTE:57600   8N1  DTR:1  RTS:1  Rel.:1  Comp.:1  MNP:0  LAPM:1

Sending 5

Scrambles.

File STRESS.TAB updated.

## 13.4	STR.LOGXE "MODEMTST.LOG"§ File

STR keeps a log of each run in a file named STR.LOG.  This file is appended to each time, and so if you do not edit and delete it once in awhile, it will grow large.  the optional debug switch adds to the information stored in STR.LOG.

# Chapter 14 - Errors and Common Problems

## 14.1	No Response to STS

Assuming that you can use the modem other ways, e.g. use Xtalk to attach to a BBS, then

1.	Check that responses are echoed, verbal and extended, ATE1V1\V2.

2.	Check that you are using the correct COM port and that the IRQ and ADDR are not non-standard.

3.	Try lower dte rates.  Some modems simply freeze when the dte is set "correctly" but too high.  If this is the case do not buy that modem again.  Let us know for our Goofs&Glitches documentation.

4.	If all else fails let us help isolate the problem.  After fixing it we will ask for some of your time to help us document the problem correctly and to discuss it with the manufacturer.

# Chapter 15 - SuggestionsXE "Goals, Goofs & Glitches"§

## 15.1	Modem Handling

You will have noticed that you can override the modem setup easily.  This is an important feature.  In structured/modularized systems it is sometimes called a trapdoor - when a facility provides a function but also lets the skilled user change the assumptions.  Many software packages do not provide this and many programmers waste about 40% of their time attempting to change the assumptions that do not apply to their situation and which were made by some other unreachable programmer (system designer or marketing person).

Consistent with that we make the minimum changes to your modem setup.  If you use the defaults you will notice that you can use any other program, such as TESTCOM, to set the parameters for the modem.

If you have difficulties it may be that some other communications package has left your modem in an unusable condition.  Before you are too frustrated, try sending an AT&F to reset the modem.  This might look like:

STS 3 57600 5 AT&FDT1-603-256-8055

## 15.2	Timeout - AT\T1

Having said we do the minimum in modem setup, we now recommend that you change the setups.  We recommend that you include the \T1 command in your setup strings for your modems.  What the \T1 says is that the modem should disconnect after 1 minute of silence.

There are a host of reasons that a disconnect might fail and your phone thinks is remains connected to a remote site.  Not only could bad software in your communications package or on the remote BBS (there is a lot of it around) might fail to disconnect, the disconnect transaction might be hit by line noise, or a failed physical switch in a modem (yup, it is a physical switch, you can usually even hear it click) which could leave you attached.  Then, if you walk away for the evening you might get a 12 hour connect charge.  So we recommend that you add the \T1 to your setup strings.

If you have an external modem you might consider turning it off as soon as a session is complete.  If you have an internal modem you should cycle the power Off/On or at least Reset the computer.

# Chapter 16 - Anomalies

## 16.1	AnomaliesXE "Anomalies"§

This section is very technical.

Apparently the often quoted figure of a factor of 4 for LAPMXE "LAPM"§ or V.42bisXE "V.42bis"§ compression is an estimate.  In practice with slower baud rates and a very fast DTE, as in baud=4800 and dte=57,600, the transmission of our packed records goes at about 3,000 bytes per second which is 24,000 bits per second, well above the estimated maximum of 12,000.

Either LAPM can compress much more than just a factor of four, or the Intel modems we use for testing are performing a re-train and use a higher baud without announcing it to the software.  This latter situation does not seem to be the case because the incompressible records go across at the expected speeds.  If the modem had re-trained then the incompressible records would go faster as well.  As we said, anomaly.  Here are some results of testing.

Baud

Compressible

Incompressible

2400

4815

311

4800

5086

585

7200

4987

842

9600

5003

1,118

12000

5086

1,405

14400

5086

1684

Please test your higher speed modems this way and let us know the results.  To play with the baud rate you want to reset S-register 31, as in ATS31=6DT1-603-256-8055.  To turn off LAPM use \N0 and \N3 for back on.

On the Intel 400 at least the values for S31 are:

Value

Baud

1, 2, 4

Re

served

0

Speed of COM: port

3

300

5

1200

6

2400

7

4800

8

7200

9

9600

10

12000

11

14400

For testing we used commands like:

STS 3 57600 5 ATS31=7DT387-2763

which sets the dte to 57,600 and the baud to 4800.

# Chapter 16 - Questions and Answers

## 16.1	Change Control

One of the nastiest questions to answer about a modem which has stopped working is what has changed about it?  Frequently the change has been in the setup.  Some service, BBS, program or even game has set the profile differently.

With ModemAudit this is no longer much of a problem.  One can compare new and old settings with a DIFF, if you have it, or DOS's FC (File Compare).  After doing one of these you have an exception list of the differences between the two setups.

You may use DIFF or FC on either the COMMAND.LST output from MA, or a re-directed output from P2A.

## 16.2	Editing a Status File

Suppose you wish to change a setup and you have the output from an P2A.  You may edit the file to create a different profile.  For example if you wanted a modem to answer the phone but not use V.42bis, you could turn it off, for example by changing the \N  and -J commands.  Using EDIT, Brief or any other editor and assuring the result is an ASCII file, you could change:

\A0 %A000 \C0 %C1 %E0 \G0 \J0 \K0 \N0 \Q3 \T00 \V0 \X0 -J0 "H3 "S0 "O250

to:

\A0 %A000 \C0 %C1 %E0 \G0 \J0 \K0 \N3 \Q3 \T00 \V0 \X0 -J1 "H3 "S0 "O250

If you do this, be sure to  remove most of the S register settings or the subsequent restoration of the S registers may undo the changes in the commands.

Actually A2P looks for "Active Profile" and then starts the AT commands.  When it finds S00 it stops the AT commands and starts setting the S registers until it reaches "Stored Profile".  Therefore, your input file for A2P could be as simple as:

ACTIVE PROFILE:

\N3 -J1

S00:000

STORED PROFILE 0:

However, if you are going to use the A2P input file as a sort of script language for modem setups, we strongly recommend that you include a reset to manufacturer specs in the front, the &F command, as in

ACTIVE PROFILE:

&F

\N3

-J1

S00:000

STORED PROFILE 0:

Of course, if you have simplified it this much, might as well just use the AT command:

AT com_port &F \N3 -J1  S00=000

# Appendix 1 - Hayes ATI4 Specification

Below is a piece of the Hayes ATI4 response specification.  It is available on the Zoom and Hayes BBSs.

I4 "a" String..

The first string, the a-string, is encoded by characters, hex symbols

following the "a" character, each of which represents four bits.

a D1 D2 D3 D4 D5 D6 D7 D8 D9 D10 D11 D12 D13 D14 D15 D16 D17 D18

D1, D2 Reserved

D3     Bit 3: Indicates modem based on SM1200FE commands

Bit 2: Indicates modem based on SM2400 commands

Bit 1: Indicates that modem supports &J commands

Bit 0: Indicates that modem supports &L commands

D4     Bit 3: Indicates that modem supports AutoSync (&Q4)

D5     Bit 3: Plug-in board modem product

Bit 2: Stand alone modem product

Bit 1: Supports &H0 through &H4

Bit 0: Supports &I0 through &I4

D7     Bit 3: Supports V.22 at 1200 bps

Bit 2: Supports Bell 212A

Bit 1: Supports ASB (&Q6) in V.23 75xmt/1200rcv

Bit 0: Supports ASB (&Q6) in V.23 1200xmt/75rcv

D8     Bit 3: Supports V.23 1200xmt/75rcv

Bit 2: Supports V.23 1200 half duplex

Bit 1: Supports V.23 75xmt/1200rcv

Bit 0: Supports V.23 75xmt/600rcv

D9     Bit 3: Supports V.21 100/300

Bit 2: Supports ASB (&Q6) in V.23 75xmt/600rcv

Bit 1: Supports ASB (&Q6) in V.23 600xmt/75rcv

Bit 0: Supports V.23 600xmt/75rcv

D10    Bit 2: Supports V.22bis at 2400 bps

Bit 1: Supports V.27ter at 2400 bps

Bit 0: Supports V.27ter at 4800 bps

D11    Bit 3: Supports V.32 full duplex at 4800 bps

Bit 2: Supports V.32 half duplex at 4800 bps

Bit 1: Supports (Reserved) at 4800 bps

Bit 0: Supports V.29 half duplex at 4800 bps

D12    Bit 3: Supports V.29 half duplex at 7200 bps

Bit 2: Supports V.32 full duplex at 9600 bps

Bit 1: Supports V.32 half duplex at 9600 bps

Bit 0: Supports (Reserved) at 9600 bps

D13    Bit 3: Supports V.29 half duplex at 9600 bps

Bit 2: Supports Bell 103 110/300 bps

D15    Bit 1: Supports S95 Bit 5 for COMPRESSION: result code

Bit 0: Supports S95 Bit 4 for AUTOSTREAM: result code

D16    Bit 3: Supports S95 Bit 3 PROTOCOL: result code

Bit 2: Supports S95 Bit 2 CARRIER result code

Bit 1: Supports S95 Bit 1 CONNECT/ARQ result code

Bit 0: Supports S95 Bit 0 CONNECT XXXX (for DCE rate)

I4 "b" String

The second string, the b-string, is composed of the ASCII character "b"

followed by nine bytes (D1-D9). The bit map for each byte is defined as

follows:

b D1 D2 D3 D4 D5 D6 D7 D8 D9

D1    Bit 3: V.42 Alternate Protocol Supported

Bit 2: V.42 LAPM Protocol Supported

Bit 1: X.25 Protocol Supported

Bit 0: LAPB (Original V-series Point-to-Point error-control) Protocol

Supported

D2    Bit 3: Reserved (should be set to zero)

Bit 2: MNP Class 5 Supported

Bit 1: V.42bis Supported

Bit 0: Compression Through the X.25 Network

Supported

D3/D4 These combine to indicate the number of AutoStream Type A channels which

are supported. The formula (D3*16 + D4) is used. Zero means AutoStream

is not supported

I4 "r1" and "r3" strings

These ID strings allow software to determine the available speeds that may be

used to send AT commands. The r1-string contains a bit map that indicates at

which DTE rates the autobaud process is supported.

The r3-string is issued if synchronous DTE speeds are supported. The map

indicates which DTE rates are supported in Synchronous modes. Each r-string

begins with the lower case letter "r" and may be followed by as many as 39

additional characters, not counting <CR><LF>'s that will be used to separate

them from other strings. Those 39 additional characters are limited to the

ASCII-HEX alphabet 0-9 and A-F.

After the two lead-in characters (i.e., r<n>), all subsequent characters

contain the DTE rate maps. All maps have the same mapping for convenience to

software. (Refer to the chart below.) Not all bit assignments are possible:

for example, the split speeds have no meaning in the r3 synchronous map and

are always filled with zeros.

In r1, if the bit is filled with a 1, the corresponding DTE rate is supported

for sending AT commands to the device. A zero indicates that DTE rate is not

supported for AT commands.

In the r3 map, the bits simply indicate which DTE rates are supported for

synchronous operation on-line. This does not indicate, however, the supported

rates for synchronous V.25bis commands.

Split speeds if available are marked in the r1 string only when the

appropriate B command option has been selected. Future expansion of these

strings may include new speeds that are not in strict ascending order.

DTE Rate Bit Map for r1 and r3 Strings

Character    Bit #       DTE Rate

-----------------------------------------------------------------------------

3:1       bit 0       45.45 bps

3:2       bit 1       50

3:4       bit 2       75

3:8       bit 3       75/600 (xmt is 75, rcv is 600)

4:1       bit 4       75/1200

4:2       bit 5       110

4:4       bit 6       134.5

4:8       bit 7       50

5:1       bit 8       300

5:2       bit 9       450

5:4       bit 10      600

5:8       bit 11      600/75

6:1       bit 12      1200

6:2       bit 13      1200/75

6:4       bit 14      1800

``      6:8       bit 15      2000

7:1       bit 16      2400

7:2       bit 17      3000

7:4       bit 18      3600

7:8       bit 19      4200

8:1       bit 20      4800

8:2       bit 21      5400

8:4       bit 22      6000

8:8       bit 23      6600

9:1       bit 24      7200

9:2       bit 25      7800

9:4       bit 26      8400

9:8       bit 27      9000

10:1      bit 28      9600

10:2      bit 29      12000

10:4      bit 30      14400

10:8      bit 31      16800

11:1      bit 32      19200

11:2      bit 33      21600

11:4      bit 34      24000

11:8      bit 35      26400

12:1      bit 36      28800

12:2      bit 37      31200

12:4      bit 38      33600

12:8      bit 39      36000

13:1      bit 40      38400

13:2      bit 41      43200

13:4      bit 42      48000

13:8      bit 43      52800

14:1      bit 44      56000

14:2      bit 45      57600

14:4      bit 46      62400

14:8      bit 47      64000

15:1      bit 48      67200

15:2      bit 49      72000

15:4      bit 50      76800

15:8      bit 51      81600

16:1      bit 52      86400

16:2      bit 53      91200

16:4      bit 54      96000

16:8      bit 55      100800

17:1      bit 56      105600

17:2      bit 57      110400

17:4      bit 58      115200

17:8      bit 59      reserved ("0")

Actually these work under Windows if you replace the WIN.INI line defining the COM port,   For Example:

COM3:=19200,n,8,1,p                        may be changed to

COM3:=COM3.DOS

This tells Windows to get out of the way.  Great, Windows COM support is provided by dumping Windows.  Since this is not a Microsoft supported or documented feature, we cannot support it, however all the information we have is that it works fine.  Thanks to Colin Nowell of AMOCO, UK, Ltd. and Bo Bendtsen of Terminate for telling us about this.

Actually it tries a TIES sequence, because that is faster than the Hayes '302.  If TIES fails, it tries '302 (guard time, +++, guard time ATH).  If that fails it tries to drop the DTR.

DTE rates are the rates at which the CPU speaks to the COM port.  This may be different from the Baud rate because LAPM, or MNP compression can mean that data is sent from the CPU to the modem faster and uncompressed than the modems send it to eachother, slower but compressed.

In fact the turnaround time is the real reason why streaming (always going one way) protocols like Zmodem is faster than the ten year older Xmodem.(Ward Christensen, Omen Technologies and Chuck Forsberg credited.  What might Ward Christensen have produced if the industry had been wise enough to give him a living wage for playing in communications?).  As it is, when you are transmitting a bunch of small files even with Zmodem, a lot of time is wasted turning the line around.

Seventy milli-seconds doesn't seem like much.  When you realize that a 14,400 baud modem can transmit at 57,600 bits per second 70 milliseconds is enough time to send about 500 data characters which can be quite a few small files.   Furthermore Zmodem turns the line around many times per file.§  As ZModem chats to itself turning the line around 6 or more times per file, one could send 10 BAT files of 300 characters each in the time it takes ZModem to set up for a single file.

Zmodem is good but there is a lot of room for improvement.

DTE rates are the rates at which the CPU speaks to the COM port.  This may be different from the Baud rate because LAPM, or MNP compression can mean that data is sent from the CPU to the modem faster and uncompressed than the modems send it to eachother, slower but compressed.