MCS-51 Microcontroller Family Cross Assembler

AAA     SSSSSS  EEEEEEE  MM   MM        5555555   11

AA AA   SS       EE       MMM MMM        55       111

AA   AA  SS       EE       MM M MM        55        11

AA   AA   SSSSS   EEEEEEE  MM   MM  ====  555555    11

AAAAAAA       SS  EE       MM   MM             55   11

AA   AA       SS  EE       MM   MM             55   11

AA   AA  SSSSSS   EEEEEEE  MM   MM        555555   1111

U S E R ' S    M A N U A L

Version  1.1

October 14, 1994

copyright (c) 1994  by  W.W. Heinz

TABLE OF CONTENTS

-----------------

Foreword to Version 1.0

I.    Introduction

II.   Getting started

II.1  Files

II.2  Invoking ASEM-51

II.3  The HEXBIN Utility

II.4  The DEMO Program

III.  The ASEM-51 Assembly Language

III.1  Statements

III.2  Symbols

III.3  Constants

III.4  Expressions

III.5  The 8051 Instruction Set

III.6  Pseudo Instructions

III.7  Segment Type

III.8  Assembler Controls

III.9  Predefined Symbols

III.10 Conditional Assembly

IV.   Compatibility with the Intel Assembler

IV.1  Restrictions

IV.2  Extensions

IV.3  Further Differences

V.    List File Format

VI.   Support of 8051 Derivates

Appendix A:     ASEM-51 Error Messages

Appendix B:     HEXBIN Error Messages

Appendix C:     Predefined Symbols

Appendix D:     Reserved Keywords

Appendix E:     Specification of the Intel-HEX Format

Appendix F:     The ASCII Character Set

Appendix G:     Literature

Appendix H:     Trademarks

Appendix I:     8051 Instructions in numerical Order

Foreword to Version 1.0

=======================

Today microcontrollers are used in a wide range of applications from simple

consumer electronic products to complex avionic components. Thus I was not

very surprised to find an 80C31 on the videotext decoder board, I purchased

some time ago. Since it had a poor user interface and many bugs, I thought

I could do it better and so I began to look for an 8051 cross assembler.

But in contrast to the huge number of hardware components sold, the number

of people developing microcontroller software seemed to be remarkable small,

and so was the number of devlopment tools on the market.

There was a very small number of good professional cross assemblers for $250

and up - too expensive for hobby purposes. Aside of useless demos, there were

no restricted starter kits or school versions available.

I found also a few shareware and public domain assemblers. But either they

were poor and not very reliable, or not very 8051-specific, or they used some

kind of fantasy syntax that was 100 % compatible to itself, but far away from

the Intel standard. I didn't like them all!

There seems to be a general lack of useful and affordable microcontroller

development software. This is a pity, because their universality, simple

architectures and low prices make microcontrollers so interesting especially

for hobby and education.

So I decided to write a handy 8051 cross assembler for the PC.

And here it is:             ASEM-51 V1.0

I hope it will help to discover the wonderful world of microcontrollers.

Have fun!

Deisenhofen,  July 19, 1994

W.W. Heinz

I. Introduction

===============

ASEM-51 is a generic two-pass cross assembler for the MCS-51 microcontroller

family. It is running on IBM-PC/XT/AT computers and all true compatibles.

It only requires 256K of memory and MS-DOS 2.0 (or higher).

The ASEM-51 assembly language is a subset of the Intel standard, that

guarantees maximum compatibility with existing 8051 assembler sources.

ASEM-51 generates an Intel-HEX file output, which is directly accepted by

most EPROM burners. Thus ASEM-51 is suitable for small and medium 8051

microcontroller projects in the non-profit area.

However, ASEM-51 has been designed to process also very large programs!

Its most important features are:

- fast, compact, reliable, easy to use, affordable and well-documented

- easy installation, no configuration required

- command line operation, batch and networking capability

- Intel-compatible syntax

- five location counters, one for each of the 8051 address spaces

- built-in symbols for 8051 special function registers (can be disabled)

- 8051 register bank support

- assembly-time evaluation of arithmetic and logical expressions

- segment type checking for instruction operands

- automatic code optimization of generic jumps and calls

- nested include file processing

- nested conditional assembly

- detailed assembler listing with symbol table

- Intel-HEX file output

- hex-to-binary conversion utility

- direct support of eighteen 8051 derivates (will be extended)

- support of user-defined 8051 derivates

- update service by the author

- also running under OS/2 2.1       (in the DOS box)

- also running under Windows 3.1    (in the DOS box)

- also running under LINUX 1.1.12   (with dosemu 0.52)

II. Getting started

===================

The ASEM-51 package doesn't require a fuzzy software installation or

configuration. Simply copy all files of the package to your working

directory and enjoy the benefits of true plug-and-play compatibility!

II.1 Files

----------

Your ASEM-51 distribution disk or archive should contain the following files:

ASEM_51.DOC          ASEM-51 User's Manual  (this file)

LICENSE.DOC          ASEM-51 License Agreement

RELEASE.*            ASEM-51 Release Notes

ASEM.EXE          assembler

HEXBIN.EXE          hex-to-binary conversion utility

DEMO.A51          a sample 8051 assembler program

*.MCU          processor definition files of 8051 derivates

(for a detailed list of MCU files see chapter

"VI. Support of 8051 Derivates")

II.2 Invoking ASEM-51

---------------------

ASEM-51 provides full support of command line operation and batch

capability as the best commercial development tools.

ASEM-51 is invoked by typing:

ASEM <sourcefile> [<hexfile> [<listfile>]]

where <sourcefile> is the 8051 assembler source, <hexfile> is the output

file in Intel-HEX format, and <listfile> is the assembler listing.

The parameters <hexfile> and <listfile> are optional. When omitted, the

file names are derived from the <sourcefile> name, but with the extensions

HEX and LST. All file names may be specified without extensions. In these

cases, the assembler adds default extensions as shown below:

file                 extension

------------------------------

<sourcefile>         .A51

<hexfile>            .HEX

<listfile>           .LST

If you want a file name to have no extension, terminate it with a '.'!

Instead of file names you may also specify device names to redirect the

output to character I/O ports. Device names must be terminated with a ':'!

It is not checked, whether the device is existing or suitable for the task.

Although it is possible to read the source file from a character device

(e.g. CON:) instead of a file, this cannot be recommended: Since ASEM-51

is a two-pass assembler, it always reads the source file twice!

Examples:

1.)      ASEM PROGRAM

will assemble the 8051 assembly language program PROGRAM.A51 and

produce an Intel-HEX file PROGRAM.HEX and a listing PROGRAM.LST.

2.)      ASEM TARZAN.ASM JANE JUNGLE.PRN

will assemble the 8051 assembly language program TARZAN.ASM and

produce an Intel-HEX file JANE.HEX and a listing JUNGLE.PRN.

3.)      ASEM PROJECT EPROM.

will assemble the 8051 assembly language program PROJECT.A51 and

produce an Intel-HEX file EPROM and a listing PROJECT.LST.

4.)      ASEM sample COM2: NUL:

will assemble the 8051 assembly language program SAMPLE.A51, send

the HEX file output to the serial interface COM2 and suppress the

list file output by sending it to the NUL device.

When program errors are detected, they are flagged on the console. This

may look as follows:

MCS-51 Family Cross Assembler ASEM-51 V1.1

APPLICAT.A51(14): must be known on first pass

USERBITS.INC(6): attempt to divide by zero

DEFINES.INC(37): symbol not defined

APPLICAT.A51(20): symbol not defined

APPLICAT.A51(27): no END statement found

5 errors detected

Every error is flagged with the name of the source or include file, the

local line number where it was found, and the error message itself.

This output format makes it easy to integrate ASEM-51 into existing foreign

development environments or workbenches.

When terminating ASEM-51 returns an exit code to the operating system:

situation                           ERRORLEVEL

----------------------------------------------

no errors                               0

program errors detected                 1

fatal runtime error                     2

II.3 The HEXBIN Utility

-----------------------

Most EPROM burners accept the Intel-HEX object file format that is output

by ASEM-51. However, for dumb EPROM burners and special purposes it might be

useful to convert the HEX file to a pure binary image file.

For this the conversion utility HEXBIN is provided.

It is invoked as follows:

HEXBIN <hexfile> [<binfile>] [/OFFSET=<o>] [/FILL=<f>] [/LENGTH=<l>]

where <hexfile> is the input file in Intel-HEX format, and <binfile> is the

binary output file. The parameter <binfile> is optional. When omitted, the

file name is derived from the <hexfile> name, but with the extension BIN.

All file names may be specified without extensions. In these cases, the

program adds default extensions as shown below:

file                 extension

------------------------------

<hexfile>            .HEX

<binfile>            .BIN

If you want a file name to have no extension, terminate it with a '.'!

Instead of file names you may also specify device names to redirect the

output to character I/O ports. Device names must be terminated with a ':'!

It is not checked, whether the device is existing or suitable for the task.

The binary file output can also be controlled with the options /OFFSET,

/FILL and /LENGTH.

Normally the first byte in the binary file is the first byte of the HEX

record with the lowest load address. If a number of dummy bytes is to be

inserted on top of the file (e.g. for alignment in an EPROM image), this

can be performed with the /OFFSET option:

/OFFSET=1000

would insert 4096 dummy bytes before the first byte of the first HEX record

loaded. The offset must always be specified as a hex number. The default

offset is 0.

Since there may be peepholes between the HEX records, a fill byte value can

be defined with the /FILL option:

/FILL=0

would fill all peepholes between the HEX records with zero bytes as well as

all the dummy bytes that might have been inserted with the /OFFSET or /LENGTH

option. The fill byte value must always be specified as a hex number.

The default fill byte is the EPROM-friendly FFH.

By default the last byte in the binary file is the last byte of the HEX

record with the highest load address. If the binary file should have a

well defined length, then a number of dummy bytes can be appended to the

file (e.g. for exactly matching an EPROM length), this can be performed

with the /LENGTH option:

/LENGTH=8000

would append as many dummy bytes behind the last byte of the file, that the

total file length becomes exactly 32768 bytes. The file length must always

be specified as a hex number.

Options may be abbreviated as long as they remain unique!

The binary image file is always output in units of 128 bytes, the internal

HEXBIN record length. The original length of the loading data is therefore

rounded to 128 byte units.

Examples:

1.)      HEXBIN PROGRAM

will convert the Intel-HEX file PROGRAM.HEX to a pure binary image file

PROGRAM.BIN.

2.)      HEXBIN TARZAN.OBJ JUNGLE/FILL=E5

will convert the Intel-HEX file TARZAN.OBJ to a binary image file

JUNGLE.BIN and fill all peepholes between the HEX file records with

the binary value E5H.

3.)      HEXBIN PROJECT EPROM. /off=8000 /f=0 /length=10000

will convert the Intel-HEX file PROJECT.HEX to a binary image file

EPROM, insert 32K dummy bytes on top of file, fill all peepholes

and the dummy bytes with nulls, and extend the file to exactly 64K.

When terminating HEXBIN returns an exit code to the operating system:

situation                           ERRORLEVEL

----------------------------------------------

no errors                               0

runtime error                           1

II.4 The DEMO Program

---------------------

For getting started with a new assembler, it is always good to have a

program, that can be assembled with it. For this purpose, the 8051

assembler program DEMO.A51 is provided, that can be used for a first

test of ASEM-51. Be sure to have all files of the ASEM-51 package on your

disk! Then simply type

ASEM DEMO

HEXBIN DEMO

at the DOS prompt. ASEM and HEXBIN should finish without errors and you

should have the following new files on your disk:

DEMO.HEX          Intel-HEX file

DEMO.LST          assembler list file of DEMO.A51

DEMO.BIN          binary image file of DEMO.HEX

When something goes wrong, there may be files missing in your distribution!

DEMO.A51 may also serve as a sample assembler program that includes

examples for all machine instructions, pseudo instructions and assembler

controls, that have been implemented in ASEM-51. Whenever in doubt how to

use a particular command, DEMO.A51 may be a valuable help.

III.  The ASEM-51 Assembly Language

===================================

The user should be familiar with 8051 microcontrollers and assembly

language programming. This manual will not explain the architecture of

the MCS-51 microcontroller family nor will it discuss the basic concepts

of assembly language programming. It only describes the general syntax

of assembler statements and the assembler instructions that have been

implemented in ASEM-51.

III.1 Statements

----------------

Source files consist of a sequence of statements of one of the forms:

[symbol:]  [instruction [arguments]]     [;comment]

symbol     instruction  argument        [;comment]

$control   [argument]                    [;comment]

Everything that is written in brackets is optional.

The maximum length of source code lines is 255 characters.

Everything from the ';' character to the end of line is assumed to be

commentary. Blank lines are considered to be commentary, too.

The lexical elements of a statement may be separated by blanks and tabs.

Aside of character string constants, upper and lower case letters are

equivalent.

Examples:     HERE:   MOV A,#0FFH     ;define label HERE and load A with FFH

YEAR EQU 1994   ;define symbol for current year

$INCLUDE (80C517.MCU)   ;include SAB80C517 register definitions

III.2 Symbols

-------------

Symbols are user-defined names for addresses or numbers.

Their maximum significant length is 31 characters. They can be even

longer, but everything behind the first 31 characters is ignored.

Symbols may consist of letters, digits, '_' and '?' characters.

A symbol name must not start with a digit!

Upper and lower case letters are considered to be equivalent.

Note: Assembly language keywords must not be redefined as user symbols!

Example:     Is_this_really_a_SYMBOL_?       is a legal symbol name!

III.3 Constants

---------------

Numeric constants consist of a sequence of digits, followed by a radix

specifier. The first character must always be a decimal digit.

The legal digits and radix specifiers are:

constant     digits        radix

------------------------------------

binary       0 ... 1       B

octal        0 ... 7       Q or O

decimal      0 ... 9       D or none

hex          0 ... F       H

Thus, for example, the following constants are equivalent:

1111111B      binary

177Q      octal

177o      octal

127      decimal

127d      decimal

07FH      hex

Character constants may be used wherever a numeric value is allowed.

A character constant consists of one or two printing characters enclosed

in single or double quotes. The quote character itself can be represented

by two subsequent quotes. For example:

'X'       8 bit constant:       58H

"a@"      16 bit constant:     6140H

''''       8 bit constant:       27H

In DB statements, character constants may have any length.

In this case, we call it a character string. For example:

"It's only text!"

III.4 Expressions

-----------------

Arithmetic expressions are composed of operands, operators and parentheses.

Operands may be user-defined symbols, constants or special assembler symbols.

All operands are treated as unsigned 16-bit numbers.

Special assembler symbols, that can be used as operands are:

AR0, ... , AR7     direct addresses of registers R0 thru R7

$                  the location counter of the currently active segment

(start address of the current assembler statement)

The following operators are implemented:

Unary operators:   +          identity:             +x = x

-          two's complement:     -x = 0-x

NOT        one's complement:  NOT x = FFFFH-x

HIGH       high order byte

LOW        low order byte

Binary operators:  +          unsigned addition

-          unsigned subtraction

*          unsigned multiplication

/          unsigned division

MOD        unsigned remainder

SHL        logical shift left

SHR        logical shift right

AND        logical and

OR         logical or

XOR        exclusive or

.          bit operator used for bit-adressable locations

EQ  or =   equal to                ----.

NE  or <>  not equal to                |  results are:

LT  or <   less than                   |

LE  or <=  less or equal than          |     0 if FALSE

GT  or >   greater than                |  FFFF if TRUE

GE  or >=  greater or equal than   ____|

Operators that are no special characters but keywords as SHR or AND must

be separated from their operands by at least one blank or tab.

In general expressions are evaluated from left to right according to

operator precedence, which may be overridden by parentheses.

Parentheses may be nested to any level.

Expressions always evaluate to unsigned 16-bit numbers, while overflows

are ignored. When an expression result is to be assigned to an 8-bit

quantity, the high byte must be either 00 or FF.

Operator precedence:

-----------------------------------------------------------------

( )                                                   ^   highest

+ - NOT HIGH LOW                       (unary)        |

.                                                     |

* / MOD                                               |

SHL SHR                                               |

+ -                                    (binary)       |

EQ = NE <> LT < LE <= GT > GE >=                      |

AND                                                   |

OR XOR                                                v   lowest

-----------------------------------------------------------------

Example:  The expression  P1.((87+3)/10 AND -1 SHR 0DH)  will evaluate to 91H.

III.5 The 8051 Instruction Set

------------------------------

ASEM-51 implements all 8051 machine instructions including generic jumps

and calls. The assembler implements two instructions

JMP  <address>

CALL <address>

that do not represent a specific opcode: generic jump and call.

These instructions will always evaluate to a jump or call, not necessarily

the shortest, that will reach the specified address.

JMP may assemble to SJMP, AJMP or LJMP, while CALL can only evaluate to

ACALL or LCALL. Note that the assembler decision may not be optimal. For

code addresses that are forward references, the assembler always generates

LJMP or LCALL respectively. However, for backward references this is a

powerful tool to reduce code size without extra trouble.

The rest of the 8051 instruction mnemonics is listed in Appendix D.

Appendix I contains a table of all 8051 instructions with their opcodes,

mnemonics, arguments, lengths, affected flags and durations.

The comprehensive example program DEMO.A51 provided shows all the 8051

instructions in a syntactical context.

All 8051 instruction mnemonics are copyright (c) by Intel corporation!

III.6 Pseudo Instructions

-------------------------

In the subsequent paragraphs, all ASEM-51 pseudo instructions are described.

Lexical symbols are written in lower case letters, while assembler keywords

are written in upper case.

Instruction arguments are represented by <arg>, <arg1> or something like

that. Expressions are represented by <expr>, <expr1> and so on.

Syntax elements enclosed in brackets are optional.

The ellipsis "..." means always "a list with any number of elements".

DB <arg1> [,<arg2> [,<arg3> ... ]]              define bytes

The DB instruction reserves and initializes a number of bytes with

the values defined by the arguments. The arguments may either be

expressions (which must evaluate to 8-bit values) or character

strings of any length. DB is only allowed in the CODE segment!

Example:   DB 11,'june',94,(3*7+12)/11

DW <expr1> [,<expr2> [,<expr3> ... ]]           define words

The DW instruction reserves and initializes a number of words with

the values defined by the arguments. Every argument may be an

arbitrary expression and requires two bytes of space.

DW is only allowed in the CODE segment!

Example:   DW 0,0C800H,1994,4711

DS <expr>                                       define space

Reserves a number of uninitialized bytes in the current segment.

The value of <expr> must be known on pass 1!

DS is allowed in every segment!

Example:   DS 200H

DBIT <expr>                                     define bits

Reserves a number of uninitialized bits.

The value of <expr> must be known on pass 1!

DBIT is only allowed in the BIT segment!

Example:   DBIT 16

ORG <expr>                                      origin of segment

Sets the location counter of the current segment to the value <expr>.

Additional ORG statements may be used to generate program segments

which will load at different locations.

The value of <expr> must be known on pass 1!

The default value of all location counters at program start is 0.

Example:   ORG 08000H

USING <expr>                                    using register bank

Sets the register bank used to <expr>, which must be in the range

of 0...3. The USING instruction only affects the values of the

special assembler symbols AR0, ... , AR7 representing the direct

addresses of registers R0, ... , R7 in the current register bank.

The value of <expr> must be known on pass 1!

The default value for the register bank is 0.

Example:   USING 1

END                                             end of program

This must be the last statement in the source file. After the END

statement only commentary and blank lines are allowed!

Example:   END       ;end of program

symbol EQU <expr>                               define constant

symbol SET <expr>                               define variable

The EQU instruction defines a symbolic constant of the type NUMBER.

A symbol defined with EQU can never be changed!

The SET instruction defines a symbolic value of the type NUMBER,

that may be changed with subsequent SET statements.

The value of <expr> must be known on pass 1!

A symbol that has been SET, cannot be redefined with EQU!

A symbol that has been EQU'd cannot be reSET!

On pass 2, forward references to a SET symbol always evaluate

to the last value, that symbol has been SET to on pass 1.

Examples:   MAXMONTH  EQU 12

OCTOBER   EQU MAXMONTH-2

CHAPTER   SET  1

CHAPTER   SET  CHAPTER+1

symbol CODE  <expr>                             define ROM address

symbol DATA  <expr>                             define direct RAM address

symbol IDATA <expr>                             define indirect RAM address

symbol BIT   <expr>                             define bit address

symbol XDATA <expr>                             define external RAM address

These instructions define symbolic addresses for the five 8051

memory segments (address spaces). For DATA, IDATA and BIT type

symbols, the value of <expr> must not exceed 0FFH!

The value of <expr> must be known on pass 1!

Once defined with one of the above instructions, the symbols cannot

be redefined.

Examples:   EPROM    CODE  08000H

STACK    DATA       7

V24BUF   IDATA   080H

REDLED   BIT     P1.5

SAMPLER  XDATA  0100H

CSEG [AT <expr>]                           switch to CODE  space [at address]

DSEG [AT <expr>]                           switch to DATA  space [at address]

ISEG [AT <expr>]                           switch to IDATA space [at address]

BSEG [AT <expr>]                           switch to BIT   space [at address]

XSEG [AT <expr>]                           switch to XDATA space [at address]

These instructions switch to one of the five 8051 memory segments

(address spaces) and optionally set the location counter of that

segment to a particular address <expr>.

When the construction "AT <expr>" is omitted, the location counter

keeps its previous value.

The value of <expr> must be known on pass 1!

At program start the default segment is CODE and all the location

counters are set to zero.

Examples:   DSEG            ;switch to DATA segment

CSEG AT 8000h   ;switch to CODE segment at address 8000H

XSEG at 0       ;switch to XDATA segment at address 0

III.7 Segment Type

------------------

Every assembly time expression is assigned a segment type, depending on

its operands and operators. The segment type indicates the address space,

the expression result might belong to, if it were used as an address.

There are six possible segment types:

CODE

DATA

IDATA

XDATA

BIT

NUMBER    (typeless)

Most expression results have the segment type NUMBER. That means they are

assumed to be typeless. However, in some cases it may be useful to assign

a particular segment type!

The following six rules apply when the segment type is evaluated:

1. Numerical constants are always typeless.

Consequently their segment type is NUMBER.

2. Symbols are assigned a segment type during definition. Symbols

that are defined with EQU or SET have no segment type.

Labels get the segment type of the currently active segment.

3. The result of a unary operation (+, -, NOT, HIGH, LOW) will have

the segment type of its operand.

4. The results of all binary operations (except "+", "-" and ".") will

have no segment type.

5. If only one operand in a binary "+" or "-" operation has a segment

type, then the result will have that segment type, too. In all other

cases, the result will have no segment type.

6. The result of the bit operation "." will always have the segment

type BIT.

Examples:

---------       The following symbols have been defined in a program:

OFFSET  EQU   16

START   CODE  30H

DOIT    CODE  0100H

REDLED  BIT   P1.3

VARIAB4 DATA  20H

PORT    DATA  0C8H

RELAY   EQU   5

1.)  The expression  START+OFFSET+3  will have the segment type CODE.

2.)  The expression  START+DOIT  will be typeless.

3.)  The expression  DOIT-REDLED  will be typeless.

4.)  The expression  2*VARIAB4  will be typeless.

5.)  The expression  PORT.RELAY  will have the segment type BIT.

The segment type is checked, when expressions appear as addresses. When the

expression result is not typeless and does not have the segment type of the

corresponding segment, the instruction is flagged with an error message.

The only exceptions are the segment types DATA and IDATA, which are assumed

to be compatible in the address range of 0 to 7FH. Since ASEM-51 does only

support absolute segments, those addresses are really always pointing to the

same physical location in the internal memory.

Example:

--------

Line  Addr    Code            Source

1:                   30             DSEG AT 030H     ;internal RAM

2:    30             01     COUNT:  DS 1             ;counter variable

3:

4:                                  CSEG             ;ROM

5:  0000    C2 30           START:  CLR COUNT

^

@@@@@ segment type mismatch @@@@@

The CLR instruction is flagged with the error message "segment type mismatch"

in the assembler list file, because only a BIT type address is allowed here.

However, COUNT is a label with the segment type DATA!

III.8 Assembler Controls

------------------------

ASEM-51 implements a number of assembler controls that influence the

assembly process and list file generation. There are two groups of

controls: primary and general controls.

Primary controls can only be used once on top of the source file prior

to all other statements. Aside of other primary controls they may be

preceded only by blank and commentary lines.

General controls may be used multiple times everywhere in the program.

Primary controls:   $NOMOD51          Switches off the built-in 8051 special

function register definitions.

$NOPAGING         Switches off the page formatting in

the list file.

$PAGELENGTH (n)   Sets the list file page length to

n lines. (10 <= n <= 32767)

Default is n=64.

The control has no effect, when the

$NOPAGING control has been specified.

$NOSYMBOLS        Suppresses the symbol table at the

end of the list file.

General controls:   $INCLUDE (file)   Includes an external source file into

the assembler program just behind the

$INCLUDE statement.

Include files may also be nested.

$EJECT            Starts a new page in the list file.

The control has no effect, when the

$NOPAGING control has been specified.

$NOLIST           After this control statement source

code lines are no longer listed,

provided they contain no errors, until

$LIST             the next $LIST statement occurs.

Examples:   $NOMOD51              ;switch off 8051 SFR symbol definitions

$NOLIST               ;switch off listing

$INCLUDE (8052.MCU)   ;include 8052 SFR symbol definition file

$LIST                 ;switch on listing

III.9 Predefined Symbols

------------------------

For easy access to the 8051 special function register and interrupt

addresses, ASEM-51 has a number of predefined (built-in) DATA, BIT and

CODE symbols.

These predefined symbols can be switched off with the $NOMOD51 control.

For detailed information on symbols and addresses refer to Appendix C.

III.10 Conditional Assembly

---------------------------

To support easy configuration control and maintenance of 8051 application

programs, the following three meta instructions have been implemented for

conditional assembly:

IF <expr>

ELSE

ENDIF

Conditional assembly works as follows:

When the expression <expr> in the IF statement is not equal to 0 (TRUE),

then the statements 1 to n are assembled and the statements n+1 to n+m

are ignored.

IF <expr>

<statement 1>

<statement 2>       ;assembled when <expr> is TRUE

.

.

<statement n>

ELSE

<statement n+1>

<statement n+2>     ;assembled when <expr> is FALSE

.

.

<statement n+m>

ENDIF

Should <expr> be equal to 0 (FALSE), it is exactly vice versa! That means

the statements 1 to n are ignored and the statements n+1 to n+m are

assembled.

This works also, when the IF or ELSE branches contain no statements at all.

When the ELSE branch doesn't enclose any statements, the whole construction

can be simplified to a special case:

IF <expr>

<statement 1>

<statement 2>       ;assembled when <expr> is TRUE

.

.

<statement n>

ENDIF

Then the statements 1 to n are assembled, if <expr> is not equal to 0,

otherwise they are ignored.

The value of <expr> must be known on pass 1!

IF...ELSE...ENDIF constructions may be nested to any depth!

Example:     TARGET EQU 0    ;configuration:  1 for application board

;--------------  0 for evaluation board

IF TARGET

ORG 0         ;program start address of application board

ELSE

ORG 08000H    ;program start address of evaluation board

ENDIF

IV. Compatibility with the Intel Assembler

==========================================

The ASEM-51 assembly language is a subset of the Intel standard, that

guarantees maximum compatibility with existing 8051 assembler sources.

It implements all 8051 instruction mnemonics as well as a useful subset

of the Intel pseudo instructions and assembler controls.

IV.1 Restrictions

-----------------

Since ASEM-51 generates an Intel-HEX file output instead of relocatable

object modules, the whole source code of an 8051 application program has to

reside in one single file. Consequently all pseudo instructions, that deal

with relocatable segments or external symbols, have not been implemented:

NAME

PUBLIC

EXTRN

SEGMENT

RSEG

The SET command cannot redefine special assembler symbols (e.g. registers)!

Up to now only the following assembler controls have been implemented:

primary controls       |  general controls

-----------------------+-------------------

$NOMOD51               |  $INCLUDE (<file>)

$NOPAGING              |  $EJECT

$PAGELENGTH (<lines>)  |  $LIST

$NOSYMBOLS             |  $NOLIST

The corresponding Intel abbreviations for the above controls are not accepted!

Macros are not supported!

IV.2 Extensions

---------------

Assembler controls need not start in column 1, but may be preceded by any

number of blanks and tabs.

Character strings may also be enclosed in double quotes.

To support easy conditional assembly, the following three meta instructions

have been implemented:

IF <expression>

ELSE

ENDIF

IV.3 Further Differences

------------------------

To make semantics unique, especially the precedence of unary operators in

expressions is slightly different. Furthermore, expressions with a bit

operation "." evaluate to a BIT type result, not to NUMBER. This is described

in detail in chapters "III.4 Expressions" and "III.7 Segment Type".

Except in DB instructions, the zero length string constant '' is illegal.

V. List File Format

===================

The ASEM-51 list file format has been designed to give the user as much

information about the generated code as possible.

Besides the source code listed, there are four basic layout structures

in the listing:

- the page header

- the file header

- the line headings

- the symbol table

Normally every page of the listing starts with a page header as shown below:

ASEM-51  V 1.1          Copyright (c) 1994  by  W.W. Heinz          PAGE 1

It identifies the assembler, contains the copyright information and shows the

actual page number at the right margin. After the page header, source lines

are output in the list file format. When the maximum number of lines per page

is reached, another page header is output after a form feed character. When

the printer used doesn't work with form feeds, the page header can be

suppressed with the $NOPAGING control. The number of lines per page can be

adjusted to the paper format with the $PAGELENGTH control.

The file header appears only on the first page. It identifies the assembler,

lists all input and output files and marks the columns for the line headings.

A typical file header is looking as shown below:

MCS-51 Family Cross Assembler   A S E M - 5 1   V 1.1

=====================================================

Source File:   DEMO.A51

Object File:   DEMO.HEX

List File:     DEMO.LST

Line  Addr    Code            Source

Directly after the file header starts the listing of the source code lines.

Every source code line is preceded by a line heading. The line heading

consists of three columns: line number, line address and generated code.

The column "Line" contains the source file line number. It is not necessarily

the local line number within the particular source file, but a global line

number, that is counted over the main source and all include files.

The column "Addr" shows the start address of the listed line in the currently

active segment (8051 address space). All addresses are represented as hex

numbers. The addresses in the CODE and XDATA segments are four-digit numbers.

Addresses in all other segments are two-digit numbers. For lines that cannot

be assigned to a particular segment, the "Addr" field is left blank.

The "Code" column may contain up to four bytes of generated code, which is

sufficient for all 8051 instructions. The code is listed in hex byte

quantities starting from the left margin of the "Code" column.

However, the code generated for DB and DW instructions may be longer than

four bytes. In these cases, the source code line is followed by additional

line headings until the whole code of the line is listed.

The "Code" column does not always contain code, that consumes space in the

8051 CODE segment. In contrast to many other assemblers, ASEM-51 lists the

evaluation results of all expressions that may appear in pseudo instructions

or assembler controls. These values are listed in hex representation at the

right margin of the "Code" column.

The "Source" column finally contains the original source code line.

A typical source code listing is looking as follows:

Line  Addr    Code            Source

1:                                  ;A sample List File Demo Program

2:                                  ;-------------------------------

3:                          $NOMOD51                 ;no 8051 SFR

4:                   90             P1    DATA 090H  ;port 1 address

5:                   93             INPUT BIT  P1.3  ;pulse input

6:

7:                 8000             ORG  08000H      ;set location counter

8:  8000    80 20                   SJMP START       ;jump to start address

9:

10:  8002    01 07                   DB  1,7          ;define bytes

11:  8004    00 02 00 0C             DW  2,12,9       ;define words

8008    00 09

12:  800A    63 6F 66 66             DB  'coffeeright (c) 1994',0    ;string

800E    65 65 72 69

8012    67 68 74 20

8016    28 63 29 20

801A    31 39 39 34

801E    00

13:  801F           0003             DS  3            ;define space

14:

15:  8022    75 30 00        START:  MOV COUNT,#0     ;reset counter

16:  8025    30 93 FD        LLEVEL: JNB INPUT,LLEVEL ;wait for high

17:  8028    20 93 FD        HLEVEL: JB  INPUT,HLEVEL ;wait for low

18:  802B    05 30                   INC COUNT        ;count pulse

19:  802D    80 F6                   JMP LLEVEL       ;next pulse

20:

21:                   30             DSEG AT 030H     ;internal RAM

22:    30             01     COUNT:  DS 1             ;counter variable

23:

24:                                  END

If an error is detected in a source line, its position is flagged with a ^

character as good as possible, and a comprehensive error message is inserted.

This is looking as shown below:

15:  8022    75 30 00        START:  MOV COUNT,#0     ;reset counter

16:  8025    30 93 FD        LLEVEL: JNB INPUT,LLEVEL ;wait for high

17:  8028    20 93 00        HLEVEL: JB  INPUT,HLEUEL ;wait for low

^

@@@@@ symbol not defined @@@@@

18:  802B    05 30                   INC COUNT        ;count pulse

19:  802D    80 F6                   JMP LLEVEL       ;next pulse

The error diagnosis at the end of program lists the total number of errors

detected during assembly.

After the source code listing and error diagnosis, the symbol table

listing starts. The symbols are listed in alphabetical order with

their symbol name, segment type, hex value and definition line.

Predefined symbols are listed without a definition line number.

The symbol table listing can be suppressed with the $NOSYMBOLS control.

A typical symbol table listing is looking as shown below:

L I S T   O F   S Y M B O L S

=============================

SYMBOL                                  TYPE           VALUE         LINE

-------------------------------------------------------------------------

COUNT                                   DATA              30           22

HLEVEL                                  CODE            8028           17

INPUT                                   BIT               93            5

LLEVEL                                  CODE            8025           16

P1                                      DATA              90            4

START                                   CODE            8022           15

VI. Support of 8051 Derivates

=============================

Today a large number of 8051 derivates is available, that grows almost

monthly! They all use the same instruction set of the 8051 processor kernel,

but are different in peripheral components, to cover a wide range of

applications. The difference for the assembly language programmer is mainly

the varying set of special function registers and interrupt addresses.

It is always good practice to use the same SFR names in a microcontroller

application program that the manufacturer of the derivate used has defined.

For this the processor definition files *.MCU are provided. They all are

include files with the special function register definitions of a particular

8051 derivate. However, the predefined symbols of ASEM-51 must be switched

off prior to including the SFR definitions of another derivate as shown

below:

$NOMOD51

$INCLUDE (80C515.MCU)

This would switch off the predefined symbols of the 8051 and include the

register definitions of the 80C515 or 80C535 respectively.

Hence it is easy for the user to adapt ASEM-51 to a brandnew 8051 derivate!

All what he has to do is to write a corresponding include file with the SFR

definitions derived from the manufacturer's data sheet.

The name of every processor definition file is corresponding to the ROM

version of a particular derivate. Of course it also applies to the ROM-less

and EPROM version (if any) of that derivate.

By the way, the file 8051.MCU provided contains exactly the predefined

symbols of ASEM-51, because its internal symbol table has been generated

from it!

Currently the following processor definition files are provided with ASEM-51:

Name        Manufacturer    Versions

---------------------------------------------------------------------

8051.MCU    Intel           8051, 8031, 8751BH

(and others)    8051AH, 8031AH, 8751H, 8051AHP, 8751H-8

80C51BH, 80C31BH, 87C51, 80C51BHP

8052.MCU    Intel           8052AH, 8032AH, 8752BH

SIEMENS         80513, 8352-5

80C52.MCU    Intel           80C52, 80C32

83C51F.MCU    Intel           83C51FA, 80C51FA, 87C51FA

83C51FB, 87C51FB, 87C51FC

83C152.MCU    Intel           80C152JA, 83C152JA, 80C152JB

80C152JC, 83C152JC, 80C152JD

83C452.MCU    Intel           83C452, 80C452

8044.MCU    Intel           8044AH, 8344AH, 8744AH

80512.MCU    SIEMENS         80512, 80532

80515.MCU    SIEMENS         80515, 80535, 80515K, 83515-4

80C515.MCU    SIEMENS         80C515, 80C535, 83C515H

83C515A.MCU    SIEMENS         83C515A-5, 80C515A

80C517.MCU    SIEMENS         80C517, 80C537

83C517A.MCU    SIEMENS         83C517A-5, 80C517A

83C552.MCU    Philips         83C552, 80C552

80C521.MCU    AMD             80C521, 80C541, 87C521, 87C541, 80C321

80C324.MCU    AMD             80C324

83C154.MCU    OKI             83C154, 80C154, 85C154VS

80C320.MCU    DALLAS          80C320, 87C320, 80C323, 87C323

Will be extended!

Appendix A

==========

ASEM-51 Error Messages

----------------------

Assembly errors:

illegal control statement

illegal constant

string exceeds end of line

illegal character

constant out of range

illegal operator

symbol already defined

illegal statement syntax

commands after END statement

attempt to divide by zero

invalid bit number

invalid base address

too many opening parentheses

too many closing parentheses

binary operator expected

unary operator expected

symbol not defined

illegal operand

expression out of range

only allowed in CODE segment

operand expected

not allowed in BIT segment

only allowed in BIT segment

segment limit exceeded

only allowed on top of file

must be preceded by IF

ENDIF statement expected

too many operands

no END statement found

comma expected

address out of range

must be known on first pass

segment type mismatch

Runtime errors:

no parameters

file not found

too many open files

cannot create file

out of memory

no input file

too many parameters

unknown option

disk full

Appendix B

==========

HEXBIN Error Messages

---------------------

Conversion errors:

hex file format error

illegal hex digit

invalid record length

illegal record type

checksum error

record exceeds FFFFH

multiple EOF records

data after EOF record

no data records found

no EOF record found

Runtime errors:

no parameters

record exceeds file length

file not found

cannot create file

disk full

seek error

disk write error

disk read error

no hex file specified

unknown option

invalid argument

offset out of range

fill-byte out of range

file length out of range

too many parameters

Appendix C

==========

Predefined Symbols

------------------

DATA Addresses:

---------------

P0          080H                        P1          090H

SP          081H                        SCON        098H

DPL         082H                        SBUF        099H

DPH         083H                        P2          0A0H

PCON        087H                        IE          0A8H

TCON        088H                        P3          0B0H

TMOD        089H                        IP          0B8H

TL0         08AH                        PSW         0D0H

TL1         08BH                        ACC         0E0H

TH0         08CH                        B           0F0H

TH1         08DH

BIT Addresses:

--------------

IT0         088H                        EA          0AFH

IE0         089H                        RXD         0B0H

IT1         08AH                        TXD         0B1H

IE1         08BH                        INT0        0B2H

TR0         08CH                        INT1        0B3H

TF0         08DH                        T0          0B4H

TR1         08EH                        T1          0B5H

TF1         08FH                        WR          0B6H

RI          098H                        RD          0B7H

TI          099H                        PX0         0B8H

RB8         09AH                        PT0         0B9H

TB8         09BH                        PX1         0BAH

REN         09CH                        PT1         0BBH

SM2         09DH                        PS          0BCH

SM1         09EH                        P           0D0H

SM0         09FH                        OV          0D2H

EX0         0A8H                        RS0         0D3H

ET0         0A9H                        RS1         0D4H

EX1         0AAH                        F0          0D5H

ET1         0ABH                        AC          0D6H

ES          0ACH                        CY          0D7H

CODE Addresses:

---------------

RESET      0000H                        EXTI1      0013H

EXTI0      0003H                        TIMER1     001BH

TIMER0     000BH                        SINT       0023H

Appendix D

==========

Reserved Keywords

-----------------

Special Assembler Symbols:

--------------------------

$                                   location counter

A                                   accumulator

AB                                  A/B register pair

AR0,AR1,AR2,AR3,AR4,AR5,AR6,AR7     direct register addresses

C                                   carry flag

DPTR                                data pointer

PC                                  program counter

R0, R1, R2, R3, R4, R5, R6, R7      registers

Instruction Mnemonics

---------------------

ACALL       DA          JNB         MUL         RR

ADD         DEC         JNC         NOP         RRC

ADDC        DIV         JNZ         ORL         SETB

AJMP        DJNZ        JZ          POP         SJMP

ANL         INC         LCALL       PUSH        SUBB

CALL        JB          LJMP        RET         SWAP

CJNE        JBC         MOV         RETI        XCH

CLR         JC          MOVC        RL          XCHD

CPL         JMP         MOVX        RLC         XRL

Pseudo Instructions

-------------------

AT          CSEG        DS          EQU         SET

BIT         DATA        DSEG        IDATA       USING

BSEG        DB          DW          ISEG        XDATA

CODE        DBIT        END         ORG         XSEG

Operators

---------

AND         GT          LOW         NE          SHL

EQ          HIGH        LT          NOT         SHR

GE          LE          MOD         OR          XOR

Assembler Controls

------------------

$EJECT      $LIST       $NOMOD51    $NOSYMBOLS

$INCLUDE    $NOLIST     $NOPAGING   $PAGELENGTH

Meta Instructions

-----------------

ELSE        ENDIF       IF

Appendix E

==========

Specification of the Intel-HEX Format

-------------------------------------

This object file format is supported by many cross assemblers, utilities,

and most EPROM burners.

An Intel-HEX file is a 7-bit ASCII text file, that contains a sequence of

data records and an end record. Every record is a line of text that starts

with a colon and ends with CR and LF.

Data records contain up to 16 data bytes, a 16-bit load address, a record

type byte and an 8-bit checksum. All numbers are represented by upper case

ASCII-hex characters.

DATA RECORD:

Byte 1               colon (:)

2 and 3         number of binary data bytes for this record

4 and 5         load address for this record, high byte

6 and 7         load address  "    "     "     low byte

8 and 9         record type: 00  (data record)

10 to  x         data bytes, two characters each

x+1 to  x+2       checksum (two characters)

x+3 to  x+4       CR and LF

A typical data record looks like

:10E0000002E003E4F588758910F58DF58BD28E302A

The end record is the last line of the file.

In principle it is structured like a data record, but the number of data

bytes is 00, the record type is 01 and the load-address field is 0000.

END RECORD:

Byte 1               colon (:)

2 and 3         00   (number of data bytes)

4 and 5         00   (load address, high byte)

6 and 7         00   (load address, low byte)

8 and 9         record type: 01  (end record)

10 and 11        checksum (two characters)

12 and 13        CR and LF

The typical END record looks like

:00000001FF

The checksum is the two's complement of the 8-bit sum, without carry, of

the byte count, the two load address bytes, the record type byte and all

data bytes.

Appendix F:

===========

The ASCII Character Set

-----------------------

hex |   00   10   20   30   40   50   60   70

-----+------------------------------------------

0  |  NUL  DLE         0    @    P    `    p

1  |  SOH  DC1    !    1    A    Q    a    q

2  |  STX  DC2    "    2    B    R    b    r

3  |  ETX  DC3    #    3    C    S    c    s

4  |  EOT  DC4    $    4    D    T    d    t

5  |  ENQ  NAK    %    5    E    U    e    u

6  |  ACK  SYN    &    6    F    V    f    v

7  |  BEL  ETB    '    7    G    W    g    w

8  |   BS  CAN    (    8    H    X    h    x

9  |   HT   EM    )    9    I    Y    i    y

A  |   LF  SUB    *    :    J    Z    j    z

B  |   VT  ESC    +    ;    K    [    k    {

C  |   FF   FS    ,    <    L    \    l    |

D  |   CR   GS    -    =    M    ]    m    }

E  |   SO   RS    .    >    N    ^    n    ~

F  |   SI   US    /    ?    O    _    o  DEL

Appendix G:

===========

Literature

----------

Ordering Number

Intel:        MCS-51 Macro Assembler User's Guide

8-Bit Embedded Controllers 1990

SIEMENS:      SAB 80512/80532 User's Manual                B2-B3808-X-X-7600

SAB 80515/80535 User's Manual                B2-B3976-X-X-7600

SAB 80C515/80C535 Data Sheet

SAB 80C515A/83C515A-5 Addendum             B158-H6613-X-X-7600

SAB 80C515A/83C515A-5 Data Sheet           B158-H6605-X-X-7600

SAB 80C517/80C537 User's Manual            B258-B6075-X-X-7600

SAB 80C517A/83C517A-5 Addendum             B158-H6612-X-X-7600

SAB 80C517A/83C517A-5 Data Sheet           B158-H6581-X-X-7600

SIEMENS Microcontrollers Data Catalog      B158-H6569-X-X-7600

SAB 80513/8352-5 Data Sheet                B158-B6245-X-X-7600

Philips:      PCB83C552 User's Manual

PCB83C552, PCB80C552 Development Data

8051-based 8-bit Microcontrollers - Data Handbook 1991

AMD:          Eight-Bit 80C51 Embedded Processors - Data Book 1990

OKI:          MSM80C154, MSM83C154 User's Manual

Microcontroller Data Book, 5th Edition 1990

DALLAS:       High-Speed Micro User's Guide, V1.3 January 1994        011994

DS80C320 High-Speed Micro, PRELIMINARY                  081793

Andreas Roth: Das MICROCONTROLLER Kochbuch, 1989, iWT     ISBN 3-88322-225-9

Appendix H:

===========

Trademarks

----------

ASEM-51 is a trademark of W.W. Heinz.

MCS-51 is a trademark of Intel Corporation.

IBM-PC, IBM-XT, IBM-AT and OS/2 are trademarks of IBM Corporation.

MS-DOS and Windows are trademarks of Microsoft Corporation.

All device codes of 8051 derivates are trademarks of the manufacturers.

Other brand and product names are trademarks of their respective holders.

Appendix I:

===========

8051 Instructions in numerical Order

------------------------------------

Abbreviations:      direct   =   8-bit DATA address in internal memory

const8   =   8-bit constant in CODE memory

const16  =  16-bit constant in CODE memory

addr16   =  16-bit long CODE address

addr11   =  11-bit absolute CODE address

rel      =  signed 8-bit relative CODE address

bit      =   8-bit BIT address in internal memory

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

00      NOP                           1                    1

01      AJMP    addr11                2                    2

02      LJMP    addr16                3                    2

03      RR      A                     1                    1

04      INC     A                     1              P     1

05      INC     direct                2                    1

06      INC     @R0                   1                    1

07      INC     @R1                   1                    1

08      INC     R0                    1                    1

09      INC     R1                    1                    1

0A      INC     R2                    1                    1

0B      INC     R3                    1                    1

0C      INC     R4                    1                    1

0D      INC     R5                    1                    1

0E      INC     R6                    1                    1

0F      INC     R7                    1                    1

10      JBC     bit, rel              3                    2

11      ACALL   addr11                2                    2

12      LCALL   addr16                3                    2

13      RRC     A                     1     CY       P     1

14      DEC     A                     1              P     1

15      DEC     direct                2                    1

16      DEC     @R0                   1                    1

17      DEC     @R1                   1                    1

18      DEC     R0                    1                    1

19      DEC     R1                    1                    1

1A      DEC     R2                    1                    1

1B      DEC     R3                    1                    1

1C      DEC     R4                    1                    1

1D      DEC     R5                    1                    1

1E      DEC     R6                    1                    1

1F      DEC     R7                    1                    1

20      JB      bit, rel              3                    2

21      AJMP    addr11                2                    2

22      RET                           1                    2

23      RL      A                     1                    1

24      ADD     A, #const8            2     CY AC OV P     1

25      ADD     A, direct             2     CY AC OV P     1

26      ADD     A, @R0                1     CY AC OV P     1

27      ADD     A, @R1                1     CY AC OV P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

28      ADD     A, R0                 1     CY AC OV P     1

29      ADD     A, R1                 1     CY AC OV P     1

2A      ADD     A, R2                 1     CY AC OV P     1

2B      ADD     A, R3                 1     CY AC OV P     1

2C      ADD     A, R4                 1     CY AC OV P     1

2D      ADD     A, R5                 1     CY AC OV P     1

2E      ADD     A, R6                 1     CY AC OV P     1

2F      ADD     A, R7                 1     CY AC OV P     1

30      JNB     bit, rel              3                    2

31      ACALL   addr11                2                    2

32      RETI                          1                    2

33      RLC     A                     1     CY       P     1

34      ADDC    A, #const8            2     CY AC OV P     1

35      ADDC    A, direct             2     CY AC OV P     1

36      ADDC    A, @R0                1     CY AC OV P     1

37      ADDC    A, @R1                1     CY AC OV P     1

38      ADDC    A, R0                 1     CY AC OV P     1

39      ADDC    A, R1                 1     CY AC OV P     1

3A      ADDC    A, R2                 1     CY AC OV P     1

3B      ADDC    A, R3                 1     CY AC OV P     1

3C      ADDC    A, R4                 1     CY AC OV P     1

3D      ADDC    A, R5                 1     CY AC OV P     1

3E      ADDC    A, R6                 1     CY AC OV P     1

3F      ADDC    A, R7                 1     CY AC OV P     1

40      JC      rel                   2                    2

41      AJMP    addr11                2                    2

42      ORL     direct, A             2                    1

43      ORL     direct, #const8       3                    2

44      ORL     A, #const8            2              P     1

45      ORL     A, direct             2              P     1

46      ORL     A, @R0                1              P     1

47      ORL     A, @R1                1              P     1

48      ORL     A, R0                 1              P     1

49      ORL     A, R1                 1              P     1

4A      ORL     A, R2                 1              P     1

4B      ORL     A, R3                 1              P     1

4C      ORL     A, R4                 1              P     1

4D      ORL     A, R5                 1              P     1

4E      ORL     A, R6                 1              P     1

4F      ORL     A, R7                 1              P     1

50      JNC     rel                   2                    2

51      ACALL   addr11                2                    2

52      ANL     direct, A             2                    1

53      ANL     direct, #const8       3                    2

54      ANL     A, #const8            2              P     1

55      ANL     A, direct             2              P     1

56      ANL     A, @R0                1              P     1

57      ANL     A, @R1                1              P     1

58      ANL     A, R0                 1              P     1

59      ANL     A, R1                 1              P     1

5A      ANL     A, R2                 1              P     1

5B      ANL     A, R3                 1              P     1

5C      ANL     A, R4                 1              P     1

5D      ANL     A, R5                 1              P     1

5E      ANL     A, R6                 1              P     1

5F      ANL     A, R7                 1              P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

60      JZ      rel                   2                    2

61      AJMP    addr11                2                    2

62      XRL     direct, A             2                    1

63      XRL     direct, #const8       3                    2

64      XRL     A, #const8            2              P     1

65      XRL     A, direct             2              P     1

66      XRL     A, @R0                1              P     1

67      XRL     A, @R1                1              P     1

68      XRL     A, R0                 1              P     1

69      XRL     A, R1                 1              P     1

6A      XRL     A, R2                 1              P     1

6B      XRL     A, R3                 1              P     1

6C      XRL     A, R4                 1              P     1

6D      XRL     A, R5                 1              P     1

6E      XRL     A, R6                 1              P     1

6F      XRL     A, R7                 1              P     1

70      JNZ     rel                   2                    2

71      ACALL   addr11                2                    2

72      ORL     C, bit                2     CY             2

73      JMP     @A+DPTR               1                    2

74      MOV     A, #const8            2              P     1

75      MOV     direct, #const8       3                    2

76      MOV     @R0, #const8          2                    1

77      MOV     @R1, #const8          2                    1

78      MOV     R0, #const8           2                    1

79      MOV     R1, #const8           2                    1

7A      MOV     R2, #const8           2                    1

7B      MOV     R3, #const8           2                    1

7C      MOV     R4, #const8           2                    1

7D      MOV     R5, #const8           2                    1

7E      MOV     R6, #const8           2                    1

7F      MOV     R7, #const8           2                    1

80      SJMP    rel                   2                    2

81      AJMP    addr11                2                    2

82      ANL     C, bit                2     CY             2

83      MOVC    A, @A+PC              1              P     2

84      DIV     AB                    1     CY    OV P     4

85      MOV     direct, direct        3                    2

86      MOV     direct, @R0           2                    2

87      MOV     direct, @R1           2                    2

88      MOV     direct, R0            2                    2

89      MOV     direct, R1            2                    2

8A      MOV     direct, R2            2                    2

8B      MOV     direct, R3            2                    2

8C      MOV     direct, R4            2                    2

8D      MOV     direct, R5            2                    2

8E      MOV     direct, R6            2                    2

8F      MOV     direct, R7            2                    2

90      MOV     DPTR, #const16        3                    2

91      ACALL   addr11                2                    2

92      MOV     bit, C                2                    2

93      MOVC    A, @A+DPTR            1              P     2

94      SUBB    A, #const8            2     CY AC OV P     1

95      SUBB    A, direct             2     CY AC OV P     1

96      SUBB    A, @R0                1     CY AC OV P     1

97      SUBB    A, @R1                1     CY AC OV P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

98      SUBB    A, R0                 1     CY AC OV P     1

99      SUBB    A, R1                 1     CY AC OV P     1

9A      SUBB    A, R2                 1     CY AC OV P     1

9B      SUBB    A, R3                 1     CY AC OV P     1

9C      SUBB    A, R4                 1     CY AC OV P     1

9D      SUBB    A, R5                 1     CY AC OV P     1

9E      SUBB    A, R6                 1     CY AC OV P     1

9F      SUBB    A, R7                 1     CY AC OV P     1

A0      ORL     C, /bit               2     CY             2

A1      AJMP    addr11                2                    2

A2      MOV     C, bit                2     CY             1

A3      INC     DPTR                  1                    2

A4      MUL     AB                    1     CY    OV P     4

A5      illegal opcode

A6      MOV     @R0, direct           2                    2

A7      MOV     @R1, direct           2                    2

A8      MOV     R0, direct            2                    2

A9      MOV     R1, direct            2                    2

AA      MOV     R2, direct            2                    2

AB      MOV     R3, direct            2                    2

AC      MOV     R4, direct            2                    2

AD      MOV     R5, direct            2                    2

AE      MOV     R6, direct            2                    2

AF      MOV     R7, direct            2                    2

B0      ANL     C, /bit               2     CY             2

B1      ACALL   addr11                2                    2

B2      CPL     bit                   2                    1

B3      CPL     C                     1     CY             1

B4      CJNE    A, #const8, rel       3     CY             2

B5      CJNE    A, direct, rel        3     CY             2

B6      CJNE    @R0, #const8, rel     3     CY             2

B7      CJNE    @R1, #const8, rel     3     CY             2

B8      CJNE    R0, #const8, rel      3     CY             2

B9      CJNE    R1, #const8, rel      3     CY             2

BA      CJNE    R2, #const8, rel      3     CY             2

BB      CJNE    R3, #const8, rel      3     CY             2

BC      CJNE    R4, #const8, rel      3     CY             2

BD      CJNE    R5, #const8, rel      3     CY             2

BE      CJNE    R6, #const8, rel      3     CY             2

BF      CJNE    R7, #const8, rel      3     CY             2

C0      PUSH    direct                2                    2

C1      AJMP    addr11                2                    2

C2      CLR     bit                   2                    1

C3      CLR     C                     1     CY             1

C4      SWAP    A                     1                    1

C5      XCH     A, direct             2              P     1

C6      XCH     A, @R0                1              P     1

C7      XCH     A, @R1                1              P     1

C8      XCH     A, R0                 1              P     1

C9      XCH     A, R1                 1              P     1

CA      XCH     A, R2                 1              P     1

CB      XCH     A, R3                 1              P     1

CC      XCH     A, R4                 1              P     1

CD      XCH     A, R5                 1              P     1

CE      XCH     A, R6                 1              P     1

CF      XCH     A, R7                 1              P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

D0      POP     direct                2                    2

D1      ACALL   addr11                2                    2

D2      SETB    bit                   2                    1

D3      SETB    C                     1     CY             1

D4      DA      A                     1     CY       P     1

D5      DJNZ    direct, rel           3                    2

D6      XCHD    A, @R0                1              P     1

D7      XCHD    A, @R1                1              P     1

D8      DJNZ    R0, rel               2                    2

D9      DJNZ    R1, rel               2                    2

DA      DJNZ    R2, rel               2                    2

DB      DJNZ    R3, rel               2                    2

DC      DJNZ    R4, rel               2                    2

DD      DJNZ    R5, rel               2                    2

DE      DJNZ    R6, rel               2                    2

DF      DJNZ    R7, rel               2                    2

E0      MOVX    A, @DPTR              1              P     2

E1      AJMP    addr11                2                    2

E2      MOVX    A, @R0                1              P     2

E3      MOVX    A, @R1                1              P     2

E4      CLR     A                     1              P     1

E5      MOV     A, direct             2              P     1

E6      MOV     A, @R0                1              P     1

E7      MOV     A, @R1                1              P     1

E8      MOV     A, R0                 1              P     1

E9      MOV     A, R1                 1              P     1

EA      MOV     A, R2                 1              P     1

EB      MOV     A, R3                 1              P     1

EC      MOV     A, R4                 1              P     1

ED      MOV     A, R5                 1              P     1

EE      MOV     A, R6                 1              P     1

EF      MOV     A, R7                 1              P     1

F0      MOVX    @DPTR, A              1                    2

F1      ACALL   addr11                2                    2

F2      MOVX    @R0, A                1                    2

F3      MOVX    @R1, A                1                    2

F4      CPL     A                     1              P     1

F5      MOV     direct, A             2                    1

F6      MOV     @R0, A                1                    1

F7      MOV     @R1, A                1                    1

F8      MOV     R0, A                 1                    1

F9      MOV     R1, A                 1                    1

FA      MOV     R2, A                 1                    1

FB      MOV     R3, A                 1                    1

FC      MOV     R4, A                 1                    1

FD      MOV     R5, A                 1                    1

FE      MOV     R6, A                 1                    1

FF      MOV     R7, A                 1                    1