Syntax: hddcheck [/?] [/help] [/h]

/?, /h or /help prints this text.

HDDCHECK, copyright (c) 1997 by Frank Berreth.

WHAT DOES THIS PROGRAM

======================

'HDDCHECK' reads your complete hard disk structure, regardless of installed

operating systems, and displays any known errors. In addition to this,

'HDDCHECK' analyzes the damage status of the magnetic surface on your hard

disk and forecast a drop out within the next few weeks.

HOW DOES IT WORK

================

'HDDCHECK' reads any sector of your hard disk. If a sector appears to be

erroneous, an intelligent IDE drive tries to compensate the fault using

internally stored error correction code (ECC) and copies this sector to a

previously reserved area, which is (hopefully) error free. This operation is

completely transparent to all operating systems like WinNT or Linux and is

called remapping.

There'll be a good chance to keep data undamaged (even hardly used files) if

you check your hard disk periodically with this program.

Into deep ...

-----------------------

Usually, the hard disk's firmware reserves a few sectors for the remapping of

faulty sectors. No operating system is aware of this extra space, because

the remapping is transparently handled by the firmware. Of course, if these

special sectors are all in use, no attempts will be launched to ECC-correct

the sector and remap it afterwards. A disk with a lot remaps is also slower

than a less erroneous drive because of extra reload operations.

Things get worse if there is actually a 'weakened' sector which should be

remapped, but all reserved sectors are used. Your hard disk could only try

to read this sector until it can be corrected with ECC. Have in mind that

information is still stored analogous, so this makes sense (Note: This

re-read operations are time consuming, the context with HDDCHECK will be ex-

plained later). If your hard disk has read this sector several times but

could not correct it, an error will be reported to the os. If a hard error

is reported, there'll be only a very slim chance to get intact data of this

sector with repeated read operations. Fact: All disk utilities like 'scandisk'

can/will only display hard errors. At that time it's already to late and your

data is lost.

This is where HDDCHECK kicks in

-------------------------------

This theory has lead to a program which reads your hdd in huge blocks (for

DOS this is 32KByte) and measures the time for each operation. Those time

values are stored in a diagram that is written into a log file.

The time units used are timer ticks, where one tick is about 0.84 microsec.

The diagram first shows the time and than how many read operations have been

accomplished in this interval. The protocolled time varys from 3000 ticks to

50000 ticks. Each hdd has it's individual distribution of these values and

it'll vary in each test. Normally, the peak should be at 5000-7000 ticks for

fast hhd's and at 10000-15000 ticks for slow hdd's.

You have told me a lot, what about some test examples

-----------------------------------------------------

Enough on theory, some real world tests showed that this program is genious.

In my first case I tested a hdd which behaves suspiciously. The used

operating system (WinNT) sometimes made entries in the event log indicating

hd problems, but mostly WinNT crashed without even a warning or a blue

screen. The log file after the test reviewed _many_ values above 50000 ticks

(about 800 read operations took more than 50000 ticks). Note that the hdd

only sometimes reported one or more bad sectors (read errors) on a regular

basis to the os. This explains why NT only sometimes reported an error into

the event log. Note also, that the position of the bad sector reported by the

hdd varied which seems to be senseless. In conclusion, normal test programs

like 'scandisk' mostly don't report errors. You also have trouble to test

more than one partition or a whole hdd regardless of the used file system.

Think about it: The traditionally scanned hdd could have a partially damaged

surface and you will never know for sure. Have in mind that 'scandisk' cannot

help you, because reported errors by scandisk are typically real, uncorrect-

able errors as stated above.

After this initial test there were many other checks following, but the hdd's

I have had for testing seemed to work OK over a half year. Note, that there

must be proofed, that hdd's first show some hard errors and than more and more

hard errors and that HDDCHECK tells you that there will be errors just before

the first one shows up.

OK, I must apologize, but until now I only have one another example which is

not the best you can get. We have bought two equal hdd's at the same time,

where one of them was build in a WinNT server and the other one was stored

for future use. After 3 month the server behaves suspiciously. Once again,

HDDCHECK, which was first run on that hdd, tells that this hdd is destined to

fill our trash can. The second hdd, which was unused till now and checked by

HDDCHECK got the same judgment, although this hdd does not report any

errors. I'm sorry that I did not check the WinNT server hdd before it was

build in.

I know, obviously this test does not tell much because the surface need not

to be of the same quality, but there is a good chance.

Is there something I should know

--------------------------------

OK, I say that we can try to classify between a good hdd (with intact

surface) and a hdd with defecting surface. Note that the test does not tell

you anything about a head crash or motor loss, that means these are dangerous

failures now and forever.

If you start 'HDDCHECK', the program scans all hdd's in your system. After

the scan of one hdd, a protocol file is created, named 'hddX.chk', where X is

the number of your hdd. If there exists a log file, the protocol is appended.

If your hard disk really reports an error, the error text is immediately

printed on screen. After the test, a quick result is printed. Note that this

classification of your hdd is done with really no great use of artificial

intelligence. It is recommended that you personally view the log and make

your own opinion. Have in mind that there should not be more than 30-50

read operations beyond 30000 ticks on a fast hdd. Because your old log is

not deleted, you can compare these logs to see if something gets worse. Check

your hdd periodically (let's say every week) and backup all your critical

data (think of head crash or crazy programs).

GOOD LUCK

LEGAL STUFF

===========

This program was written by Frank Berreth. All rights are reserved.

You have permission to use and distribute 'HDDCHECK' (executable format)

for free. You cannot sell it, but you can demand a handling charge for

distributing it on a media.

This software is provided 'as is' without express or implied warranty.

The author is not responsible for any damage caused by this program.

The idea and the algorithm, as described above, is a copyright of Frank

Berreth. You are not allowed to implement this algorithm in any of your

programs. If someone wants to port this application to e.g. Linux, mail

me!

Frank Berreth

Fruehlingstr. 4

85354 Freising

-GERMANY-

e-mail: fberreth@mikron.de

The address may change in future.

If you find this program useful or braindead, drop me a mail in both cases!

Thanks.