Disk Opt v1.0 � Copyright 1988 by Jack A. Orman

Correct usage is:

DISKOPT /Sx /Tx /Rx

/S=Steprate. X can equal 3, 6, 9, or 12

/T=Disk motor off-time. X can equal 15 to 25

/R=Refresh rate.  X can equal 01 to 99

**  D I S K O P T  **

Diskopt allows you to set the steprate of the floppy

drives on your system. This can speed up you disk access

if the drive is capable of handling the faster steprate

(lower number = faster access).  If the drive makes a

grinding or other abnormal noise, set the steprate to the

next higher selection. Only 3, 6, 9 or 12 are valid rates.

Diskopt can also set the delay time that is used to

determine how long the floppy drive motor continues to run

after the disk access is finished (Lower number = faster

access). You don't want to set this number so low that the

drive motor is starting and stopping excessively, but you

can cut down on unnecessary drive idling. Determine the

optimum value by setting the /T value to 22; then type

DIR. Watch the drive light to see if it stays lit

continuously or if it goes on and off. If it stays on,

select the next lower value. Continue this process until

the drive light begins to flicker on and off. The next

higher number for /T is correct for your system.

Refresh rates:  CAUTION! This is a setting that can cause

loss of data if it is set too HIGH. Tandy 1000 users should

be especially careful when playing with this setting. It

can be used to increase the actual processing speed of

your computer by increasing the time between refreshes of

the RAM. This allows the CPU to do more productive work

instead of refreshing those dynamic ram chips so often.

Use a program such as CPUMHZ.com to test the speed of your

system and then vary the refresh rate (start low and work

up). Apparent speed increases can be achieved in this

manner.  If you set the time between ram refreshes to a

value that is too high, the capacitors that are the memory

cells in the ram will lose their charge (and consequently

the data bytes).

W A R N I N G:  Use this program at your own risk! The

drive will probably not be damaged by any setting of

steprate or drive-off time but be cautious. If the drive

sounds like it's struggling, it probably is. So back off

on the steprate.

W A R N I N G   !!!   W A R N I N G

Messing with the refresh rate will cause data loss if you

set it too high!  However, rebooting will cause all of the

setting to return to normal. Don't be afraid to play

around with the refresh rate to get better speed, but

don't use the refresh option until you are SURE that data

bits are not being lost. I will NOT be responsible for any

data loss, drive damage or other malfunctions resulting

from use of this program. USER BEWARE!

*   Copyright Notice

These programs and the documentation are the

copyrighted property of Jack A. Orman and are not to

be resold without express written permission. The

complete archive MAY be uploaded to a BBS or

commercial information service if all of the files

are included and are not modified in any way.

*   Shareware Notice

DISKOPT.arc is from the ARMADA UTILITIES Disk #2 which

may be obtained by sending a $20.00 check or money

order to:

Jack A. Orman

Box 858

Southaven, MS 38671

*   ARMADA UTILITIES Disk #2 contains:

BOOT-C  .arc                BOOTREX .arc

COP     .arc                EKHO    .arc

EOF     .arc                NOLF    .arc

SIGNAL  .arc                LOCATE  .arc

DISKOPT .arc                PRNmaker.arc

TXTmaker.arc                MAILIT  .arc

and more!

*   ARMADA UTILITIES Disk #1 available for $20.00 contains:

ALTPAGE .arc                AUTHOR  .arc

BOOT    .arc                CASE    .arc

CASIOID .arc                CRLF    .arc

CRT31   .arc                CURSOR  .arc

DOCMAKER.arc                FRQCNV4 .arc

FXPRNT22.arc                MIDIPOP .arc

PRINTIT .arc                RB1000  .arc

TSRMAKER.arc                README

TSRHELP .DOC