D I S K O T I M*

USER MANUAL

for PC-DOS* and MS-DOS* Systems

Copyright (C) 1983 by Ray Hallman

No part of this publication may be reproduced,

transmitted,    transcribed,   stored   in   a

retrieval   system  or  translated  into   any

language or computer or computer language,  in

any   form  or  by  any   means,   electronic,

mechanical,   magnetic,   optical,   chemical,

manual or otherwise, except as provided herein

without   prior  written  permission  of   Ray

Hallman,  10120  Pasture Gate  Lane  Columbia,

Maryland, 21044  ( COMPUSERVE --> 71445,1060 )

Disclaimer

Ray   Hallman  makes  no   representation   or

warranties with respect to the contents hereof

and   specifically   disclaims   any   implied

warranties  or merchantability or fitness  for

any particular purpose.   Further, Ray Hallman

reserves the right to revise this  publication

and  to make changes from time to time in  the

content  hereof  without obligation to  notify

any person of such revision or changes.

Version 1.1

May 28, 1983

*DISKOTIM is a trademark of Ray Hallman

*MS-DOS is a trademark of MICROSOFT.

*PC-DOS is a trademark of I.B.M.

DISKOTIM - 2

TABLE OF CONTENTS

1.   WARRANTIES, GUARRANTEES, and LIABILITIES

2.   LICENSE AGREEMENT

3.   GENERAL DESCRIPTION AND FEATURES OF "DISKOTIM"

4.   LOADING INSTRUCTIONS

5.   MAIN MENU

6.   DISK DRIVE SPEED TESTS

7.   DISK DRIVE HEAD POSITION AND TRACK SELECTION

8.   SELECTION OF "QUAD DENSITY" AND BACK TO "DUAL DENSITY" DRIVE

PARAMETERS.

9.   USE OF TEST DISKETTES WITH DISKOTIM

10.  QUITTING FROM THE DISKOTIM PROGRAM BACK TO THE SYSTEM

11.  SOME COMMENTARY ON THE DRIVE SPEED ADJUSTMENT

12.  SOME COMMENTARY ON THE DRIVE ALIGNMENT PROCEDURES

DISKOTIM - 3

1.   WARRANTIES, GUARRANTEES, and LIABILITIES

THE  PROGRAM  IS  PROVIDED "AS IS" WITHOUT WARRANTY  OF  ANY

KIND,  EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO

THE  IMPLIED  WARRANTIES  OF MERCHANTABILITY AND  FITNESS  FOR  A

PARTICULAR  PURPOSE.   THE  ENTIRE  RISK AS TO  THE  QUALITY  AND

PERFORMANCE OF THE PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE

DEFECTIVE, YOU ASSUME THE ENTIRE COST OF ALL NECESSARY SERVICING,

REPAIR OR CORRECTION.

IN  NO  EVENT  WILL RAY HALLMAN BE LIABLE  TO  YOU  FOR  ANY

DAMAGES,  INCLUDING  ANY  LOST  PROFITS,  LOST SAVINGS  OR  OTHER

INCIDENTAL  OR  CONSEQUENTIAL DAMAGES ARISING OUT OF THE  USE  OR

INABILITY  TO  USE  SUCH  PROGRAM EVEN IF RAY  HALLMAN  HAS  BEEN

ADVISED OF THE POSSIBILITY OF SUCH DAMAGES,  OR FOR ANY CLAIM  BY

ANY OTHER PARTY.

Information  in  this document is subject to change  without

notice  and  does  not represent commitment on the  part  of  RAY

HALLMAN.   The  software described in this document is  furnished

under license agreement.  The software may be used or copied only

in  accordanced with the terms of the agreement.   It is  against

the law to copy this software on magnetic tape,  disk,  diskette,

or  any  other medium for any purpose other than the  purchaser's

own  personal  use  and then only after  having  agreed  to  said

license agreement.

I sincerely hope that you find the program package contained

herein  to  be  of  use to you.   I have  decided  to  make  this

available  to you and any users of the "distributor system"  from

which  you  obtained it because I have personally  received  much

good  software from such systems and would like to repay some  of

this  benefit.   Also,  it is my hope that this will promote good

will among good programmers to distribute their good efforts in a

similar  manner.   As others before me have said,   "if you  find

this  program helpful and would like to continue to promote  this

kind  of effort you may contribute something to my  above  stated

address".   I  would be delighted to receive $10 or so by  return

mail.   This  would  certainly  encourage me  to  continue  these

efforts  as  I do have some other good ideas!   I do have  a  few

copies  of the distribution version of this program with hardcopy

manual  which I will send to you for a donation of  $15.   Please

bear  in mind that there is no obligation as long as its only for

your own use.   Thanks!  In any case your comments on this either

by telephone, email, or mail would be pleasantly accepted.

Sincerely, Ray Hallman

DISKOTIM - 4

2.   LICENSE AGREEMENT

YOU  SHOULD READ THE FOLLOWING TERMS AND CONDITIONS BEFORE MAKING

USE OF THE PROGRAMS CONTAINED IN THIS PACKAGE.  USING THE PROGRAM

PACKAGE  INDICATES  THAT  YOU  ACCEPT  THE  FOLLOWING  TERMS  AND

CONDITIONS.

1.   RAY  HALLMAN provides these program softwares  "as  is"

without  warranty of any kind.   The entire risk as  to

the quality and performance of these programs is yours.

2.   If  the  programs prove defective,  you  (and  not  RAY

HALLMAN  or any distributor) assume the entire cost  of

all necessary servicing, correction, or repair.

3.   You may: A) Use these program softwares on a machine at

a single location,  B) Copy these program softwares for

backup  or modification purposes in support of your use

of them at a single location.

4.   You may NOT:  A) Use,  copy,  modify, or transfer these

program softwares, or any copy, modification, or merged

portion,  in  whole  or in part,  except  as  expressly

provided for in this license,  B) Sub-license,  assign,

or transfer the license or these program softwares.

5.   This   program  is the property of RAY  HALLMAN,  10120

Pasture Gate Lane, Columbia, Maryland, 21044  (301-730-

5624)  (  COMPUSERVE --> 71445,1060 )  all  rights  are

retained  except as provided herein.  A limited license

for  use of versions released prior to May.,  1983   is

granted.   These  early  versions  may  be  distributed

to  software users only,  providing that both the  .EXE

file  and the .DOC files are not separated or  modified

and no costs other than media are charged.   Reports of

bugs will be appreciated. No maintenance is committed.

ANY  ATTEMPT  TO DO ANY OF THE ABOVE (SECTION 4)  WILL  VOID

THIS AGREEMENT.

RAY  HALLMAN's ENTIRE LIABILITY AND YOUR EXCLUSIVE REMEDY IS

THAT YOU MAY TERMINATE THIS AGREEMENT BY RETURNING THESE  PROGRAM

SOFTWARES.

IN  NO  EVENT  WILL RAY HALLMAN BE LIABLE FOR  ANY  DAMAGES,

INCLUDING  ANY LOST PROFITS,  LOST SAVING,  OR ANY OTHER  DAMAGES

ARISING  OUT  OF  THE  USE OR  INABILITY  TO  USE  THESE  PROGRAM

SOFTWARES.

DISKOTIM - 5

3.   GENERAL DESCRIPTION AND FEATURES OF "DISKOTIM"

Congratulations on your acceptance of "DISKOTIM" which shows

your  excellent  judgment  of exceptional  value.   For  you  are

getting two valuable test and diagnostic tools for 5" drives:

1.   A  handy  Diskette Drive speed measurement  tool  which

indicates  very  precisely the Rotation Speed in a  non

destructive manner without writing on the diskette.

2.   A very useful Drive Stepper Utility that enables you to

exercise  your drives without the expensive Disk  Drive

Test Set usually required for this purpose.

"DISKOTIM"  will work with any Single Sided or Double  Sided

Floppy  Drive connected to any IBM-PC Compatible Computer  System

running  under  PC-DOS  (MS-DOS).   The program  is  really  self

explanatory due to its verbose menu driven format.   THIS PROGRAM

UTILITY HAS NOT BEEN TESTED WITH THE PC-jr AND IS NOT EXPECTED TO

WORK SINCE THERE IS NO DMA CIRCUIT IN THE PC-jr.

This  instruction  booklet  explains  every  detail  of  the

program  usage and even goes so far as to provide some commentary

on  Drive  Speed Adjustment and Alignment although you  the  user

must assume all risk and liability for such adjustment.   In  any

case you can easily see if your drive is operating at the correct

speed  and thereby eliminate this very important area of concern.

The disk drive time check can be done in only a few moments using

any PC-DOS (MS-DOS) formatted diskette whenever there is a doubt.

No  copy protection scheme is employed so that you may  copy  the

program  to your hard-disk for easy usage.   The program does not

measure  the  speed  of  hard-disks  which  are  inherently  more

accurate  and  therefore  rarely  need  attention.   Some  Floppy

Diskette Drives made by some manufacturers are however  notorious

in  this  regard as you may see using this program - if  you  are

unfortunate to own one of the bad ones.

The  "Drive Stepping" Utility is more complicated in purpose

and  while you may easily exercise your drives with no  risks  of

damage  to  them,  the real value comes when used in  conjunction

with  a  special  alignment  diskette  which  may  be   purchased

separately.  More about this in a later section of this manual.

Finally,  these  utilities  will  work well with  all  drive

position  assignments A:,  B:,  C:,  and D:  from the  easy  menu

selection.   No read/write tests are provided by "DISKOTIM" since

you  may  verify  read/write using the  normal  "FORMAT"  utility

provided with the PC-DOS/MS-DOS System.

DISKOTIM - 6

4.   LOADING INSTRUCTIONS

DISKOTIM

IBM DISKETTE

LOADING INSTRUCTIONS

This  program  will  not auto-boot as shipped.   To make  a  self

booting disk, use the following procedure:

1.   Boot  the  IBM-PC compatible machine with  your  system

diskette.

2.   If  you  have  a  2 disk  system,  place  the  DISKOTIM

diskette in drive B.   (see NOTE below)

3.   Transfer   your  DISKOTIM.EXE  file  to  the   "SYSTEM"

diskette by entering the following command:

COPY B:DISKOTIM.EXE  A:   & CARRIAGE RETURN <ENTER>

4.   "DISKOTIM"  is started simply by booting the disk  made

in step 3 above and then typing DISKOTIM.

5.   It  is  strongly  suggested that you  store  away  your

original  copy  now and not use the  original  for  any

testing to protect against accidental loss of DISKOTIM.

NOTE:     For  single drive systems,  the DOS SYSTEM Diskette  is

considered  as drive A:,  and the DISKOTIM Diskette  as

drive  B:.    Follow  the  prompts  displayed  by  your

Computer System.

DISKOTIM - 7

5.   MAIN MENU

The  Main  Menu  is displayed as a result of either  of  two

operator actions:

1.   When  the  program  is  initially  invoked  by   typing

"DISKOTIM"  from the Operator Console after the  system

is booted.

2.   When  "H" is entered from the console after the  prompt

"Next Command?"

The Main Menu presents 3 groups of commands:

1.   Commands A, B, C, and D are the Time/Speed Tester.

2.   Commands I, O, K, L, i, o, k, and l are the Stepper.

3.   Commands H and Q provide Helps and Quits.

The Main Menu:

DISKOTIM

PERSONAL COMPUTER DISK TESTING UTILITY, V 1.1

"DISKOTIM" HAS 14 BASIC COMMANDS:

A       DISPLAYS THE ROTATION SPEED OF DRIVE A:

B       DISPLAYS THE ROTATION SPEED OF DRIVE B:

C       DISPLAYS THE ROTATION SPEED OF DRIVE C:

D       DISPLAYS THE ROTATION SPEED OF DRIVE D:

I       STEPS DRIVE A: IN ONE TRACK

O       STEPS DRIVE A: OUT ONE TRACK

K       STEPS DRIVE B: IN ONE TRACK

L       STEPS DRIVE B: OUT ONE TRACK

i       STEPS DRIVE C: IN ONE TRACK

o       STEPS DRIVE C: OUT ONE TRACK

k       STEPS DRIVE D: IN ONE TRACK

l       STEPS DRIVE D: OUT ONE TRACK

H       HELPS by re-printing the entry INSTRUCTIONS

Q       Quits and RETURNS to the SYSTEM

Next Command?

DISKOTIM - 8

6.   DISK DRIVE SPEED TESTS

The  Drive  Rotation  Time (Inverse Speed) Test  Utility  is

entered when A, B, C, or D is Keyed from the Main Menu.  When "A"

is entered the following appears:

DISKTIME for DRIVE A:

Insert a Formatted diskette in A

Then strike any key.

At this point the program waits for the operator to indicate

that  a diskette is present in Drive A.   When any key is struck,

the rest of the Disk Drive Timing Screen appears:

STRIKE ANY KEY TO END TEST

DISK ROTATION TIME IN MILLISECONDS

D I S K - O - T I M E

197       198       199       200       201       202       203

|....:....|....:....|....:....!....:....|....:....|....:....|

!

As  can  be seen above,  The Disk rotation is  specified  in

milliseconds by the little "exclamation point" symbol shown under

the  200  value.   When the rotation speed is  slow,  the  symbol

"needle pointer" moves to the right.   The ideal speed setting is

200  milliseconds  as shown.   For most drives,  it is OK if  the

meter  reading is within + or - 1 ms or in the range 199  to  201

milliseconds.   On  the  following  2  pages,  the  entire  Timer

Function  displays are presented for Commands A,  B,  C,  and  D,

which are the four Drive Speed Functions.

DISKOTIM - 9

DISKTIME for DRIVE A:

Insert a Formatted diskette in A

Then strike any key.

STRIKE ANY KEY TO END TEST

DISK ROTATION TIME IN MILLISECONDS

D I S K - O - T I M E

197       198       199       200       201       202       203

|....:....|....:....|....:....!....:....|....:....|....:....|

!

=================================================================

DISKTIME for DRIVE B:

Insert a Formatted diskette in B

Then strike any key.

STRIKE ANY KEY TO END TEST

DISK ROTATION TIME IN MILLISECONDS

D I S K - O - T I M E

197       198       199       200       201       202       203

|....:....|....:....|....:....!....:....|....:....|....:....|

!

DISKOTIM - 10

DISKTIME for DRIVE C:

Insert a Formatted diskette in C

Then strike any key.

STRIKE ANY KEY TO END TEST

DISK ROTATION TIME IN MILLISECONDS

D I S K - O - T I M E

197       198       199       200       201       202       203

|....:....|....:....|....:....!....:....|....:....|....:....|

!

=================================================================

DISKTIME for DRIVE D:

Insert a Formatted diskette in D

Then strike any key.

STRIKE ANY KEY TO END TEST

DISK ROTATION TIME IN MILLISECONDS

D I S K - O - T I M E

197       198       199       200       201       202       203

|....:....|....:....|....:....!....:....|....:....|....:....|

!

DISKOTIM - 11

7.   DISK DRIVE HEAD POSITION AND TRACK SELECTION

The  Drive  Execiser Function may be invoked from  the  main

menu by entering any of I,  O,  K,  L,  i, o, k, or l.  If "I" is

entered the following display is seen on your monitor:

STEPPER for DRIVE A:

Use I key to STEP IN one Track

Use O key to STEP OUT one Track

Use R key to RESTORE Drive if required after Disk Error

Use S key to toggle other SIDE of disk, same track position

Use Q/D Keys to Set QUAD/DUAL Density Drive Parameters

STRIKE ANY OTHER KEY TO END TEST AND RETURN TO MAIN MENU

Drive Operating on Side One at Track Number (HEX): 00

-----------------------------------------------------------------

As can been seen from the display,  you are presented with a

"mini"  Menu  that  is  a list of choices  that  will  cause  the

following  different operations or actions for Drive "A:" in this

case.   The "I" key "STEPS" the Drive Head Carriage in one  track

while the "O" key "STEPS" it out from the center hub.   The track

number  is  indicated in HEX at the display bottom.   Under  some

circumstances,  disk  errors may occur which can cause  the  head

carriage  to get "out of step" with where the computer controller

"thinks"  it is.   The "R" command will restore the Drive  in  an

attempt  to correct this situation.   The "S" command toggles the

"Side Select Bit" from side 1 to side 2 and if pushed again  will

toggle  it  back again.   This is useful when comparing the  head

response  and  alignment between the 2 sides of  a  double  sided

drive.  This is explained in more detail below.  The displays for

the other 3 drives are presented on the next two pages.

DISKOTIM - 12

STEPPER for DRIVE B:

Use K key to STEP IN one Track

Use L key to STEP OUT one Track

Use R key to RESTORE Drive if required after Disk Error

Use S key to toggle other SIDE of disk, same track position

Use Q/D Keys to Set QUAD/DUAL Density Drive Parameters

STRIKE ANY OTHER KEY TO END TEST AND RETURN TO MAIN MENU

Drive Operating on Side One at Track Number (HEX): 00

-----------------------------------------------------------------

STEPPER for DRIVE C:

Use i key to STEP IN one Track

Use o key to STEP OUT one Track

Use r key to RESTORE Drive if required after Disk Error

Use s key to toggle other SIDE of disk, same track position

Use q/d Keys to Set QUAD/DUAL Density Drive Parameters

STRIKE ANY OTHER KEY TO END TEST AND RETURN TO MAIN MENU

Drive Operating on Side One at Track Number (HEX): 00

DISKOTIM - 13

STEPPER for DRIVE D:

Use k key to STEP IN one Track

Use l key to STEP OUT one Track

Use r key to RESTORE Drive if required after Disk Error

Use s key to toggle other SIDE of disk, same track position

Use q/d Keys to Set QUAD/DUAL Density Drive Parameters

STRIKE ANY OTHER KEY TO END TEST AND RETURN TO MAIN MENU

Drive Operating on Side One at Track Number (HEX): 00  disk error!

-----------------------------------------------------------------

Notice  that in the display for drive D:  we show the  error

message  "disk error!" that may occur.   This indicates that  the

drive  head carriage position is not successfully being  verified

by  the drive controller.   When using the Special Alignment Disk

you may get this error message.  This is OK and should be ignored

since  those  skilled  in  the  art  of  Floppy  Drive  Alignment

procedures  can  verify  through experience  that  the  drive  is

operating on the proper track.  This is known by the pattern seen

using  an oscilloscope during an alignment procedure - more about

this later.

8.   SELECTION OF "QUAD DENSITY" AND BACK TO "DUAL DENSITY" DRIVE

PARAMETERS.

During  the "Stepper Function" the "Q" and "D" (also "q" and

"d")  keys  will  function to select the  QUAD  Density  or  Dual

Density  drive parameters required for stepping the drive to  the

proper track positions.  Use the "Q" and "q" keys to set the QUAD

Density   parameters.    You  may  return  to  the  DUAL  Density

parameters by using the "D" and "d" keys.  The "Stepper Function"

comes up in the DUAL Density mode which is the default mode.

DISKOTIM - 14

9.   USE OF TEST DISKETTES WITH DISKOTIM

You  may  use  any  test diskette with  "DISKOTIM"  for  the

"STEPPER"  Function.   You will however,  get the  "disk  error!"

display  for  most  test diskettes since the controller  will  be

unable  to verify that the correct track is operating.   You will

learn  to ignore this because the familiar test pattern  on  each

track will be recognized.   The instruction sheet packed with the

test  diskette  will tell you what the test patterns are on  each

track.   Test  diskettes are expensive (in the $50 range) and can

be obtained from several sources among them are:

The DYSAN CORPORATION

5201 Patric Henry Drive

Santa Clara, California  95050

(800) 538-8133

For  DUAL  Density  Drives with 48 tracks/inch  specify  P/N

224/2A  which  can  be used for single and  double  sided  Floppy

Drives.

For  QUAD  Density  Drives with 96 tracks/inch  specify  P/N

802020-206-30  which  can  be used for single  and  double  sided

Floppy Drives also.

For  the  "TIMER"  Function of "DISKOTIM" you  may  use  any

diskette  that  is formatted under the PC-DOS (MS-DOS)  operating

system.  The diskette may be write protected if desired since the

"TIMER" test is NON Destructive in that it doesn't ever write  on

the diskette to measure the floppy drive rotation time.

DISKOTIM - 15

10.   QUITTING FROM THE DISKOTIM PROGRAM BACK TO THE SYSTEM

You  may exit any test function by striking any  key.   This

returns  you to the main menu.   The "Q" command will then return

you  to the "SYSTEM" for normal operations.   It is advisable  to

"cold  start"  the  computer system after  using  the  "DISKOTIM"

program  since certain parameters in the computer may be  changed

for  the  convenience of the test program.   You can be  sure  of

correct  initial conditions only by a cold restart after  running

any test program - this is just S. O. P.

Finally,  this  program  should not be executed on a  multi-

user,  multi-tasking  machine such as the Columbia Data  Products

Multi-Personal Computer when other users are on the System  since

this may alter the other User's processes.

Other   messages   seen  occasionally  during  the  use   of

"DISKOTIM" are presented below:

(C) Copyright RAY HALLMAN 1983

***** QUITTING SEQUENCE BY OPERATOR *****

***** SEQUENCE ABORTED BY OPERATOR *****

... DONE!

***** ERROR *****   Try again!

***** NOT IMPLEMENTED! *****

The  above messages need no discussion and occur as a result

of operator action.

DISKOTIM - 16

11.  SOME COMMENTARY ON THE DRIVE SPEED ADJUSTMENT

The  drive  speed  adjustment is easy to  perform  for  most

Floppy Drives if you know what you are doing!  The main  mistakes

that can cause you the most trouble are:

1.   Getting the adjustment so far out of alignment that the

controller doesn't function properly.

2.   Adjusting the wrong adjustment!

You  must be careful to make small adjustments only.   Also,

make sure that you see some effect when you turn the pot 1/4 turn

- if  you don't see a change in the time display there is  either

something  really wrong with your drive or you are on  the  wrong

adjustment!   This  adjustment  is usually found near  the  drive

motor  power  wires.   Refer to the Maintenance manual  for  your

drives.

12.  SOME COMMENTARY ON THE DRIVE ALIGNMENT PROCEDURES

In  general,  you can't make Floppy Drive Alignments  unless

you are a qualified repair shop or at least know allot about  the

floppy  drives.   The Maintenance Manual for your drives  usually

gives  all  the details for this procedure using a  special  test

diskette such as the ones mentioned in section 9.

In  addition  to  the test diskette,  you will need  a  good

dual  trace oscilloscope with differential input capability  such

as the Tektronix 465.

The  kinds of checks that are usually made to Floppy  Drives

in  the  field  are:

C. E. Lobes              track 10

Index to Data            track 1 and 22

Azimuth                  track 22

I.S.V.                   track 5

Output Amplitude         track 27

Speed                      ---

Resolution               track 27

Tracks  shown  are  for the DUAL Density test  diskette  for

example  only - tracks and other instructions are available  with

the test diskette.

#       example  only - tracks and other instructions are available  with

the test diskette.

################################################################################################################################