Power Mountain Software Systems

P.O.Box 243

Provo, Utah 84603

Copyright 1987

all rights reserved

Filter 1.1 Documentation

By Larry Ashworth

2/09/87

** What's New! **

There have been some big changes in the program since version 1.0.  It  is

now  possible  to design 75 different filters instead of 7,  and the program is

now Shareware.  We hope you like it, and will support its' further development.

All rights to this program are  reserved,  and  no  modification  to  this

program  will  be  allowed,  whatsoever.  Feel  free  to write us if you feel a

modification to this program might be warranted. We will make any modifications

we  think  desirable.  We  intend  to  continue to develop it into a VERY major

program, and we would like your input.

You may distribute this program freely,  and in fact are encouraged to  do

so.  If  you are seeing this program for the very first time,  remember this is

not the latest version more than likely.  Registering will provide you with the

current  one,  and also get you the next update.  It is virtually impossible to

keep you informed as to all the most recent additions.  So if you  really  like

what you see, by registering you will get to see what's happening.

I'm not sure at this time how many versions will be released to the public

as shareware,  it will depend on the support of the public.  We'll release some

more things I'm sure,  but we don't know how well the  Shareware  concept  will

work  with  this  type  of  software,  so I don't want to promise more than I'm

prepared to deliver.

==============================================================================

!!  YOU MAY DISTRIBUTE ONLY THE SHAREWARE VERSION, NOT THE REGISTERED VERSION !!

==============================================================================

The purpose of this program is to:

A.)  Provide some basic background for those who �are investigating the

principles  of electronics,  and filters in particular,  �for the first

time,  or to provide a review for those who studied  it  sometime  ago.

Potential problems are covered, to make it easier to get what you want.

B.)  To aid those who need a good bandpass filter design for some audio

application, �and would like the computer to generate a schematic,  and

compute values.

C.)  To  help  those who need a filter,  to choose components that will

perform as desired,  or at�least to help make  a  sensible  compromise.

This   includes   guidelines  and  suggestions  as  to�the�operational�

amplifier�choices available.

Page 1

** BACKGROUND AND BASICS **

Since  we  have  to start somewhere,  and since we don't have room on this

disk for a complete course in electronics,  we are going  to  assume  that  you

already have some background in the basics.

To  begin  remember that all of electronics begins with the study of a few

basic components, known as resistors, capacitors,  and inductors.  �These basic

components  �can  be used in an infinite variety of circuits and are the �basic

building blocks of all electronics.  Combined with semiconductor technology and

the invention of the vacuum tube, �virtually all of the developments �of modern

technology have been �created.  ��These fundamental elements form the  function

blocks upon which we have built everything.

There are many different function blocks that are often used in�the design

of communications and audio circuits.  One of these is the filter.  As a matter

of fact virtually �all �electronic �equipment �uses some �sort �of filtering. A

filter �is �a �circuit that electronically determines which frequency, or range

of frequencies, will be allowed through.

There �are essentially two classes of filters known as R.F.,  �and  Audio.

R.F.  �filters  �are  so called because these are used �for �Radio Frequencies,

consisting of high �frequency ranges used in communications and in  navigation.

These  are  much higher than the range of the human ear.  Audio frequencies are

those which are within the range of human hearing.  In this case from 10 hz  to

25 khz or so.

For  �our purposes we will discuss audio filters,  �but the principles are

the same for R.F. types.

All filters basically consist of four types,  they are LOWPASS,  BANDPASS,

NOTCH,  �BAND REJECT,  and HIGHPASS.  �I �say four types because notch and band

reject �are both the same thing, �differing only in the width,  �or range,  �of

frequencies that are rejected.

A  wide bandpass filter design can be formed from the use of a highpass in

front of a lowpass filter,  in fact  this  is  the  way  a  speech  filter  for

communications  is  designed.   In  the  case  of  speech,  only  the  range of

frequencies extending from 300 hz to 3000 hz are required to understanding what

is said.  By concentrating all the power of a transmitter over a narrower range

of frequencies better communicating under adverse conditions  is  possible.  In

this  situation  we  would  use  a  high pass filter of 300 hz which would pass

frequencies above 300 hz to the input of a low pass filter  at  3000  hz,  thus

forming a 300 - 3000 hz bandpass filter system.

A  bandpass  filter  can  be very narrow,  and there are many applications

where only a very small band of frequencies are desired.  As  in  the  case  of

morse code communications in amateur radio. The Butterworth bandpass filter has

the property of being able to pass a small band of frequencies with nearly  the

same  amplitude throughout the pass band,  while allowing very steep skirts for

maximum attenuation  of  unwanted  signals  or  noise.  (The  number  of  poles

determines just how steep these skirts are.)

Notch,  or  band  reject  filters  are  used  to  eliminate or attenuate a

frequency or band of frequencies.  This particular version ("Filter 1.1")  does

not  create  notch filters,  though we intend to add them to the program in the

future. Band reject filters can be created through the use of high and low pass

filters,  simply by selecting the appropriate filters to reject the frequencies

you consider undesireable.

Page 2

** COMPONENT CHOICES **

This program is designed to allow you to very quickly design and  document

a  filter design.  A great deal of effort and research has gone into creating a

program that will make your life easier.  However,  there are hidden  'gotchas'

just waiting in the wings to ruin an otherwise excellent design.

Before we get into a few of these here are  a  few  pointers:

A.  The  resistor  and component values given are calculated values and

need to be rounded  out  to  the  nearest  standard  value.  The  tolerance  of

resistors  and  capacitors  is suggested in the program,  and as you use it you

will see that exotic filter  shapes  require  very  tight  tolerances.  Simpler

filter requirements yeild less stringent component accuracies. In general, when

choosing components remember,  the closer you can come to the calculated values

the more likely you are to get the predicted results.

One  of  the really nice features of this program,  is the inherent

ability to experiment with values,  until the right combination is  found  that

meets  your  requirements.  Capacitor values should be kept below 1 uf.,  while

resistor values should be kept at around 2k and above.  Juggle capacitor values

around until you come up with the results that meet your requirements. Remember

also that resistance values above a Megohm may not be a good idea, try a larger

capacitor.  Some  very  low  frequency  filters require a large capacitor,  but

they're expensive.

B.  There are a great many good op-amps to choose from, and if you feel

a  little insecure about picking one for the job you're doing,  I would like to

suggest the TL071 (single),  TL072 (dual),  or TL074 (quad) packages.  They are

high performance, low noise, high slew rate devices that work very very well in

this type of circuit.  Their pin outs are industry standard,  and the price  is

reasonable.  If  you  need extremely low power consumption the TL061 series has

the same pin out and can be substituted.  These devices can be found in a Texas

Instruments Linear Data Manual.  The TL061,  '62, and '64 devices are not quite

as good in critical applications,  however,  due to the sacrifice in slew  rate

for lower power consumption.

C.  If you have noticed there are  potentiometers  in  the  Butterworth

filter  schematics,  these can be 1/4 watt pc-mount types,  and the reason they

are there is to make it possible to make the fine tuning adjustments  necessary

to  get  the  system  to  work.  The  tuning procedure requires that you have a

frequency generator, and a frequency counter.  Using the values given, all that

you  need  to  do  is  to  make  sure that each section is tuned to the correct

frequency,  and then to make sure with your ac voltmeter that the gain for that

section is correct.  Start with the first section,  and continue through to the

last.

Page 3

** LOW & HIGH PASS FILTER TYPES **

When  you  run  the low pass and high pass filter design options,  you are

asked to choose a slope option.  In case you aren't  sure  what  we're  talking

about,  I'll define them for you.  This new feature we have put in the program,

greatly enhances it's power and flexibility. You can now tailor each section to

give you the exact slope you need for your application.

These slope options are created by varying the DAMPING, of the section. As

the  damping  is  reduced  from  2 toward 0,  the initial rolloff of the filter

becomes steeper.  This allows higher rejection of unwanted signals  than  would

otherwise  be  the  case,  but the thing that I want to bring out is trade offs

involved.

Changing the damping has an affect on the passband of the filter, reducing

the  damping  causes ripples to occur.  You will notice in fact,  slope options

called 1db,  2db,  and 3db dip.  This refers to the passband ripple inherent to

these  filters.  Flattest  amp  refers  to  the  fact that the amplitude in the

passband   is   flat.    Very   desirable   in   audio   applications  such  as

bi-amplification.

'Highly damped' and 'best delay' are the  slowest  to  rolloff  initially,

'compromise' a little faster, and so on....

** POTENTIAL PROBLEMS **

This program works very well, but the results can be unrealistic.  This is

due  to  the  fact  that  no  matter how rediculous your specifications are the

program will try to come up with values.  Values that can be  impossible.  In a

case such as this a little common sense is in order.

When you design a Butterworth bandpass filter,  keep the bandwidth as wide

as you can to get what you need accomplished.  Excessively narrow  filters  are

extremely difficult to tune,  the requirements for the op-amp get unreasonable,

and very often even if you get it running the ringing  of  the  filter  is  bad

enough to cause more troubles than you gain.  There are some important physical

limits that there just is no way around.

A one hertz wide filter will take  1  second  to  reach  approximately  90

percent of the maximum amplitude.  The higher the Q,  the closer to oscillation

you are,  and the greater the problems.  Trust me.  Keep it as wide as you can.

You'll be happier.  Use more poles if you need it really narrow,  but don't cut

those sidebands,  'cause you need 'em.  There  have  been  a  great  many  good

articles on filters in the various hobby electronic magazines, and of course in

many good textbooks on communications,  which can give you more information  on

why narrower isn't always better.

We've  tried  to  trap  out unreasonable inputs,  but I suppose there is a

chance we missed something.....

R E M E M B E R !

The ultimate rolloff of a filter is governed by the number of poles,  only

the initial rolloff is changed.

More poles, as a general rule, is more desirable than trying to make a two

pole filter act like a six pole.

Components become more critical as you lower the damping,  and it  gets  a

litle tricky to build a practical filter.  Use discretion.

On a final note,  remember that if you want to print  out  the  schematics

generated  by  the  program,  you must run the GRAPHICS program from DOS or the

equivalent or you will not get a proper print-out of the screen.

Page 4

** THANK-YOU **

I  would  like to take this opportunity to thank Dale Spanheimer,  for his

help in putting this program together.  A lot of  effort  has  gone  into  this

thing,  and  I really appreciate his help.  The updates are coming a lot faster

due to his assistance.

Thanks are also in order to Thayne Harmon, who adapted the source code for

use  on  the  Atari ST.  He has made,  and helped to implement,  some very nice

changes in the program.

Now  let  me  say  thank-you  to  YOU.  It's people like you that make the

Share-ware software concept work.  We have faith in you, and we want to provide

a program that is professional, and as high a quality as possible.  I feel that

this way our future will be assured,  and that people  such  as  yourself  will

support this kind of effort.  Please don't let us down,  and we will work to be

of service to you.

Larry Ashworth

Power Mountain Software Systems

P.O.Box 243

Provo, Utah 84601

** FINAL NOTES **

03/11/87

At present,  the program has been tested  and  found  to  run  on  an  IBM

compatible  with  an  EGA/CGA card and a Tandy 1000.  A patch has been added to

make the program look better on the EGA card without blanking the screen on the

CGA  machines.  After the commercials in the beginning of the program,  you are

asked if you have an EGA card.  If you  respond  with  "N",  the  program  will

function  normally for CGA.  If you input "N" and you do have an EGA card,  you

will get green graphics on a white background.  If you respond "Y" and you have

a  CGA  card,  you  will get a blank screen after you have input all the values

requested.  If this happens,  just hit the retun key a couple of times and  you

will be asked if you want to design another filter. Respond "N" and you can run

the program again and respond "N" when it asks if you have an EGA card. We have

had a couple of copies of the program escape without this patch, so if you find

your screen going blank on a CGA card after you have  entered  all  the  values

requested,  then  you will find that problem is now cured.  The revision number

has not been changed to reflect that patch  being  added  to  the  program.  We

appologize for any inconvenience that may have been caused by this bug.

Page 5