TITLE ENHANCED.ASM (24 March 1988 - Rufus S. Hendon)

;=============================================================================

; ENHANCED.ASM is adapted from the procedure called NewInt16 in ENHKBD.ASM

; Version 1.2 (2/23/88), written by Tim Kokkonen (with help from Brian Foley).

; ENHKBD is part of a Turbo Pascal 4.0 unit that makes it possible for Turbo

; Pascal programs to turn the enhanced keyboard on for the duration of the

; program, restoring the standard keyboard when the program terminates.

; ENHANCED, on the other hand, is independent of Turbo Pascal, and the .COM

; file produced from it is meant to be invoked from AUTOEXEC.BAT in order to

; make a permanent switch to the enhanced keyboard.  Like ENHKBD.ASM,

; ENHANCED.ASM is released to the public domain.

;=============================================================================

; This is a terminate-and-stay-resident replacement for the handler for type

; 16H interrupts.  It activates the enhanced keyboard as implemented by the

; Phoenix 80286 ROM BIOS Version 3.02 used in the AST Premium/286 computer.

;

; Among other reasons for activating the enhanced keyboard is the fact that

; the enhanced keyboard processes ALT-224 correctly, generating the code for

; the ordinary character alpha (code 224 = E0H).  The standard keyboard as

; implemented by the Phoenix BIOS incorrectly generates a null character

; (character code and scan code both = 00H) when ALT-224 is typed.

;

; The new handler for type 16H interrupts does two things:  (1) It changes

; functions 00H and 01H (in AH) to 10H and 11H respectively, which are the

; enhanced-keyboard functions.  It then calls the original handler.  (2) Upon

; return from the original handler, a check is made for cases in which an

; extended code (in AH) is paired with "character" code E0H (in AL) instead

; of 00H.  Such codes are generated for the keys that are specific to the

; enhanced keyboard, such as the cursor-movement keys that are separate from

; the numeric keypad.  To prevent these from being interpreted by application

; programs as the ordinary character with the code E0H (224), the E0H is

; replaced by 00H before return is made to the interrupter.

;=============================================================================

; The following table summarizes the keys which are affected by using this

; unit. The entries are the values of the scan word in hex.  The high byte of

; the scan word is the scan code, the low byte the ASCII character.  See

; Notes, following the table, for the key to special symbols.  [Kokkonen's

; table has been modified to reflect the operation of NewInt16 with the

; enhanced-keyboard implementation in the Phoenix 80286 ROM BIOS Version 3.02,

; without the replacement for interrupt 9H included in his ENHKBD.ASM.]

;

;               Plain     Shift     Control   Alt

;               -----     -----     -------   ----

; F11           8500%     8700%     8900%     8B00%

; F12           8600%     8800%     8A00%     8C00%

; Esc           011B      011B      011B      0100*

; Backquote     2960      297E       -        2900*

; Backspace     0E08      0E08      0E7F      0E00*

; Tab           0F09      0F00      9400*     A500*

; Left Brack    1A5B      1A7B      1A1B      1A00*

; Right Brack   1B5D      1B7D      1B1D      1B00*

; Backslash     2B5C      2B7C      2B1C      2B00*

; Semicolon     273B      273A       -        2700*

; Quote         2827      2822       -        2800*

; Enter         1C0D      1C0D      1C0A      1C00*

; Comma         332C      333C       -        3300*

; Period        342E      343E       -        3400*

; Slash         352F      353F       -        3500*

;-------------------------------------------------

; Numeric keypad keys:

; Insert        5200      5230      9200*      -

; Del           5300      532E      9300*      -

; Home          4700      4737      7700       -

; End           4F00      4F31      7500       -

; PgUp          4900      4939      8400       -

; PgDn          5100      5133      7600       -

; Up            4800      4838      8D00*      -

; Down          5000      5032      9100*      -

; Left          4B00      4B34      7300       -

; Right         4D00      4D36      7400       -

; Pad-Asterisk  372A       !         !        3700

; Pad-Minus     4A2D      4A2D      8E00*     4A00

; Pad-Plus      4E2B      4E2B      9000*     4E00

; Pad-5         4C00*     4C35      8F00*      -

;-------------------------------------------------

; Separate cursor-movement keys:

; Insert        5200      5200      9200*     A200*

; Del           5300      5300      9300*     A300*

; Home          4700      4700      7700      9700*

; End           4F00      4F00      7500      9F00*

; PgUp          4900      4900      8400      9900*

; PgDn          5100      5100      7600      A100*

; Up            4800      4800      8D00*     9800*

; Down          5000      5000      9100*     A000*

; Left          4B00      4B00      7300      9B00*

; Right         4D00      4D00      7400      9D00*

;

; Notes:

; - These keystrokes are ignored.

; * These keystrokes are not normally returned by the non-enhanced keyboard.

; ! These keystrokes control printscreen and print echoing. They cannot

;   simulate the effect of the enhanced keyboard, which has the Asterisk and

;   PrtSc keys separated.

; % The F11 and F12 keys are returned only if they actually exist on the

;   keyboard.

;

; With a few exceptions, NewInt16 returns results identical to those of the

; enhanced keyboard BIOS. One difference makes the enhanced keyboard simpler

; to use in a program: those scan words which normally contain E0 in the low

; byte to indicate that the key is specific to the enhanced keyboard (like the

; dedicated cursor keys) will have the low cleared to zero by NewInt16.