WESTERN DIGITAL

EtherCard PLUS LAN ADAPTER BOARD DRIVER

FOR USE WITH SUN MICROSYSTEMS' PC-NFS

INSTALLATION GUIDE        March 8, 1988

CONTENTS

The following files are needed for installing PC-NFS (*):

1. WD8003E.SYS - PC-NFS Link Level Driver for EtherCard PLUS.

2. MYEADDR.EXE - Program to determine the Ethernet address of the

board being used.

HARDWARE AND SOFTWARE REQUIREMENTS

1. One Western Digital EtherCard PLUS (tm) LAN Adapter.

2. An IBM PC (*), PC XT (*), PC AT (*) or 100% compatible personal

computer.

3. At least 512 Kbytes of usable RAM.

4. PC-DOS (*) 3.0 or greater, or a compatible version based on

MS-DOS (*).

5. One or more NFS (*) servers.

For additional information, refer to the PC-NFS User Manual, available

from Sun Microsystems.

CONFIGURING AND INSTALLING THE EtherCard PLUS LAN ADAPTER

1.  Jumper the adapter board according to your system configuration.

The WD8003E.SYS Device Driver is installed at PC boot time according to

the command line specified in the DOS CONFIG.SYS file (described below).

The user must verify that the jumper settings on the LAN adapter

board correspond to the software settings given on the device driver

command line.

Jumper settings for some adapter boards are described in the

JUMPERS.DOC file.  For more information, refer to the Western Digital

"User Installation Guide" that comes with each adapter board.

2.  Insert the EtherCard PLUS adapter in your PC.

3.  Power up your PC.

DETERMINE ETHERNET ADDRESS

You will need to know the Ethernet address of the adapter board during

the PC-NFS installation procedure (refer to PC-NFS documentation,

INSTALL program.) To determine your LAN adapter Ethernet address,

run the MYEADDR.EXE program as follows:

MYEADDR WD8003E xxx

where xxx is the I/O BASE ADDRESS in hexadecimal of the adapter board

as jumpered above. xxx can be omitted if the factory default setting of

280 is used.

DRIVER INSTALLATION PROCEDURE

For PC-NFS Version 3.0 and above, the EtherCard PLUS Driver can be

configured and installed via the PC-NFS NFSCONF Utility.  For PC-NFS

Version 2.0, the WD8003E.SYS Driver must be included in the DOS

CONFIG.SYS file, as described below.

To install the EtherCard PLUS driver, follow the procedures outlined

below:

1.  Jumper the adapter board according to your system configuration,

as described above.

2.  Include the following statement in your CONFIG.SYS file:

DEVICE=\NFS\WD8003E.SYS /i# /pxxx /myyyy

where

#     is the PC interrupt value selected by the jumpers on

the adapter,

xxx   is the base I/O port address in hexadecimal selected

by the jumpers on the adapter, and

yyyy  is the 4-digit memory segment base address in hex

that you want the driver to use for the 8K on-board

shared buffer memory.

For example, the following statement corresponds to the factory jumper

configuration of the adapter and a buffer memory base address of D0000:

DEVICE=\NFS\WD8003E.SYS /i3 /p280 /mD000

Make sure the software configured parameters match the adapter jumper

settings.  (Note that the PC-NFS driver defaults to IRQ2 while the board

is shipped with the jumper set to IRQ3; therefore, the /i parameter

must be specified or else the jumper must be changed.)

Your # options for the /i Interrupt parameter are: 2, 3, 4, 5, 6, or 7.

Your xxx options for the /p I/O port parameter are: 3E0, 3C0, 3A0, 380,

360, 340, 320, 300, 2E0, 2C0, 2A0, 280, 260, 240, 220, or 200.

Your yyyy options for the /m Shared Buffer parameter are: 8000 - F000,

on 8K-byte boundaries (increments of 200).  (The PC-NFS driver defaults

to D000; there is nothing to jumper on the adapter board.)

3. If your CONFIG.SYS file does not explicitly specify at least 20

buffers and files, your should modify CONFIG.SYS to include the

following:

BUFFERS=20

FILES=20

4. You must have a copy of Sun's PC-NFS (rev. 2.0 or greater).  Refer

to that documentation for additional information on PC-NFS.

If you need help, contact Western Digital at (714) 474-2033 ext. 4900,

or Sun Microsystems at (617) 671-0378.

(*) PC-NFS and NFS are trademarks of Sun Microsystems, Inc.

PC XT, PC AT, and PC-DOS are trademarks and IBM is a registered

trademark of International Business Machines Corporation.

MS-DOS is a registered trademark of Microsoft Corporation.

EtherCard PLUS is a trademark of Western Digital Corporation.