XFLASH Version 1.0

by David Hite

March 17, 1987

Memory-resident FreeWare.  960 bytes required.

Usage: xflash <CR>            (for the 8088 chip)

or

xflash-2 <CR>          (for the 8088-2 chip)

XFLASH eliminates the flash that occurs during scrolling on the

CGA, without introducing snow.  It does so by intercepting the

bios scroll up and scroll down functions, and performing these

functions itself.

It works by moving characters only during horizontal and

vertical retrace periods.  It moves as many characters as

possible during these periods, and its operation is therefore

dependent on the speed of your processor.  XFLASH is optimized

for the 8088 chip.  XFLASH-2 is optimized for the 8088-2 chip.

XFLASH.ASM and XFLASH-2.ASM are slightly different, in that the

8088-2 can move a word during a horizontal retrace, whereas the

8088 can move only a byte.

Both ASM versions contain instructions for changing the number of

characters moved during the vertical retrace period.  You may

experiment with these values in order to eliminate snow or to

speed up the scroll.

The ASM programs also contain some code to change the border

color to the same color as the fill attribute specified in the

scroll.  This code is commented out in the distributed version.

If you want to set the border to the fill attribute, un-comment

this code and re-assemble.

Please report any bugs by leaving a message to me on the Computer

Connections Bulletin Board, Washington, DC.  (202) 547-2008.