ABOUT MEM.EXE v 3.0

CHANGES: Mem now evaluates whether or not you are operating in a windowed

environment, using the fastest possible video access.  DOS redirection

is not allowed.  Scrolling is improved.

MEM is a general memory analysis program.  It tells you your DOS speed (not

processor speed), breaks down your memory arenas, tells you the size and

amount available of all disk drives, expanded memory, and system memory.

Syntax:

MEM [d:]

^ last valid drive in your system

LAST DRIVE

MEM looks at all the drives (from A: to Z:).  Sometimes, this is not

desirable.  For that reason, you may specify the last valid drive on the

command line.  Also, you may enter "SET LastDrive=d:" into DOS for the

same purpose.

DISPLAY

Mem splits the screen into three section.  The three sections are DOS

speed, Drive/Memory size, Memory arenas.

DOS SPEED:  This operation finds out how many times DOS can calculate the

current time in one second.  This is a pretty good test to see how much

processor time is stollen by resident programs and routines.

DRIVE/MEMORY SIZE:  Each drive is accessed and displayed for disk size and

amount still available.  If the drive is not active, then it is simply

skipped.  Then expanded memory is surveyed.  WARNING::: Be sure that

CMOS know whether you have expanded memory (AT machines only).  Then the

amount of system memory available to COMMAND.COM is displayed.  The

final piece (amount of system memory is available) is calculated and

displayed.

MEMORY ARENA:  COMMAND.COM splits your memory up into blocks (arenas).

Each arena is owned by a program that you have executed.  MEM finds this

arena and tracks through the arenas.  You find out the hex paragraph

number on which the arena is located and it's size.  If a multitasking

environment is in place, then it will also track most of that

environment too (although it isn't very useful).

==================

GEnie:       D.Boteler

Compuserve:  74666,701

Postal:      26785 Ironwood   Moreno Valley   CA   92360

==================###############