PORT TEST

Comprehensive I/O Port Diagnostics for PCs

USER'S MANUAL

Version 1.1

MICROSYSTEMS DEVELOPMENT

4100 Moorpark Ave. #104

San Jose, CA  95117   USA

Phone:  (408)296-4000

FAX:  (408)296-5877

Copyright (C) MicroSystems Development   1990

All rights reserved

DISCLAIMER

MicroSystems Development makes no warranty of any kind, express

or implied, including without limitation, any warranties of

merchantability and/or fitness for any specific application or

use.  MicroSystems Development shall not be liable for ANY loss

or damage arising from a failure of this program to operate in

the manner described, or in a manner desired by the user.

MicroSystems Development shall not be liable for any damage to

data or property which may be caused directly or indirectly by

use of the program.

IN NO EVENT SHALL MICROSYSTEMS DEVELOPMENT OR ITS SUPPLIERS BE

LIABLE FOR ANY CONSEQUENTIAL, INCIDENTAL OR EXEMPLARY DAMAGES

INCLUDING, BUT NOT LIMITED TO LOSS OF PROFITS OR REVENUES, LOSS

OF USE OF ANY PRODUCT, SERVICE OR REPLACEMENT COSTS, OR CLAIMS BY

ANY THIRD PARTY.

COPYRIGHT AND TRADEMARK INFORMATION

The software code and screen displays used in PORT TEST are the

sole property of MicroSystems Development, and may not be copied,

in any form, in whole or in part, or included in any other

program or document without the express written permission of

MicroSystems Development.

IBM, IBM PC, IBM XT and IBM AT are registered trademarks of

International Business Machines Corporation.

MS-DOS is a trademark of Microsoft Corporation.

PORT TEST is a trademark of MicroSystems Development.

MicroSystems Development is a division of

MicroSystems Development Technologies, Inc.

TABLE OF CONTENTS

1.0  BACKGROUND INFORMATION................................1

1.1  Serial Port Basics...............................1

1.2  Parallel Port Basics.............................1

2.0  ADDRESSING SCHEMES....................................3

2.1  Port Addressing..................................3

2.2  What the BIOS does...............................3

2.3  What DOS does....................................4

2.4  Logical vs. Physical Addresses...................4

2.5  Interrupt Usage..................................5

3.0  SERIAL AND PARALLEL TESTS.............................6

4.0  COMMAND LINE PARAMETERS...............................7

CUSTOMER FEEDBACK FORM.....................................8

In addition to tests and diagnostics, PORT TEST can do

the following:

porttest swap lpt1 lpt2

Swaps table addresses for lpt1 and lpt2.

porttest com3:19200,n,8,1

Set baud rate, parity, data bits, and stop bits for a serial

port.

porttest as 280

Adds address 0280 hex to the serial port table as the next

logical com port.

porttest ds

Automatically detects serial ports and updates the table.

And more...

1.0  INTRODUCTION

When computer information, known as data, is sent from one

device to another, it is carried by a wire cable.  Data is

generally sent in one of two basic ways.  Many bits at a time

over many wires in the cable is known as parallel transfer.

Sending one bit at a time over one wire is known as serial

transfer.  Each type of transfer has its own advantages and

disadvantages.  The serial or parallel connections to the

computer are called PORTS.  Collectively, they are called

Input/Output, or I/O ports.

PORT TEST is comprehensive diagnostic program for I/O ports

installed in IBM PC and AT compatibles computers running DOS.  It

is useful to identify ports, manipulate system tables, test the

ports, and to identify and correct problems.  This manual is

intended to supplement the information provided in PORT TEST by

explaining the ways in which I/O ports are commonly used and

implemented in IBM compatible PCs.  It also describes what each

test does and provides information on what to look for if any of

the tests fail.

1.1  SERIAL PORT BASICS

The PC serial port uses voltages and connectors established

by the Electronic Industries Association (EIA) standard

specification known as RS-232C.  In addition to one bit of data,

other control signals are also sent and received.  Sending data

one bit at a time is relatively slow, but serial ports have the

advantage of using fewer wires to connect two devices and being

able to communicate over relatively long cable distances.  Most

serial ports can easily communicate over 100 ft of cable.  Data

is sent using +12 volts and -12 volts to represent a 0 or a 1

data bit.  In addition, there are a wide variety of serial

peripherals available which make use of the standard RS-232C

specification.  Furthermore, recent advances have made serial

data transmission reliable at higher data rates.  These data

rates are referred to as "baud" rates.

The heart of all serial ports is an Asychronous

Communications Element, referred to as an ACE.  The CPU reads and

writes a byte (8 bits) of data to this chip, and the ACE handles

the serial transmissions and other functions related to the RS-

232C specification.  The ACE used in most PC & XT type (8088 and

8086) systems is the INS8250B or equivalent.  Most AT class

machines (80286 & above) use the NS16450 device.  The 16450 chip

is designed to handle the higher data bus rates of the faster

computers.  The serial data communications however is identical

to the 8250.  It is beyond the scope of this manual to describe

the detailed workings of these chips.  Refer to the

manufacturer's data sheets for more detailed information.

1.2  PARALLEL PORT BASICS

Eight bits of data are transferred at a time over PC

parallel ports.  Data is sent using +5 volts and 0 volts to

1                                       PORT TEST User's Manual

represent a 1 or a 0 data bit.  As you might expect, data can be

transferred relatively fast in this manner.  The disadvantages of

this method is that more wires are needed and that most common

parallel ports can only work reliably at distances of under 20

feet.

Unlike serial ports which rely on a chip to do the data

transmission, parallel data is handled entirely with software.

Parallel ports have three registers, one for data out, one for

output control lines, and one for input control lines.

The way is which data is transmitted over parallel ports

varies, but a general description follows.  To send a byte of

data, the software outputs the byte to the data lines, then

pulses the STROBE output.  The device on the other end (normally

a printer) then asserts its BUSY line.  The PC waits for BUSY to

go away before sending the next byte.  In other implementations,

the ACKNOWLEDGE line is used for the printer to signal to the PC

that it has received the data.

The other control lines are used for various purposes.  On

the printer side, PAPER-END is a signal to the PC that the

printer is out of paper.  The printer also sends a SELECTED

signal to the PC indicating that it is 'on line' and ready to

receive data.  The ERROR signal from the printer can be used for

any type of error that would cause the printer to be unable to

receive data.  In general, if any of these signals are asserted,

BUSY is also asserted.

The PC has three additional output control lines.  Usage of

these lines varies with the particular software and printer in

use.  In the original PC implementation, they are used as

follows:  AUTOFEED tells the printer to automatically insert a

line feed after each carriage return.  INIT initializes the

printer upon power up.  SELECT deselects the printer and takes it

'off line'.  SELECT was most often used as a method to signal the

operator that some attention was required.  For example, when the

software needed to change to a different font, it would deselect

the printer.  This would cause the printer to turn off its

SELECTED line and its front panel light.  The user would then

change the print wheel in the printer and press the SELECT button

on the printer.  The printer would then turn on its SELECTED line

signalling the software to proceed.  Many printers will stay in

the 'off line' condition for as long as SELECT or INIT are

asserted even if the SELECT button is pressed.

As stated earlier, not all of these controls lines are used

in the same manner.  Some of the latest laser printers on the

market ignore all the control lines to it except for the STROBE

line.

2                                       PORT TEST User's Manual

2.0  ADDRESSING SCHEMES

2.1 PORT ADDRESSING

Any I/O device in a PC must be located at a particular

address.  Actually, a serial port consists of eight addresses and

a parallel port has three.  The address used to refer to a

particular port is the first or BASE address.  For both serial

and parallel ports, this first address is where the data to be

transferred is read and written.

These addresses are the PHYSICAL addresses of the ports.  It

is where the Central Processing Unit (CPU) of the PC must read or

write data to send or receive data over a port.  There are no set

rules for what addresses must be used for ports, but some

commonly used addresses are supported by most manufacturers.

Generally, the first two serial ports are at physical addresses

3F8 and 2F8.  These are referred to as COM1 and COM2, the LOGICAL

addresses.   Generally, parallel ports are located at addresses

3BC, 378 and 278, which are referred to as LPT1, LPT2, and LPT3.

Many monitor cards have a parallel port built in.  In these

cases, the port is usually at 3BC.

NOTE:  The physical addresses are shown in hexadecimal notation.

2.2  WHAT THE BIOS DOES

Most PC BIOS routines do a quick check upon power up to

determine how many serial and parallel ports are installed in a

system.  They examine specific addresses by writing to them, then

reading them back to determine if a device is present at that

address.  The BIOS then places the address of each responding

device in a table in RAM located at 40:0 for serial ports and at

40:8 for parallel ports.

There are eight bytes at each of the above locations

reserved for port addresses.  Since a port address is two bytes,

up to four serial and four parallel addresses can be put in the

tables.

For example, for a system with two serial ports and two

parallel ports, typical tables might be:

Serial Port table:       Parallel port table:

ADDRESS    DATA          ADDRESS    DATA

40:0      F8 03          40:8      BC 03

40:2      F8 02          40:A      78 03

40:4      00 00          40:C      00 00

40:6      00 00          40:E      00 00

Note:  The high & low bytes of the address are reversed in the

tables.

3                                       PORT TEST User's Manual

2.3  WHAT DOS DOES

After a PC has booted and the BIOS is no longer in control,

DOS uses the addresses in these tables as COM1, COM2, etc. for

serial ports, and LPT1, LPT2, etc. for parallel ports.  For

example, DOS includes a program called MODE to set various

parameters such as the baud rate, of a port.  In order to use the

MODE command, the user must refer to the desired port by its COM

or LPT assignment.  The physical address used is the one stored

in the table.  It is very possible that COM1 in one system is not

at the same physical address as COM1 in another system.

In order for DOS to recognize a COM or LPT number, there

must be a valid address listed in the table, and there can be no

gaps.  If there is a zero entry in the table, DOS considers that

the end of the list.  Therefore, as far as DOS is concerned, it

is impossible to have a COM1 & a COM4 without having a COM2 &

COM3 also!  Other software may report differently as explained

below.

2.4  LOGICAL VS. PHYSICAL ADDRESSES

Many BIOS routines only check for two serial ports and three

parallel ports.  As PCs became more popular, the need for more

ports grew.  This fact was 'fixed' by DOS Versions 3.3 and

higher, which will recognize up to four COM and four LPT ports.

However, DOS will only recognize a logical port whose physical

address has been placed in the RAM table.  If the BIOS doesn't

put the physical address there upon power up, DOS won't recognize

a logical port even when the hardware is properly configured and

installed!

If you need to use DOS to support more ports than your BIOS

will detect and install, PORT TEST can be used to place the

correct address in the system tables.  You can use the

interactive mode, but PORT TEST's batch mode was specifically

designed for this purpose.

Place the command:  porttest /ds /dp  in your autoexec.bat

file to automatically detect serial and detect parallel ports in

your system upon power up.  These ports will now be recognized by

DOS.  See "Command Line Parameters" for more information.

NOTE:      Many application programs do not use DOS to interface

to ports, so that it may not be necessary to include a port's

address in the system tables.  However, you must place the

address in the table in order to test a port with PORT TEST.

Considerable confusion exists regarding proper logical and

physical port address.  Strictly speaking, there is no proper

physical address for any of the logical ports.  The logical port

structure permits any physical address to be placed into its

tables.

4                                       PORT TEST User's Manual

Many application programs access ports directly, and do not

use the built in routines in the hardware's BIOS.  This is

usually done because the application needs a more sophisticated

device driver than is provided by the BIOS.  When this is the

case, the logical port addresses are not always used properly.

Some applications access ports directly, but properly determine

the physical address from the logical address by using the table

at 40:0.  Other programs do not use logical addresses at all, and

just refer to physical addresses.  Still others convert logical

to physical addresses using their own table.  In this case, the

program could report that there is a COM1 and a COM4 installed

with no COM2 or COM3!

2.5  INTERRUPT USAGE

In addition to a serial port's registers, the ACE also has

the ability to interrupt the processor, either after it has

transmitted a byte of data, or after it has received one.

Ideally, the interrupt request line that each port uses should be

unique, but there are not always enough interrupts to go around.

Therefore, interrupts are sometimes used by more than one device.

This is ok as long as only one of the devices is active at a

time.

A standard PC has eight hardware interrupt request lines.

Often, IRQ4 is used for the serial port at address 3F8 and IRQ3

is used for the port at address 2F8.  This is what PORT TEST

assumes unless told otherwise.  For other serial ports, the user

must assign the proper IRQ number (if any) in PORT TEST.  If left

unassigned, PORT TEST will skip the interrupt tests.

Most I/O cards have jumpers to select which interrupt will

be used.  If there are conflicts, the user should examine these

jumpers to determine the cause of conflict.

Some parallel ports can be configured to use IRQ7.  When

enabled, the processor will receive an interrupt when

/ACKNOWLEDGE (pin 10) transitions from high to low.  This is not

supported on all cards and few applications use this feature.

PORT TEST does not test parallel port interrupts.

5                                       PORT TEST User's Manual

3.0  SERIAL AND PARALLEL TESTS

Detailed information on the content of the tests is in the

manual which comes with the registered version of PORT TEST.

6

PORT TEST User's Manual

4.0  COMMAND LINE PARAMETERS

In addition to its interactive mode, PORT TEST supports

several command line parameters as follows:

Usage:    porttest [mode switches] [batch commands]

All command line parameters are optional.

mode switches:

/p  = Use the system's default print screen handler

Normally PORT TEST uses its own print screen handler so that

linedraw and other non printable characters are replaced

before printing.  If your printer has the capability to

print these characters, then use this command line option to

turn off PORT TEST's character translation when doing a

Print Screen.

/m  = force Monochrome on color monitors

/i  force interactive mode

(default when no batch args)

batch commands:

detectserial        OR ds        ->  detect serial ports

detectparallel      OR dp        ->  detect parallel ports

addserial <hex>     OR as <hex>  ->  add serial port

addparallel <hex>   OR ap <hex>  ->  add parallel port

com#:<b>,<p>,<d>,<s>             ->  set serial parameters

addserial <hex>:<b>,<p>,<d>,<s>  ->  add and set parameters

delete com# OR delete lpt#       ->  delete port

swap com# com# OR swap lpt# lpt# ->  swap ports

Examples of command line usage:

porttest ds

Automatically detects serial ports and updates the table.

porttest as 280

Adds address 0280 hex to the serial port table as the next

logical com port.

porttest com3:9600,n,8,1

Set baud rate, parity, data bits, and stop bits for a serial

port.

porttest swap lpt1 lpt2

Swaps table addresses for lpt1 and lpt2.

7                                       PORT TEST User's Manual

CUSTOMER FEEDBACK FORM

We continue to strive to provide a quality product.  Your

comments, suggestions, criticisms, etc. are most welcome.  Just

fill out this form and send or fax it to us.

Brand and model of computer:______________________________

Comment (  ) Suggestion  (  ) Criticism (  )  Problem (  )

Other_____________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

Name:  _____________________________________

Company:  __________________________________

Address:  __________________________________

City, State, Zip:  _________________________

Phone:  ______________________  Fax:  ______

Do you need a reply?  _________   Are you a registered user?

We guarantee a reply to all registered users who request one.

Mail or FAX this form to:     MicroSystems Development

4100 Moorpark Ave. #104

FAX:  (408)296-5877           San Jose, CA   95117   USA

8                                       PORT TEST User's Manual

##############################################################################################################################