������������������������������������������������������������������������������ͻ

�                      CrossFire Assembler, By Mark Parry                      �

�                                  680x0.DOC                                   �

������������������������������������������������������������������������������ͼ

This document gives details of the Motorola 680x0 instruction set. You should

also look at the document "FILETYPE.DOC" to see how various output file formats

are supported, especially for types ".PRG" and ".ACC".

������������������������������������������������������������������������������Ŀ

�                                Document Guide                                �

��������������������������������������������������������������������������������

Flags:	XNZVC

*  set according to operation		X = extend carry

U  undefined				N = negative

-  not affected				Z = zero

0  cleared				V = overflow

1  set					C = carry

Examples for each instruction are given. If <ea> is used, the valid effective

addresses for the instruction are listed. ext(An) and ext(PC) means the extended

addressing modes of the 68020, 68030, 68040 and CPU32 can be used.

If address register An is used, only word and long are allowed (however, byte

address register An is allowed for CHK2, CMP2 and MOVES).

{68EC000}	�

{68010}		�

{68020}		�

{68030}		� Specifies which processors / coprocessor

{68040}		� the instruction will work on.

{68EC040}	� If none are specified, the instruction

{68LC040}	� works on the whole 680x0 family of processors.

{CPU32}		�

{68881}		�

{68882}		�

{!!!}		instruction has been added for the alternative syntax.

{priv}		privileged instruction

{priv?}		MOVE from SR is a privileged instruction unless on 68000, 68008

Only instructions that work on the 68000 have been tested!!!

������������������������������������������������������������������������������ͻ

�                            680x0 Instruction Set                             �

������������������������������������������������������������������������������ͼ

ABCD (add binary coded decimal)

Syntax:	ABCD Dx,Dy

ABCD -(Ax),-(Ay)

Flags:	*U*U*

Size:	byte

Adds the source to the destination along with the extend bit, storing

the result in the destination. Addition is performed using binary coded

decimal (BCD) arithmetic.

ABCD D4,D5

ABCD -(A4),-(A5)

--------------------------------------------------------------------------------

ADD (add)

Syntax:	ADD.? <ea>,Dn

ADD.? Dn,<ea>

Flags:	*****

Size:	byte, word, long

ADDs the source to the destination, storing the result in the

destination.

ADD.B <ea>,D4   ADD.W <ea>,D4   ADD.L <ea>,D4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

ADD.B D4,<ea>   ADD.W D4,<ea>   ADD.L D4,<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

see also: ADDA ADDI ADDQ

--------------------------------------------------------------------------------

ADDA (add to address register)

Syntax:	ADDA.? <ea>,An

ADD.? <ea>,An

Flags:	-----

Size:	word, long

ADDs the source to the destination register, storing the result in the

destination address register. Word length operands are sign-extended to

32-bits for addition.

ADDA.W <ea>,A4   ADDA.L <ea>,A4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

ADDI (add immediate)

Syntax:	ADDI.? #imm,<ea>

ADD.? #imm,<ea>

Flags:	*****

Size:	byte, word, long

ADDs the immediate value to the destination, storing the result in the

destination.

ADDI.B #imm,<ea>   ADDI.W #imm,<ea>   ADDI.L #imm,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ADDQ (add quick)

Syntax:	ADDQ.? #imm,<ea>

Flags:	***** (----- if the destination is an address register)

Size:	byte, word, long (word, long if the destination is an address register)

ADDs the immediate value (from 1 to 8) to the destination, storing the

result in the destination. If the destination is an address register,

word length operands are sign-extended to 32-bits for addition.

ADDQ.B #imm,<ea>   ADDQ.W #imm,<ea>   ADDQ.L #imm,<ea>

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ADDX (add with extend)

Syntax:	ADDX.? Dx,Dy

ADDX.? -(Ax),-(Ay)

Flags:	*****

Size:	byte, word, long

ADDs the source to the destination along with the extend bit, storing

the result in the destination.

ADDX.B D4,D5   ADDX.W D4,D5   ADDX.L D4,D5

ADDX.B -(A4),-(A5)   ADDX.W -(A4),-(A5)   ADDX.L -(A4),-(A5)

--------------------------------------------------------------------------------

AND (logical AND)

Syntax:	AND.? <ea>,Dn

AND.? Dn,<ea>

Flags:	-**00

Size:	byte, word, long

ANDs the source with the destination, storing the result in the

destination.

AND.B <ea>,D4   AND.W <ea>,D4   AND.L <ea>,D4

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

AND.B D4,<ea>   AND.W D4,<ea>   AND.L D4,<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ANDI (logical AND, immediate)

Syntax:	ANDI.? #imm,<ea>

AND.? #imm,<ea>

Flags:	-**00

Size:	byte, word, long

ANDs the immediate value with the destination, storing the result in the

destination.

ANDI.B #imm,<ea>   ANDI.W #imm,<ea>   ANDI.L #imm,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ANDI to CCR (logical AND, immediate to condition codes)

Syntax:	ANDI #imm,CCR

AND #imm,CCR

Flags:	X cleared if bit 4 of #imm is zero, unchanged otherwise

N cleared if bit 3 of #imm is zero, unchanged otherwise

Z cleared if bit 2 of #imm is zero, unchanged otherwise

V cleared if bit 1 of #imm is zero, unchanged otherwise

C cleared if bit 0 of #imm is zero, unchanged otherwise

Size:	byte

ANDs the immediate value with the condition codes, storing the result in

the condition code register.

ANDI #imm,CCR

--------------------------------------------------------------------------------

ANDI to SR (logical AND, immediate to status register) {priv}

Syntax:	ANDI #imm,SR

AND #imm,SR

Flags:	X cleared if bit 4 of #imm is zero, unchanged otherwise

N cleared if bit 3 of #imm is zero, unchanged otherwise

Z cleared if bit 2 of #imm is zero, unchanged otherwise

V cleared if bit 1 of #imm is zero, unchanged otherwise

C cleared if bit 0 of #imm is zero, unchanged otherwise

Size:	word

ANDs the immediate value with the status register, storing the result in

the status register.

ANDI #imm,SR

--------------------------------------------------------------------------------

ASL (arithmetic shift left)

Syntax:	ASL.? Dx,Dy

ASL.? #count,Dy

ASL <ea>

Flags:	*****	(-**00 if count=0)

Size:	byte, word, long	(word if ASL <ea>)

Shift the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using ASL <ea>. Bits shifted out of the high

order bit go to both the carry and extend flags; zeros are shifted into

the low order bit. The overflow bit indicates if any sign changes

occurred.

ASL.B D4,D5   ASL.W D4,D5   ASL.L D4,D5

ASL.B #count,D5   ASL.W #count,D5   ASL.L #count,D5

ASL <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ASR (arithmetic shift right)

Syntax:	ASR.? Dx,Dy

ASR.? #count,Dy

ASR <ea>

Flags:	***0*	(-**00 if count=0)

Size:	byte, word, long	(word if ASR <ea>)

Shift the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using ASR <ea>. Bits shifted out of the low

order bit go to both the carry and extend flags; the sign bit is shifted

into the high order bit.

ASR.B D4,D5   ASR.W D4,D5   ASR.L D4,D5

ASR.B #count,D5   ASR.W #count,D5   ASR.L #count,D5

ASR <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

Bcc (branch conditionally)

Syntax:	Bcc <label>

Flags:	-----

Size:	short (or byte), word or long	(long is 68020, 68030, 68040 only)

If the specified condition is true, branch to the specified label.

Short branch to the following instruction is not allowed (BEQ.S *+2).

The default branch size is word.

conditions:-

BCC	BHS	branch if carry clear / higher or same (unsigned)

BCS	BLO	branch if carry set / lower (unsigned)

BEQ		branch if equal

BGE		branch if greater or equal (signed)

BGT		branch if greater than (signed)

BHI		branch if higher than (unsigned)

BLE		branch if less or equal (signed)

BLS		branch if lower or the same (unsigned)

BLT		branch if less than (signed)

BMI		branch if minus

BNE		branch if not equal

BPL		branch if plus

BVC		branch if overflow clear

BVS		branch if overflow set

BEQ.S label   BEQ.B label   BEQ.W label   BEQ.L label

--------------------------------------------------------------------------------

BCHG (test bit and change)

Syntax:	BCHG Dn,<ea>

BCHG #bit,<ea>

Flags:	--*--

Size:	byte, long

Test the specified bit (Dn or #bit) of the destination, setting the Z

condition code. Then invert the specified bit. The bit number is modulo

32 (from 0 to 31, long) if the destination is a data register, or modulo

8 (from 0 to 7, byte) if the destination is not a data register.

BCHG D4(modulo 32),D5   BCHG #bit(0-31),D5

BCHG D4(modulo 8),<ea>   BCHG #bit(0-7),<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BCLR (test bit and clear)

Syntax:	BCLR Dn,<ea>

BCLR #bit,<ea>

Flags:	--*--

Size:	byte, long

Test the specified bit (Dn or #bit) of the destination, setting the Z

condition code. Then clear the specified bit. The bit number is modulo

32 (from 0 to 31, long) if the destination is a data register, or modulo

8 (from 0 to 7, byte) if the destination is not a data register.

BCLR D4(modulo 32),D5   BCLR #bit(0-31),D5

BCLR D4(modulo 8),<ea>   BCLR #bit(0-7),<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BFCHG (test bit field and change) {68020} {68030} {68040}

Syntax:	BFCHG <ea>{offset:width}

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

The bit field is then complemented (inverted).

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be modified by this instruction.

BFCHG <ea>{D4:D5}  {D4:#width}  {#offset:D5}  {#offset:#width}

Dn (An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BFCLR (test bit field and clear) {68020} {68030} {68040}

Syntax:	BFCLR <ea>{offset:width}

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

The bit field is then cleared.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be modified by this instruction.

BFCLR <ea>{D4:D5}  {D4:#width}  {#offset:D5}  {#offset:#width}

Dn (An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BFEXTS (extract bit field signed) {68020} {68030} {68040}

Syntax:	BFEXTS <ea>{offset:width},Dn

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

Extracts a bit field from the specified extended address location, sign

extends to 32-bits, and loads the result into the destination data

register.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be accessed by this instruction.

BFEXTS <ea>{D4:D5},D6  {D4:#width},D6  {#offset:D5},D6  {#offset:#width},D6

Dn (An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

BFEXTU (extract bit field unsigned) {68020} {68030} {68040}

Syntax:	BFEXTU <ea>{offset:width},Dn

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

Extracts a bit field from the specified extended address location, zero

extends to 32-bits, and loads the result into the destination data

register.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be accessed by this instruction.

BFEXTU <ea>{D4:D5},D6  {D4:#width},D6  {#offset:D5},D6  {#offset:#width},D6

Dn (An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

BFFFO (find first one in bit field) {68020} {68030} {68040}

BFFF1 {!!!} (find first one in bit field) {68020} {68030} {68040}

Syntax:	BFFFO <ea>{offset:width},Dn

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

Finds the first set bit in the bit field, and places the bit number in

destination data register Dn (the bit offset in the instruction plus the

offset of the first one bit). If all bits of the field are clear, Dn is

set to field offset+width.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be accessed by this instruction.

BFFFO <ea>{D4:D5},D6  {D4:#width},D6  {#offset:D5},D6  {#offset:#width},D6

Dn (An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

BFINS (insert into bit field) {68020} {68030} {68040}

Syntax:	BFINS Dn,<ea>{offset:width}

Flags:	-**00

Size:	unsized

Inserts a bit field taken from the low order bits of the specified data

register into a bit field at the effective address location. Then the

instruction sets the condition codes according to the inserted value.

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be modified by this instruction.

BFINS D6,<ea>{D4:D5}  D6,{D4:#width}  D6,{#offset:D5}  D6,{#offset:#width}

Dn (An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BFSET (test bit field and set) {68020} {68030} {68040}

Syntax:	BFSET <ea>{offset:width}

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

Each bit in the field is then set.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be modified by this instruction.

BFSET <ea>{D4:D5}  {D4:#width}  {#offset:D5}  {#offset:#width}

Dn (An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BFTST (test bit field) {68020} {68030} {68040}

Syntax:	BFTST <ea>{offset:width}

Flags:	-**00

Size:	unsized

Sets the condition codes according to the value in the bit field :-

N set the same as the most significant bit of the field.

Z set if all bits of the field are zero, cleared otherwise.

{offset:	Dn: the range is -$80000000 to +$7FFFFFFF, which will

access memory -$10000000 to +$0FFFFFFF from <ea>.

#offset: the range is 0 to 31, which will access memory

+0 to +4 from <ea>.

:width}		:Dn the range is modulo 32, with 0=32 (width = 1 to 32).

:#width the range is 1 to 32.

Up to 5 bytes of memory can be accessed by this instruction.

BFTST <ea>{D4:D5}  {D4:#width}  {#offset:D5}  {#offset:#width}

Dn (An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

BGND (enter background mode) {CPU32}

Syntax:	BGND

Flags:	-----

Size:	unsized

The processor suspends instruction execution and enters background mode

if background mode is enabled. The freeze output is asserted to

acknowledge entrance into background mode.

If background mode is not enabled, the processor initiates illegal

instruction exception processing.

BGND

--------------------------------------------------------------------------------

BKPT (breakpoint) {68EC000} {68010} {68020} {68030} {68040} {CPU32}

Syntax:	BKPT #data

Flags:	-----

Size:	unsized

Run breakpoint acknowledge cycle; Trap as illegal instruction.

BKPT #data(0-7)

--------------------------------------------------------------------------------

BRA (branch always)

Syntax:	BRA <label>

Flags:	-----

Size:	short (or byte), word or long	(long is 68020, 68030, 68040 only)

Branch to the specified label. Short branch to the following instruction

is not allowed (BRA.S *+2). The default branch size is word.

BRA.S label   BRA.B label   BRA.W label   BRA.L label

--------------------------------------------------------------------------------

BSET (test bit and set)

Syntax:	BSET Dn,<ea>

BSET #bit,<ea>

Flags:	--*--

Size:	byte, long

Test the specified bit (Dn or #bit) of the destination, setting the Z

condition code. Then set the specified bit. The bit number is modulo 32

(from 0 to 31, long) if the destination is a data register, or modulo 8

(from 0 to 7, byte) if the destination is not a data register.

BSET D4(modulo 32),D5   BSET #bit(0-31),D5

BSET D4(modulo 8),<ea>   BSET #bit(0-7),<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

BSR (branch to subroutine)

Syntax:	BSR <label>

Flags:	-----

Size:	short (or byte), word or long	(long is 68020, 68030, 68040 only)

Store the long word address of the next instruction on the stack, then

branch to the specified label. Short branch to the following instruction

is not allowed (BSR.S *+2). The default branch size is word.

BSR.S label   BSR.B label   BSR.W label   BSR.L label

--------------------------------------------------------------------------------

BTST (test bit)

Syntax:	BTST Dn,<ea>

BTST #bit,<ea>

Flags:	--*--

Size:	byte, long

Test the specified bit (Dn or #bit) of the destination, setting the Z

condition code. The bit number is modulo 32 (from 0 to 31, long) if the

destination is a data register, or modulo 8 (from 0 to 7, byte) if the

destination is not a data register.

BTST D4(modulo 32),D5   BTST #bit(0-31),D5

BTST D4(modulo 8),<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

BTST #bit(0-7),<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

CALLM (call module) {68020}

Syntax:	CALLM #data,<ea>

Flags:	-----

Size:	unsized

The effective address of the instruction is the location of an external

module descriptor. A module frame is created on the top of the stack,

and the current module state is saved in the frame. The immediate

operand specifies the number of bytes of arguments to be passed to the

called module. A new module state is loaded from the descriptor

addressed by the effective address.

CALLM #data(0-255),<ea>

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

CAS (compare and swap with operand) {68020} {68030} {68040}

Syntax:	CAS.? Dc,Du,<ea>

Flags:	-****

Size:	byte, word, long

CAS compares the effective address operand to the compare operand (Dc).

If the operands are equal, the instruction writes the update operand

(Du) to the effective address operand; otherwise, the instruction writes

the effective address operand to the compare operand (Dc).

Memory access uses locked or read-modify-write transfer sequences,

providing a means of synchronizing several processors.

CAS.B D4,D5,<ea>   CAS.W D4,D5,<ea>   CAS.L D4,D5,<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

CAS2 (compare and swap with operand) {68020} {68030} {68040}

Syntax:	CAS2.? Dc1:Dc2,Du1:Du2,(Rn1):(Rn2)

Flags:	-****

Size:	word, long

CAS2 compares memory operand 1 (Rn1) to compare operand 1 (Dc1). If the

operands are equal, the instruction compares memory operand 2 (Rn2) to

compare operand 2 (Dc2). If these are also equal, the instruction writes

the update operands (Du1 and Du2) to the memory operands (Rn1 and Rn2).

If either comparison fails, the instruction writes the memory operands

(Rn1 and Rn2) to the compare operands (Dc1 and Dc2).

Memory access uses locked or read-modify-write transfer sequences,

providing a means of synchronizing several processors.

CAS2.W D4:D5,D6:D7,(A2):(A3)   CAS2.L D4:D5,D6:D7,(A2):(A3)

--------------------------------------------------------------------------------

CHK (check register against bounds)

Syntax:	CHK <ea>,Dn

Flags:	-*UUU

Size:	word, long		(long is 68020, 68030, 68040 only)

Compares the value in the specified data register to zero and the upper

bound (effective address operand). The upper bound is a twos complement

integer. If the register value is less than zero or greater than the

upper bound, a CHK instruction exception (vector number 6) occurs. The

default instruction size is word.

CHK.W <ea>,D4   CHK.L <ea>,D4

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

CHK2 (check register against bounds) {68020} {68030} {68040} {CPU32}

Syntax:	CHK2.? <ea>,Rn

Flags:	-U*U*

Size:	byte, word, long

Compares the value in Rn to each bound. The effective address contains

the bound pair; the upper bound following the lower bound. For signed

comparisons, the arithmetically smaller value should be used as the

lower bound. For unsigned comparisons, the logically smaller value

should be the lower bound.

The size of the data and the bounds can be specified as byte, word or

long. If Rn is a data register and the operation size is byte or word,

only the byte or word part of Dn is checked. If Rn is an address

register and the operation size is byte or word, the bounds operands are

sign-extended to 32-bits, and the resultant operands are compared to the

full 32-bits of An.

If the upper bound equals the lower bound, the valid range is a single

value. If the register value is less than the lower bound or greater

than the upper bound, a CHK instruction exception (vector number 6)

occurs.

CHK2.B <ea>,D4   CHK2.W <ea>,D4   CHK2.L <ea>,D4

CHK2.B <ea>,A4   CHK2.W <ea>,A4   CHK2.L <ea>,A4

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

CINV (invalidate cache entries) {68040} {68LC040} {priv}

This instruction is not currently implemented because I am unsure of the correct

syntax.

--------------------------------------------------------------------------------

CLR (clear an operand)

Syntax:	CLR.? <ea>

Flags:	-0100

Size:	byte, word, long

Clears the destination effective address to zero.

CLR.B <ea>   CLR.W <ea>   CLR.L <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

CMP (compare)

Syntax:	CMP.? <ea>,Dn

Flags:	-****

Size:	byte, word, long

Subtracts the source operand from the destination data register and sets

the condition codes according to the result; the data register is not

changed.

CMP.B <ea>,D4   CMP.W <ea>,D4   CMP.L <ea>,D4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

CMPA (compare address)

Syntax:	CMPA.? <ea>,An

CMP.? <ea>,An

Flags:	-****

Size:	word, long

Subtracts the source operand from the destination address register and

sets the condition codes according to the result; the address register

is not changed. Word length operands are sign-extended to 32-bits for

comparison.

CMPA.W <ea>,A4   CMPA.L <ea>,A4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

CMPI (compare immediate)

Syntax:	CMPI.? #imm,<ea>

CMP.? #imm,<ea>

Flags:	-****

Size:	byte, word, long

Subtracts the immediate operand from the destination effective address

and sets the condition codes according to the result; the effective

address is not changed.

CMPI.B #imm,<ea>   CMPI.W #imm,<ea>   CMPI.L #imm,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

d16(PC) d8(PC,Ix) ext(PC)   {68020} {68030} {68040} {CPU32}

--------------------------------------------------------------------------------

CMPM (compare memory)

Syntax:	CMPM.? (Ay)+,(Ax)+

CMP.? (Ay)+,(Ax)+

Flags:	-****

Size:	byte, word, long

Subtracts the source operand from the destination operand and sets the

condition code according to the result; the destination operand is not

changed.

CMPM.B (A4)+,(A5)+   CMPM.W (A4)+,(A5)+   CMPM.L (A4)+,(A5)+

--------------------------------------------------------------------------------

CMP2 (compare register against bounds) {68020} {68030} {68040} {CPU32}

Syntax:	CMP2.? <ea>,Rn

Flags:	-U*U*

Size:	byte, word, long

Compares the value in Rn to each bound. The effective address contains

the bound pair; the upper bound following the lower bound. For signed

comparisons, the arithmetically smaller value should be used as the

lower bound. For unsigned comparisons, the logically smaller value

should be the lower bound.

The size of the data and the bounds can be specified as byte, word or

long. If Rn is a data register and the operation size is byte or word,

only the byte or word part of Dn is checked. If Rn is an address

register and the operation size is byte or word, the bounds operands are

sign-extended to 32-bits, and the resultant operands are compared to the

full 32-bits of An.

If the upper bound equals the lower bound, the valid range is a single

value.

CMP2.B <ea>,D4   CMP2.W <ea>,D4   CMP2.L <ea>,D4

CMP2.B <ea>,A4   CMP2.W <ea>,A4   CMP2.L <ea>,A4

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

cpBcc (branch on coprocessor condition) {68020} {68030}

cpDBcc (test coprocessor condition, decrement and branch) {68020} {68030}

cpGEN (coprocessor general function) {68020} {68030}

cpScc (set on coprocessor condition) {68020} {68030}

cpTRAPcc (trap on coprocessor condition) {68020} {68030}

cpRESTORE (coprocessor restore functions) {68020} {68030} {priv}

cpSAVE (coprocessor save function) {68020} {68030} {priv}

These instructions are not currently implemented because I am unsure of the

correct syntax.

--------------------------------------------------------------------------------

CPUSH (push and invalidate cache entries) {68040} {68LC040} {priv}

This instruction is not currently implemented because I am unsure of the correct

syntax.

--------------------------------------------------------------------------------

DBcc (test condition, decrement and branch)

Syntax:	DBcc Dn,label

Flags:	-----

Size:	word

This instruction first tests the condition for termination; if it is

true, no operation is performed. If the termination condition is not

true, the low order 16-bits of the counter data register are decremented

by one. If the result is -1, execution continues with the next

instruction. If the result is not -1, execution continues at label.

conditions:-

DBRA	DBF	always (unconditionally)

DBT		never (no operation)

DBCC	DBHS	until carry clear / higher or same (unsigned)

DBCS	DBLO	until carry set / lower (unsigned)

DBEQ		until equal

DBGE		until greater or equal (signed)

DBGT		until greater than (signed)

DBHI		until higher than (unsigned)

DBLE		until less or equal (signed)

DBLS		until lower or the same (unsigned)

DBLT		until less than (signed)

DBMI		until minus

DBNE		until not equal

DBPL		until plus

DBVC		until overflow clear

DBVS		until overflow set

DBRA D4,label

--------------------------------------------------------------------------------

DIVS DIVSL (signed divide)

Syntax:	DIVS.W <ea>,Dn

DIVS.L <ea>,Dq	    1	�

DIVS.L <ea>,Dr:Dq   2	� {68020} {68030} {68040} {CPU32}

DIVSL.L <ea>,Dr:Dq  3	�

Flags:	-***0

Size:	word, long		(long is 68020, 68030, 68040, CPU32 only)

Divides the signed destination operand by the signed source operand and

stores the signed result in the destination. The instruction uses one of

four forms:

word	divides a long by a word. The result is a quotient in the lower

word, and a remainder in the upper word. The sign of the

remainder is the same as the sign of the dividend.

long 1	divides a long by a long. The result is a long quotient, the

remainder is discarded.

long 2	divides a quad word (in any two different data registers) by a

long word. The result is a long quotient and a long remainder.

long 3	divides a long word by a long word. The result is a long

quotient and a long remainder.

Dr:Dq cannot be the same register. Division by zero causes a trap. The

operands are unaffected if an overflow occurs.

DIVS.W <ea>,D4

DIVS.L <ea>,D4

DIVS.L <ea>,D4:D5

DIVSL.L <ea>,D4:D5

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

DIVU DIVUL (unsigned divide)

Syntax:	DIVU.W <ea>,Dn

DIVU.L <ea>,Dq	    1	�

DIVU.L <ea>,Dr:Dq   2	� {68020} {68030} {68040} {CPU32}

DIVUL.L <ea>,Dr:Dq  3	�

Flags:	-***0

Size:	word, long		(long is 68020, 68030, 68040, CPU32 only)

Divides the unsigned destination operand by the unsigned source operand

and stores the unsigned result in the destination. The instruction uses

one of four forms:

word	divides a long by a word. The result is a quotient in the lower

word, and a remainder in the upper word.

long 1	divides a long by a long. The result is a long quotient, the

remainder is discarded.

long 2	divides a quad word (in any two different data registers) by a

long word. The result is a long quotient and a long remainder.

long 3	divides a long word by a long word. The result is a long

quotient and a long remainder.

Dr:Dq cannot be the same register. Division by zero causes a trap. The

operands are unaffected if an overflow occurs.

DIVU.W <ea>,D4

DIVU.L <ea>,D4

DIVU.L <ea>,D4:D5

DIVUL.L <ea>,D4:D5

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

EOR (logical exclusive OR)

Syntax:	EOR.? Dn,<ea>

Flags:	-**00

Size:	byte, word, long

EORs the source with the destination, storing the result in the

destination.

EOR.B D4,<ea>   EOR.W D4,<ea>   EOR.L D4,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

EORI (logical exclusive OR, immediate)

Syntax:	EORI.? #imm,<ea>

EOR.? #imm,<ea>

Flags:	-**00

Size:	byte, word, long

EORs the immediate value with the destination, storing the result in the

destination.

EORI.B #imm,<ea>   EORI.W #imm,<ea>   EORI.L #imm,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

EORI to CCR (logical exclusive OR, immediate to condition codes)

Syntax:	EORI #imm,CCR

EOR #imm,CCR

Flags:	X changed if bit 4 of #imm is one, unchanged otherwise

N changed if bit 3 of #imm is one, unchanged otherwise

Z changed if bit 2 of #imm is one, unchanged otherwise

V changed if bit 1 of #imm is one, unchanged otherwise

C changed if bit 0 of #imm is one, unchanged otherwise

Size:	byte

EORs the immediate value with the condition codes, storing the result in

the condition code register.

EORI #imm,CCR

--------------------------------------------------------------------------------

EORI to SR (logical exclusive OR, immediate to status register) {priv}

Syntax:	EORI #imm,SR

EOR #imm,SR

Flags:	X changed if bit 4 of #imm is one, unchanged otherwise

N changed if bit 3 of #imm is one, unchanged otherwise

Z changed if bit 2 of #imm is one, unchanged otherwise

V changed if bit 1 of #imm is one, unchanged otherwise

C changed if bit 0 of #imm is one, unchanged otherwise

Size:	word

EORs the immediate value with the status register, storing the result in

the status register.

EORI #imm,SR

--------------------------------------------------------------------------------

EXG (exchange registers)

Syntax:	EXG Rn,Rn

Flags:	-----

Size:	long

Exchanges the contents of two 32-bit registers.

EXG D4,D5   EXG D4,A5   EXG A4,D5   EXG A4,A5

--------------------------------------------------------------------------------

EXT (sign extend)

EXTB (sign extend byte to long) {68020} {68030} {68040} {CPU32}

Syntax:	EXT.W Dn	extend byte to word

EXT.L Dn	extend word to long

EXTB.L Dn	extend byte to long {68020} {68030} {68040} {CPU32}

Flags:	-**00

Size:	word, long

Sign extends a byte in a data register to word, a word in a data

register to long, or (EXTB) extends a byte in a data register to long.

EXT.W D4   EXT.L D4   EXTB.L D4

--------------------------------------------------------------------------------

FRESTORE (restore internal floating point state)

{68881} {68882} {68040} {priv}

FSAVE (save internal floating point state)

{68881} {68882} {68040} {priv}

These instructions are not currently implemented because I am unsure of the

correct syntax.

--------------------------------------------------------------------------------

ILLEGAL (take illegal instruction trap)

Syntax:	ILLEGAL

Flags:	-----

Size:	unsized

Forces an illegal instruction exception, vector number 4. All other

illegal instruction bit patterns are reserved for future extension of

the instruction set, and should not be used to force an exception.

ILLEGAL

--------------------------------------------------------------------------------

JMP (jump to effective address)

Syntax:	JMP <ea>

Flags:	-----

Size:	unsized

Program continues execution at the specified effective address.

JMP <ea>

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

JSR (jump to subroutine)

Syntax:	JSR <ea>

Flags:	-----

Size:	unsized

Store the long word address of the next instruction onto the stack, then

program continues execution at the specified effective address.

JSR <ea>

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

LEA (load effective address)

Syntax:	LEA <ea>,An

Flags:	-----

Size:	long

Loads the effective address into the address register. All 32-bits of

the address register are affected.

LEA <ea>,A4

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

LINK (link and allocate)

Syntax:	LINK An,#displacement

Flags:	-----

Size:	word, long		(long is 68020, 68030, 68040, CPU32 only)

Stores the contents of the specified address register on the stack, then

loads the updated stack pointer into the address register. Finally, adds

the displacement value to the stack pointer. The displacement is sign

extended for word size operation. The displacement should be a negative

value in order to allocate stack area.  LINK and UNLINK can be used to

keep local data and parameter areas on the stack for nested subroutines.

LINK.W A4,#displacement   LINK.L A4,#displacement

--------------------------------------------------------------------------------

LPSTOP (low power stop) {CPU32} {priv}

Syntax:	LPSTOP #data

Flags:	set according to the immediate word

Size:	unsized

Moves the immediate word into the status register, advances the program

counter to the next instruction, and stops the fetching and executing of

instructions. A CPU LPSTOP broadcast cycle is executed to CPU space $3

to copy the updated interrupt mask to the external bus interface (EBI).

The internal clocks are stopped.

Instruction execution resumes when a trace, interrupt, or reset

exception occurs. A trace exception will occur if the trace state is on

when the LPSTOP instruction is executed. If an interrupt request is

asserted with a priority higher than the priority level set by the new

status register, an interrupt exception occurs; otherwise, the interrupt

request is ignored. An external reset always initiates reset exception

processing.

LPSTOP #word

--------------------------------------------------------------------------------

LSL (logical shift left)

Syntax:	LSL.? Dx,Dy

LSL.? #count,Dy

LSL <ea>

Flags:	***0*	(-**00 if count=0)

Size:	byte, word, long	(word if LSL <ea>)

Shift the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using LSL <ea>. Bits shifted out of the high

order bit go to both the carry and extend flags; zeros are shifted into

the low order bit.

LSL.B D4,D5   LSL.W D4,D5   LSL.L D4,D5

LSL.B #count,D5   LSL.W #count,D5   LSL.L #count,D5

LSL <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

LSR (logical shift right)

Syntax:	LSR.? Dx,Dy

LSR.? #count,Dy

LSR <ea>

Flags:	***0*	(-**00 if count=0)

Size:	byte, word, long	(word if LSR <ea>)

Shift the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using LSR <ea>. Bits shifted out of the low

order bit go to both the carry and extend flags; zeros are shifted into

the high order bit.

LSR.B D4,D5   LSR.W D4,D5   LSR.L D4,D5

LSR.B #count,D5   LSR.W #count,D5   LSR.L #count,D5

LSR <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

MOVE (move data from source to destination)

Syntax:	MOVE.? source <ea>,destination <ea>

Flags:	-**00

Size:	byte, word, long

MOVEs the data from the source operand to the destination operand,

setting the condition codes accordingly.

MOVE.B <ea1>,<ea2>   MOVE.W <ea1>,<ea2>   MOVE.L <ea1>,<ea2>

<ea1>:	Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

<ea2>:	Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

MOVEA (move data to address register)

Syntax:	MOVEA.? <ea>,An

MOVE.? <ea>,An

Flags:	-----

Size:	word, long

MOVEs the data from the source operand to the destination address

register.

MOVE.W <ea>,A4   MOVE.L <ea>,A4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

MOVE from CCR (move from the condition code register)

{68010} {68020} {68030} {68040} {CPU32}

Syntax:	MOVE CCR,<ea>

Flags:	-----

Size:	word

MOVEs the condition code register to the destination effective address

(unimplemented bits are read as zero). MOVE from CCR is a word

operation; ANDI, ORI and EORI to CCR are byte operations.

MOVE CCR,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

MOVE to CCR (move to the condition code register)

Syntax:	MOVE <ea>,CCR

Flags:	X set to bit 4 of #imm

N set to bit 3 of #imm

Z set to bit 2 of #imm

V set to bit 1 of #imm

C set to bit 0 of #imm

Size:	word

MOVEs the source operand to the condition code register. MOVE to CCR is

a word operation; ANDI, ORI and EORI to CCR are byte operations.

Unimplemented bits of the condition code register are unaffected.

MOVE <ea>,CCR

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

MOVE from SR (move from the status register) {priv?}

Syntax:	MOVE SR,<ea>

Flags:	-----

Size:	word

MOVEs the status register to the destination effective address

(unimplemented bits are read as zero). MOVE from SR is not a privileged

instruction on the 68000 and 68008; it is a privileged instruction on

the 68EC000, 68010, 68020, 68030, 68040 and CPU32.

MOVE SR,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

MOVE to SR (move to the status register) {priv}

Syntax:	MOVE <ea>,SR

Flags:	X set to bit 4 of #imm

N set to bit 3 of #imm

Z set to bit 2 of #imm

V set to bit 1 of #imm

C set to bit 0 of #imm

Size:	word

MOVEs the data from the source to the status register. Unimplemented

bits of the status register are unaffected.

MOVE <ea>,SR

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

MOVE USP (move user stack pointer) {priv}

Syntax:	MOVE USP,An

MOVE An,USP

Flags:	-----

Size:	long

MOVEs the contents of the user stack pointer to or from the specified

address register.

MOVE USP,A5

MOVE A4,USP

--------------------------------------------------------------------------------

MOVE16 (move 16 byte block) {68040}

Syntax:	MOVE16 (Ax)+,(Ay)+

MOVE16 mem.l,(An)

MOVE16 mem.l,(An)+

MOVE16 (An),mem.l

MOVE16 (An)+,mem.l

Flags:	-----

Size:	line (16 bytes)

MOVEs the source line to the destination line. The lines are aligned

to 16 byte boundaries. Applications for this instruction include

coprocessor communications, memory initialization and fast copying.

MOVE16 (A4)+,(A5)+

MOVE16 mem.l,(A5)

MOVE16 mem.l,(A5)+

MOVE16 (A4),mem.l

MOVE16 (A4)+,mem.l

--------------------------------------------------------------------------------

MOVEC (move control register)

{68010} {68020} {68030} {68040} {CPU32} {priv}

Syntax:	MOVEC Rc,Rn

MOVEC Rn,Rc

Flags:	-----

Size:	long

MOVEs the contents of the specified control register (Rc) to or from the

specified general register (Rn). This is always a 32-bit transfer, even

though the control register may be implemented with fewer bits, with

unimplemented bits read as zero.

This is a list of the control registers :-

SFC	source function code

DFC	destination function code

USP	user stack pointer		{68010} {68020} {68030}

VBR	vector base register		{68040} {CPU32}

CACR	cache control register

CAAR	cache address register

MSP	master stack pointer

ISP	interrupt stack pointer		{68020} {68030}

CACR	cache control register

MSP	master stack pointer

ISP	interrupt stack pointer		{68040}

TC	MMU translation control register

ITT0	instruction transparent translation register 0

ITT1	instruction transparent translation register 1

DTT0	data transparent translation register 0

DTT1	data transparent translation register 1

MMUSR	MMU status register

URP	user root pointer

SRP	supervisor root pointer		{68040} {68LC040}

IACR0	instruction access control register 0

IACR1	instruction access control register 1

DACR0	data access control register 0

DACR1	data access control register 1	{68EC040}

MOVEC SFC,D5   MOVEC SFC,A5

MOVEC D4,DFC   MOVEC A4,DFC

--------------------------------------------------------------------------------

MOVEM (move multiple registers)

Syntax:	MOVEM.? register list,<ea>

MOVEM.? <ea>,register list

Flags:	-----

Size:	word, long

MOVEs the selected registers to or from consecutive memory locations

starting at the location specified by the effective address. When

transferring words from memory, each word is sign-extended to 32-bits

before being loaded into the data or address registers.

Registers are transferred to or from memory in the order of D0-D7, then

A0-A7; UNLESS using the predecrement address mode -(An), where the

registers are transferred in the order of A7-A0, then D7-D0.

Any data register or address register can be included in the register

list. Examples of register lists are given :-

D0/A0/A4/D6					mixed registers D0 A0 A4 D6

D0/D1/D2/D3/D4/D5/D6/D7   D0-D7   D0-7		all data registers

D1-D2/D4-D6/A1-2/A4-6				D1 D2 D4 D5 D6 A1 A2 A4 A5 A6

MOVEM.W register list,<ea>   MOVEM.L register list,<ea>

(An) -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

MOVEM.W <ea>,register list   MOVEM.L <ea>,register list

(An) (An)+ d16(An) d8(An,Ix) mem.w mem.l

d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

MOVEP (move peripheral data)

Syntax:	MOVEP.? Dx,d16(Ay)

MOVEP.? d16(Ay),Dx

Flags:	-----

Size:	word, long

MOVEs data between a data register and alternate bytes within the

address space starting at the location specified and incrementing by

two. The high order byte of the data register is transferred first, and

the low order byte is transferred last.

This instruction was designed for interfacing 8-bit peripherals on a

16-bit data bus. Although still supported by the 68020, 68030 and 68040,

you can not interface 8-bit peripherals on a 32-bit data bus.

MOVEP.W D4,d16(A5)   MOVEP.L D4,d16(A5)

MOVEP.W d16(A4),D5   MOVEP.L d16(A4),D5

--------------------------------------------------------------------------------

MOVEQ (move quick)

Syntax:	MOVEQ #data,Dn

Flags:	-**00

Size:	long

MOVEs a sign-extended byte to the 32-bit data register.

MOVEQ #-128 to #127 is the proper data range.

MOVEQ #-256 to #255 will give a 'sign-extended data' warning

MOVEQ.L #-256 to #255 will suppress the 'sign-extended data' warning

MOVEQ #data (-256 to 255),Dn

--------------------------------------------------------------------------------

MOVES (move address space)

{68010} {68020} {68030} {68040} {CPU32} {priv}

Syntax:	MOVES.? Rn,<ea>

MOVES.? <ea>,Rn

Flags:	-----

Size:	byte, word, long

This instruction moves the byte, word or long from the specified general

register to a location within the address space specified by the

destination function code (DFC) register, or it moves the byte, word or

long from a location within the address space specified by the source

function code (SFC) register to the general register.

If the destination is a data register, the source operand replaces the

low-order bits of that data register, depending on the size of the

operation. If the destination is a address register, the source operand

is sign-extended to 32-bits and then loaded into that register.

MOVES.B <ea>,D5   MOVES.W <ea>,D5   MOVES.L <ea>,D5

MOVES.B <ea>,A5   MOVES.W <ea>,A5   MOVES.L <ea>,A5

MOVES.B	D4,<ea>   MOVES.W D4,<ea>   MOVES.L D4,<ea>

MOVES.B A4,<ea>   MOVES.W A4,<ea>   MOVES.L A4,<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l

--------------------------------------------------------------------------------

MULS (signed multiplication)

Syntax:	MULS.W <ea>,Dn

MULS.L <ea>,Dl	    1	�

MULS.L <ea>,Dh:Dl   2	� {68020} {68030} {68040} {CPU32}

Flags:	-***0

Size:	word, long		(long is 68020, 68030, 68040, CPU32 only)

Multiplies two signed operands yielding a signed result. The instruction

uses one of three forms:

word	multiplies a word with a word. The result is a long word stored

in register Dn.

long 1	multiplies a long with a long. The result is a long word stored

in register Dl, the upper 32-bits of the result are discarded.

long 2	multiplies a long (register Dl) with a long. The result is

stored in Dh:Dl (in any two different data registers).

Dh:Dl cannot be the same register. Overflow can only occur if

multiplying two 32-bit operands to yield a 32-bit result.

MULS.W <ea>,D4

MULS.L <ea>,D4

MULS.L <ea>,D4:D5

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

MULU (unsigned multiplication)

Syntax:	MULU.W <ea>,Dn

MULU.L <ea>,Dl	    1	�

MULU.L <ea>,Dh:Dl   2	� {68020} {68030} {68040} {CPU32}

Flags:	-***0

Size:	word, long		(long is 68020, 68030, 68040, CPU32 only)

Multiplies two unsigned operands yielding an unsigned result. The

instruction uses one of three forms:

word	multiplies a word with a word. The result is a long word stored

in register Dn.

long 1	multiplies a long with a long. The result is a long word stored

in register Dl, the upper 32-bits of the result are discarded.

long 2	multiplies a long (register Dl) with a long. The result is

stored in Dh:Dl (in any two different data registers).

Dh:Dl cannot be the same register. Overflow can only occur if

multiplying two 32-bit operands to yield a 32-bit result.

MULU.W <ea>,D4

MULU.L <ea>,D4

MULU.L <ea>,D4:D5

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

NBCD (negate binary coded decimal)

Syntax:	NBCD <ea>

Flags:	*U*U*

Size:	byte

Subtracts the destination operand and the extend bit from zero, storing

the result at the destination. The operation is performed using binary

coded decimal arithmetic.

NBCD <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

NEG (negate)

Syntax:	NEG.? <ea>

Flags:	*****

Size:	byte, word, long

Subtracts the destination from zero, storing the result at the

destination.

NEG.B <ea>   NEG.W <ea>   NEG.L <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

NEGX (negate with extend)

Syntax:	NEGX.? <ea>

Flags:	*****

Size:	byte, word, long

Subtracts the destination and the extend bit from zero, storing the

result at the destination.

NEGX.B <ea>   NEGX.W <ea>   NEGX.L <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

NOP (no operation)

Syntax:	NOP

Flags:	-----

Size:	unsized

Performs no operation. The NOP instruction does not begin execution

until all pending bus cycles have completed. This synchronizes the

pipeline and prevents instruction overlap.

NOP

--------------------------------------------------------------------------------

NOT (logical complement)

Syntax:	NOT.? <ea>

Flags:	-**00

Size:	byte, word, long

Calculates the ones complement of the destination, storing the result at

the destination.

NOT.B <ea>   NOT.W <ea>   NOT.L <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

OR (logical inclusive OR)

Syntax:	OR.? <ea>,Dn

OR.? Dn,<ea>

Flags:	-**00

Size:	byte, word, long

ORs the source with the destination, storing the result in the

destination.

OR.B <ea>,D4   OR.W <ea>,D4   OR.L <ea>,D4

Dn (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

OR.B D4,<ea>   OR.W D4,<ea>   OR.L D4,<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ORI (logical inclusive OR, immediate)

Syntax:	ORI.? #imm,<ea>

OR.? #imm,<ea>

Flags:	-**00

Size:	byte, word, long

ORs the immediate value with the destination, storing the result in the

destination.

ORI.B #imm,<ea>   ORI.W #imm,<ea>   ORI.L #imm,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ORI to CCR (logical inclusive OR, immediate to condition codes)

Syntax:	ORI #imm,CCR

OR #imm,CCR

Flags:	X set if bit 4 of #imm is one, unchanged otherwise

N set if bit 3 of #imm is one, unchanged otherwise

Z set if bit 2 of #imm is one, unchanged otherwise

V set if bit 1 of #imm is one, unchanged otherwise

C set if bit 0 of #imm is one, unchanged otherwise

Size:	byte

ORs the immediate value with the condition codes, storing the result in

the condition code register.

ORI #imm,CCR

--------------------------------------------------------------------------------

ORI to SR (logical inclusive OR, immediate to status register) {priv}

Syntax:	ORI #imm,SR

OR #imm,SR

Flags:	X set if bit 4 of #imm is one, unchanged otherwise

N set if bit 3 of #imm is one, unchanged otherwise

Z set if bit 2 of #imm is one, unchanged otherwise

V set if bit 1 of #imm is one, unchanged otherwise

C set if bit 0 of #imm is one, unchanged otherwise

Size:	word

ORs the immediate value with the status register, storing the result in

the status register.

ORI #imm,SR

--------------------------------------------------------------------------------

PACK (pack) {68020} {68030} {68040}

Syntax:	PACK Dx,Dy,#adjustment

PACK -(Ax),-(Ay),#adjustment

Flags:	-----

Size:	unsized

Adjusts and packs the lower four bits of each of two bytes into a single

byte.

When both operands are data registers, the adjustment is added to the

value contained in the source register. Bits 11-8 and 3-0 of the

intermediate result are concatenated and placed in bits 7-0  of the

destination register. The remainder of the destination register is

unaffected.

When the predecrement addressing mode is specified, two bytes from the

source are fetched and concatenated. The adjustment word is added to the

concatenated bytes. Bits 0-3 of each byte are extracted. These eight

bits are concatenated to form a new byte, which is written to the

destination.

PACK D4,D5,#adjustment word

PACK -(A4),-(A5),#adjustment word

--------------------------------------------------------------------------------

PBcc (branch on PMMU condition) {68851} {priv}

PDBcc (test, decrement and branch on PMMU condition) {68851} {priv}

PFLUSH (flush ATC entries) {68030} {68040} {68551} {priv}

PLOAD (load an entry into the ATC) {68030} {68551} {priv}

PMOVE (move to/from MMU registers) {68030} {68551} {priv}

PRESTORE (PMMU restore function) {68551} {priv}

PSAVE (PMMU save function) {68551} {priv}

PScc (set on PMMU condition) {68551} {priv}

PTEST (test a logical address) {68030} {68040} {68551} {priv}

PTRAPcc (trap on PMMU condition) {68551} {priv}

PVALID (validate a pointer) {68551} {priv}

These instructions are not currently implemented because I am unsure of the

correct syntax.

--------------------------------------------------------------------------------

PEA (push effective address)

Syntax:	PEA <ea>

Flags:	-----

Size:	long

Computes the effective address and pushes it onto the stack. The

effective address is a long address.

PEA <ea>

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

RESET (reset external devices) {priv}

Syntax:	RESET

Flags:	-----

Size:	unsized

Asserts the RSTO signal for 512 (124 for MC68000, MC68EC000, MC68HC000,

MC68HC001, MC68008, MC68010 and MC68302) clock periods, resetting all

external devices.

RESET

--------------------------------------------------------------------------------

ROL (rotate left)

Syntax:	ROL.? Dx,Dy

ROL.? #count,Dy

ROL <ea>

Flags:	-**0*	(-**00 if count=0)

Size:	byte, word, long	(word if ROL <ea>)

Rotate the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using ROL <ea>. Bits rotated out of the high

order bit go to the carry and also back into the low order bit.

ROL.B D4,D5   ROL.W D4,D5   ROL.L D4,D5

ROL.B #count,D5   ROL.W #count,D5   ROL.L #count,D5

ROL <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ROR (rotate right)

Syntax:	ROR.? Dx,Dy

ROR.? #count,Dy

ROR <ea>

Flags:	-**0*	(-**00 if count=0)

Size:	byte, word, long	(word if ROR <ea>)

Rotate the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using ROR <ea>. Bits rotated out of the low

order bit go to the carry and also back into the high order bit.

ROR.B D4,D5   ROR.W D4,D5   ROR.L D4,D5

ROR.B #count,D5   ROR.W #count,D5   ROR.L #count,D5

ROR <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ROXL (rotate with extend, left)

Syntax:	ROXL.? Dx,Dy

ROXL.? #count,Dy

ROXL <ea>

Flags:	***0*

Size:	byte, word, long	(word if ROXL <ea>)

Rotate the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using ROXL <ea>. Bits rotated out of the high

order bit go to the carry and the extend; the previous value of the

extend rotates into the low order bit.

ROXL.B D4,D5   ROXL.W D4,D5   ROXL.L D4,D5

ROXL.B #count,D5   ROXL.W #count,D5   ROXL.L #count,D5

ROXL <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

ROXR (rotate with extend, right)

Syntax:	ROXR.? Dx,Dy

ROXR.? #count,Dy

ROXR <ea>

Flags:	***0*

Size:	byte, word, long	(word if ROXR <ea>)

Rotate the destination by the specified count (Dx modulo 64, or #count

from 1 to 8), or by 1 if using ROXR <ea>. Bits rotated out of the low

order bit go to the carry and the extend; the previous value of the

extend rotates into the high order bit.

ROXR.B D4,D5   ROXR.W D4,D5   ROXR.L D4,D5

ROXR.B #count,D5   ROXR.W #count,D5   ROXR.L #count,D5

ROXR <ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

RTD (return and deallocate) {68010} {68020} {68030} {68040} {CPU32}

Syntax:	RTD #displacement

Flags:	-----

Size:	unsized

Pulls the program counter value from the stack and adds the sign

extended 16-bit displacement value to the stack pointer. The previous

value of the program counter is lost.

RTD #displacement word

--------------------------------------------------------------------------------

RTE (return from exception) {priv}

Syntax:	RTE

Flags:	set according to the status register from the stack

Size:	unsized

{68000} {68008}

Pulls the status register and program counter values from the stack. The

previous values are lost.

{68010} {68020} {68030} {68040} {CPU32}

Loads the processor state information stored in the exception stack

frame located at the top of the stack into the processor. The

instruction examines the stack format field in the format/offset word

to determine how much information must be restored.

RTE

--------------------------------------------------------------------------------

RTM (return from module) {68020}

Syntax:	RTM Rn

Flags:	set according to the content of the word on the stack

Size:	unsized

A previously saved module state is reloaded from the top of stack. After

the module state is retrieved from the top of the stack, the caller's

stack pointer is incremented by the argument count value in the module

stack.

RTM Dn   RTM An

--------------------------------------------------------------------------------

RTR (return and restore condition codes)

Syntax:	RTR

Flags:	set to the condition codes from the stack

Size:	unsized

Pulls the condition code and program counter values from the stack. The

previous values are lost. The supervisor portion of the status register

is unaffected.

RTR

--------------------------------------------------------------------------------

RTS (return from subroutine)

Syntax:	RTS

Flags:	-----

Size:	unsized

Pulls the program counter from the stack. The previous program counter

value is lost.

RTS

--------------------------------------------------------------------------------

SBCD (subtract binary coded decimal)

Syntax:	SBCD Dx,Dy

SBCD -(Ax),-(Ay)

Flags:	*U*U*

Size:	byte

Subtracts the source and the extend bit from the destination, storing

the result in the destination. Subtraction is performed using binary

coded decimal (BCD) arithmetic.

SBCD D4,D5

SBCD -(A4),-(A5)

--------------------------------------------------------------------------------

Scc (set conditionally)

Syntax:	Scc <ea>

Flags:	-----

Size:	byte

If the specified condition is true, the byte at the effective address is

set to $FF. If false, the byte is cleared to $00.

conditions:-

SF		flase, clear to $00

ST		true, set to $FF

SCC	SHS	set if carry clear / higher or same (unsigned)

SCS	SLO	set if carry set / lower (unsigned)

SEQ		set if equal

SGE		set if greater or equal (signed)

SGT		set if greater than (signed)

SHI		set if higher than (unsigned)

SLE		set if less or equal (signed)

SLS		set if lower or the same (unsigned)

SLT		set if less than (signed)

SMI		set if minus

SNE		set if not equal

SPL		set if plus

SVC		set if overflow clear

SVS		set if overflow set

Scc <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

STOP (load status register and stop) {priv}

Syntax:	STOP #data

Flags:	set according to the immediate word

Size:	unsized

Moves the immediate word into the status register, advances the program

counter to the next instruction, and stops the fetching and executing of

instructions. A trace, interrupt, or reset exception causes the

processor to resume instruction execution. A trace exception occurs if

instruction tracing is enabled (T0=1, T1=0) when the STOP instruction

begins execution. If an interrupt request is asserted with a priority

higher than the priority level set by the new status register, an

interrupt exception occurs; otherwise, the interrupt request is ignored.

An external reset always initiates reset exception processing.

STOP #word

--------------------------------------------------------------------------------

SUB (subtract)

Syntax:	SUB.? <ea>,Dn

SUB.? Dn,<ea>

Flags:	*****

Size:	byte, word, long

SUBtracts the source from the destination, storing the result in the

destination.

SUB.B <ea>,D4   SUB.W <ea>,D4   SUB.L <ea>,D4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

SUB.B D4,<ea>   SUB.W D4,<ea>   SUB.L D4,<ea>

(An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

see also: SUBA SUBI SUBQ

--------------------------------------------------------------------------------

SUBA (subtract from address register)

Syntax:	SUBA.? <ea>,An

SUB.? <ea>,An

Flags:	-----

Size:	word, long

SUBtracts the source from the destination register, storing the result

in the destination address register. Word length operands are

sign-extended to 32-bits for subtraction.

SUBA.W <ea>,A4   SUBA.L <ea>,A4

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix)

mem.w mem.l #data d16(PC) d8(PC,Ix) ext(An) ext(PC)

--------------------------------------------------------------------------------

SUBI (subtract immediate)

Syntax:	SUBI.? #imm,<ea>

SUB.? #imm,<ea>

Flags:	*****

Size:	byte, word, long

SUBtracts the immediate value from the destination, storing the result

in the destination.

SUBI.B #imm,<ea>   SUBI.W #imm,<ea>   SUBI.L #imm,<ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

SUBQ (subtract quick)

Syntax:	SUBQ.? #imm,<ea>

Flags:	***** (----- if the destination is an address register)

Size:	byte, word, long (word, long if the destination is an address register)

SUBtracts the immediate value (from 1 to 8) from the destination,

storing the result in the destination. If the destination is an address

register, word length operands are sign-extended to 32-bits for

subtraction.

SUBQ.B #imm,<ea>   SUBQ.W #imm,<ea>   SUBQ.L #imm,<ea>

Dn An (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

SUBX (subtract with extend)

Syntax:	SUBX.? Dx,Dy

SUBX.? -(Ax),-(Ay)

Flags:	*****

Size:	byte, word, long

SUBs the source and the extend from the destination, storing the result

in the destination.

SUBX.B D4,D5   SUBX.W D4,D5   SUBX.L D4,D5

SUBX.B -(A4),-(A5)   SUBX.W -(A4),-(A5)   SUBX.L -(A4),-(A5)

--------------------------------------------------------------------------------

SWAP (swap register halves)

Syntax:	SWAP Dn

Flags:	-**00

Size:	word

Exchange the 16-bit words (halves) of a data register.

SWAP D4

--------------------------------------------------------------------------------

TAS (test and set an operand)

Syntax:	TAS <ea>

Flags:	-**00

Size:	byte

Tests and sets the byte operand addressed by the effective address. The

instruction tests the current value of the operand and sets the N and Z

condition codes appropriately. TAS then sets the high order bit of

the operand. The operation uses a locked or read-modify-write transfer

sequence. This instruction supports use of a flag or semaphore to

coordinate several processors.

TAS <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

--------------------------------------------------------------------------------

TBLS (table lookup and interpolate, signed, result rounded) {CPU32}

TBLSN (table lookup and interpolate, signed, result not rounded) {CPU32}

Syntax:	TBLS.? <ea>,Dx		result rounded

TBLS.? Dym:Dyn,Dx	result rounded

TBLSN.? <ea>,Dx		result not rounded

TBLSN.? Dym:Dyn,Dx	result not rounded

The TBLS and TBLSN instructions allow the efficient use of piecewise

linear compressed data tables to model complex functions. There are two

modes of operation; table lookup and interpolate mode and data register

interpolate mode.

TBLS.B <ea>,D5   TBLS.W <ea>,D5   TBLS.L <ea>,D5

TBLSN.B <ea>,D5   TBLSN.W <ea>,D5   TBLSN.L <ea>,D5

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

TBLS.B D4:D5,D6   TBLS.W D4:D5,D6   TBLS.L D4:D5,D6

TBLSN.B D4:D5,D6   TBLSN.W D4:D5,D6   TBLSN.L D4:D5,D6

--------------------------------------------------------------------------------

TBLU (table lookup and interpolate, unsigned, result rounded) {CPU32}

TBLUN (table lookup and interpolate, unsigned, result not rounded) {CPU32}

Syntax:	TBLU.? <ea>,Dx		result rounded

TBLU.? Dym:Dyn,Dx	result rounded

TBLUN.? <ea>,Dx		result not rounded

TBLUN.? Dym:Dyn,Dx	result not rounded

The TBLU and TBLUN instructions allow the efficient use of piecewise

linear compressed data tables to model complex functions. There are two

modes of operation; table lookup and interpolate mode and data register

interpolate mode.

TBLU.B <ea>,D5   TBLU.W <ea>,D5   TBLU.L <ea>,D5

TBLUN.B <ea>,D5   TBLUN.W <ea>,D5   TBLUN.L <ea>,D5

(An) d16(An) d8(An,Ix) mem.w mem.l d16(PC) d8(PC,Ix) ext(An) ext(PC)

TBLU.B D4:D5,D6   TBLU.W D4:D5,D6   TBLU.L D4:D5,D6

TBLUN.B D4:D5,D6   TBLUN.W D4:D5,D6   TBLUN.L D4:D5,D6

--------------------------------------------------------------------------------

TRAP (trap)

Syntax:	TRAP #vector

Flags:	-----

Size:	unsized

Causes a TRAP #vector exception. The instruction adds the immediate

operand (vector, 0-15) of the instruction to 32 to obtain the vector

number.

TRAP #vector (0-15)

--------------------------------------------------------------------------------

TRAPcc (trap on condition) {68020} {68030} {68040} {CPU32}

TRPcc (trap on condition) {68020} {68030} {68040} {CPU32} {!!!}

Syntax:	TRAPcc			TRPcc {!!!}

TRAPcc.W #data		TRPcc.W #data {!!!}

TRAPcc.L #data		TRPcc.L #data {!!!}

Flags:	-----

Size:	unsized, word, long

If the specified condition is true, causes a TRAPcc exception with a

vector number 7. The immediate data (word or long) is available to the

trap handler.

conditions:-

TRAPF		false, never trap

TRAPT		true, always trap

TRAPCC	TRAPHS	trap if carry clear / higher or same (unsigned)

TRAPCS	TRAPLO	trap if carry set / lower (unsigned)

TRAPEQ		trap if equal

TRAPGE		trap if greater or equal (signed)

TRAPGT		trap if greater than (signed)

TRAPHI		trap if higher than (unsigned)

TRAPLE		trap if less or equal (signed)

TRAPLS		trap if lower or the same (unsigned)

TRAPLT		trap if less than (signed)

TRAPMI		trap if minus

TRAPNE		trap if not equal

TRAPPL		trap if plus

TRAPVC		trap if overflow clear

TRAPVS		trap if overflow set

TRAPEQ

TRAPEQ.W #word

TRAPEQ.L #long

--------------------------------------------------------------------------------

TRAPV (trap if overflow)

Syntax:	TRAPV

Flags:	-----

Size:	unsized

If the overflow condition code is set, a TRAPV exception with a vector

number 7 occurs.

TRAPV

--------------------------------------------------------------------------------

TST (test an operand)

Syntax:	TST.? <ea>

Flags:	-**00

Size:	byte, word, long

Compares the operand with zero and sets the condition codes accordingly.

TST.B <ea>   TST.W <ea>   TST.L <ea>

Dn (An) (An)+ -(An) d16(An) d8(An,Ix) mem.w mem.l ext(An)

An #data d16(PC) d8(PC,Ix) ext(PC)   {68020} {68030} {68040} {CPU32}

--------------------------------------------------------------------------------

UNLK (unlink)

Syntax:	UNLK An

Flags:	-----

Size:	unsized

Loads the stack pointer from the specified address register, the loads

the address register with the long word pulled from the top of the

stack.

UNLK A4

--------------------------------------------------------------------------------

UNPK (unpack BCD) {68020} {68030} {68040}

UNPACK (unpack BCD) {68020} {68030} {68040}

Syntax:	UNPK Dx,Dy,#adjustment

UNPK -(Ax),-(Ay),#adjustment

Flags:	-----

Size:	unsized

Places the two binary-coded decimal digits in the source operand byte

into the lower four bits of two bytes and places zero bits in the upper

four bits. Adds the adjustment word to this unpacked value.

When both operands are data registers, the instruction unpacks the

source register contents, adds the adjustment word, and places the

result in the destination register. The high word of the destination is

not affected.

When the addressing mode is predecrement, the instruction extracts two

binary-coded decimal digits from a byte at the source address. After

unpacking the digits and adding the adjustment word, the instruction

writes the two bytes to the destination address.

UNPK D4,D5,#adjustment word

UNPK -(A4),-(A5),#adjustment word

������������������������������������������������������������������������������Ŀ

�                          Unimplemented Instructions                          �

��������������������������������������������������������������������������������

The following instructions are not currently implemented, because I am unsure of

the correct syntax :-

cpBcc (branch on coprocessor condition) {68020} {68030}

cpDBcc (test coprocessor condition, decrement and branch) {68020} {68030}

cpGEN (coprocessor general function) {68020} {68030}

cpScc (set on coprocessor condition) {68020} {68030}

cpTRAPcc (trap on coprocessor condition) {68020} {68030}

cpRESTORE (coprocessor restore functions) {68020} {68030} {priv}

cpSAVE (coprocessor save function) {68020} {68030} {priv}

CINV (invalidate cache entries) {68040} {68LC040} {priv}

CPUSH (push and invalidate cache entries) {68040} {68LC040} {priv}

FRESTORE (restore internal floating point state)

{68881} {68882} {68040} {priv}

FSAVE (save internal floating point state)

{68881} {68882} {68040} {priv}

PBcc (branch on PMMU condition) {68851} {priv}

PDBcc (test, decrement and branch on PMMU condition) {68851} {priv}

PFLUSH (flush ATC entries) {68030} {68040} {68551} {priv}

PLOAD (load an entry into the ATC) {68030} {68551} {priv}

PMOVE (move to/from MMU registers) {68030} {68551} {priv}

PRESTORE (PMMU restore function) {68551} {priv}

PSAVE (PMMU save function) {68551} {priv}

PScc (set on PMMU condition) {68551} {priv}

PTEST (test a logical address) {68030} {68040} {68551} {priv}

PTRAPcc (trap on PMMU condition) {68551} {priv}

PVALID (validate a pointer) {68551} {priv}

Also, floating point coprocessor instructions are not currently implemented.

������������������������������������������������������������������������������ͻ

�                                Address Modes                                 �

������������������������������������������������������������������������������ͼ

68000 Addressing Modes

Dn	D0-D7	data register direct

An	A0-A7	address register direct

(An)		address register indirect

(An)+		address register indirect, with postincrement

-(An)		address register indirect, with predecrement

d16(An)		address register indirect, with word displacement

d8(An,Ix)	address register indirect, with index and byte displacement

d16(PC)		program counter indirect, with word displacement

d8(PC,Ix)	program counter indirect, with index and byte displacement

mem.w		absolute data addressing short

mem.l		absolute data addressing long

#imm		immediate data

Ix :- the index register must have a size (.W or .L)

������������������������������������������������������������������������������Ŀ

�             Extended Addressing Modes (68020,68030,68040,CPU32)              �

��������������������������������������������������������������������������������

The 68020, 68030, 68040 and CPU32 have an extended address mode capacity.

Here follows a list of all the new address modes. There are some duplicates with

the original address modes, with the original used instead of the new in these

cases. Examples of unique new address modes are given.

{offset:width}	specify bit field (not CPU32)

bd	base displacement	� default to word if no size

od	outer displacement	� is given (.w or .l)

Ix	index register D0-D7/A0-A7, size must be given (.w or .l),

scale is optional (*1 *2 *4 *8)

An	�

PC	� base register

zPC	�

zPC (zero program counter, or suppress program counter) example :- (zPC,100) and

(100) are assembled differently, but operate in exactly the same way.

using PC without a base displacement will generate a warning.

--------------------------------------------------------------------------------

No Memory Indirect Action Mode

(base displacement, base register, index register)

(bd)		(An)		(Ix)

(PC)

(zPC)

(bd,An)		(bd,Ix)		(An,Ix)

(bd,PC)				(PC,Ix)

(bd,zPC)			(zPC,Ix)

(bd,An,Ix)

(bd,PC,Ix)

(bd,zPC,Ix)

examples:-

(PC)		(PC)					no offset

(zPC)		(zPC)

(Ix)		(D0.w)  (D0.l)  (A0.l*4)

(bd,An)		100.l(A0)  (100.l,A0)  (A0,100.l)	long offset

(bd,PC)		mem.l(PC)  (mem.l,PC)			long offset

(bd,zPC)	100(zPC)  (100,zPC)

(bd,Ix)		100(D0.w)  (100,D0.l)  (A0.l*4,100)

(PC,Ix)		(PC,D0.w)  (D0.l,PC)  (PC,A0.l*4)	no offset

(zPC,Ix)	(zPC,D0.w)  (D0.l,zPC)  (zPC,A0.l*4)

(bd,An,Ix)	100.w(An,D0.w)  (100.l,A0,D0.l)		word/long offset

(bd,PC,Ix)	mem.w(PC,D0.w)  (mem.l,PC,D0.l)		word/long offset

(bd,zPC,Ix)	100.w(zPC,D0.w)  (100.l,zPC,D0.l)

--------------------------------------------------------------------------------

Memory Indirect With Preindex / Index Suppressed (not CPU32)

([base displacement, base register, index register], outer displacement)

([bd])		([An])		([Ix])		([],od) ***

([PC])

([zPC])

([bd,An])	([bd,Ix])	([bd],od)

([bd,PC])

([bd,zPC])

([An,Ix])	([An],od)	([Ix],od)

([PC,Ix])	([PC],od)

([zPC,Ix])	([zPC],od)

([bd,An,Ix])	([bd,An],od])	([bd,ix],od)	([An,ix],od)

([bd,PC,Ix])	([bd,PC],od])			([PC,ix],od)

([bd,zPC,Ix])	([bd,zPC],od])			([zPC,ix],od)

([bd,An,Ix],od)

([bd,PC,Ix],od)

([bd,zPC,Ix],od)

examples:-

([bd])		([100.w])  ([100.l])

([An])		([A0])

([PC])		([PC])

([zPC])		([zPC])

([Ix])		([D0.w])  ([D0.l])  ([A0.l*4])

([bd,An])	([100,A0])  ([100.w,A0])  ([100.l,A0])

([bd,PC])	([mem,PC])  ([mem.w,PC])  ([mem.l,PC])

([bd,zPC])	([100,zPC])  ([100.w,zPC])  ([100.l,zPC])

([bd,Ix])	([100,D0.w])  ([100.w,D0.l])  ([100.l,A1.w*4])

([bd],od)	([100],200)  ([100.w],200.l)  ([100.l],200.w)

([An,Ix])	([A0,D0.w])  ([A0,D0.l])  ([A0,A1.w*4])

([PC,Ix])	([PC,D0.w])  ([PC,D0.l])  ([PC,A1.w*4])

([zPC,Ix])	([zPC,D0.w])  ([zPC,D0.l])  ([zPC,A1.w*4])

([An],od)	([A0],200)  ([A0],200.w)  ([A0],200.l)

([PC],od)	([A0],200)  ([A0],200.w)  ([A0],200.l)

([zPC],od)	([zPC],200)  ([zPC],200.w)  ([zPC],200.l)

([Ix],od)	([D0.w],200)  ([D0.l],200.w)  ([A1.w*4],200.l)

([bd,An,Ix])	([100,A0,D0.w])  ([100.w,A0,D0.l])  ([100.l,A0,A1.w*4])

([bd,PC,Ix])	([mem,PC,D0.w])  ([mem.w,PC,D0.l])  ([mem.l,PC,A1.w*4])

([bd,zPC,Ix])	([100,zPC,D0.w])  ([100.w,zPC,D0.l])

([100.l,zPC,A1.w*4])

([bd,An],od)	([100,A0],200)  ([100.w,A0],200.l)  ([100.l,A0],200.w)

([bd,PC],od)	([mem,PC],200)  ([mem.w,PC],200.l)  ([mem.l,PC],200.w)

([bd,zPC],od)	([100,zPC],200)  ([100.w,zPC],200.l)

([100.l,zPC],200.w)

([bd,Ix],od)	([100,D0.w],200)  ([100.w,D0.l],200.l)

([100.l,A1.w*4],200.w)

([An,Ix],od)	([A0,D0.w],200)  ([A0,D0.l],200.w)  ([A0,A1.w*4],200.l)

([PC,Ix],od)	([PC,D0.w],200)  ([PC,D0.l],200.w)  ([PC,A1.w*4],200.l)

([zPC,Ix],od)	([zPC,D0.w],200)  ([zPC,D0.l],200.w)

([zPC,A1.w*4],200.l)

([bd,An,Ix],od)	([100,A0,D0.w],200)  ([100.l,A0,D0.l],200.w)

([100.w,A0,A1.w*4],200.l)

([bd,PC,Ix],od)	([mem,PC,D0.w],200)  ([mem.l,PC,D0.l],200.w)

([mem.w,PC,A1.w*4],200.l)

([bd,zPC,Ix],od) ([100,zPC,D0.w],200)  ([100.l,zPC,D0.l],200.w)

([100.w,zPC,A1.w*4],200.l)

--------------------------------------------------------------------------------

Memory Indirect With Postindex (not CPU32)

([base displacement, base register], index register, outer displacement)

([],Ix) ***

([bd],Ix)	([An],Ix)	([],Ix,od) ***

([PC],Ix)

([zPC],Ix)

([bd,An],Ix)	([bd],Ix,od)	([An],Ix,od)

([bd,PC],Ix)	  		([PC],Ix,od)

([bd,zPC],Ix)			([zPC],Ix,od)

([bd,An],Ix,od)

([bd,PC],Ix,od)

([bd,zPC],Ix,od)

examples:-

([bd],Ix)	([100],D0.w)  ([100.w],D0.l)  ([100.l],A1.w*4)

([An],Ix)	([A0],D0.w)  ([A0],D0.l)  ([A0],A1.w*4)

([PC],Ix)	([PC],D0.w)  ([PC],D0.l)  ([PC],A1.w*4)

([zPC],Ix)	([zPC],D0.w)  ([zPC],D0.l)  ([zPC],A1.w*4)

([bd,An],Ix)	([100,A0],D0.w)  ([100.w,A0],D0.l)  ([100.l,A0],A1.w*4)

([bd,PC],Ix)	([100,PC],D0.w)  ([100.w,PC],D0.l)  ([100.l,PC],A1.w*4)

([bd,zPC],Ix)	([100,zPC],D0.w)  ([100.w,zPC],D0.l)

([100.l,zPC],A1.w*4)

([bd],Ix,od)	([100],D0.w,200)  ([100.w],D0.l,200.l)

([100.l],A1.w*4,200.w)

([An],Ix,od)	([A0],D0.w,200)  ([A0],D0.l,200.l)  ([A0],A1.w*4,200.w)

([PC],Ix,od)	([PC],D0.w,200)  ([PC],D0.l,200.l)  ([PC],A1.w*4,200.w)

([zPC],Ix,od)	([zPC],D0.w,200)  ([zPC],D0.l,200.l)

([zPC],A1.w*4,200.w)

([bd,An],Ix,od)	([100,A0],D0.w,200)  ([100.l,A0],D0.l,200.w)

([100.w,A0],A1.w*4,200.l)

([bd,PC],Ix,od)	([mem,PC],D0.w,200)  ([mem.l,PC],D0.l,200.w)

([mem.w,PC],A1.w*4,200.l)

([bd,zPC],Ix,od) ([100,zPC],D0.w,200)  ([100.l,zPC],D0.l,200.w)

([100.w,zPC],A1.w*4,200.l)

--------------------------------------------------------------------------------

How Indirect Addressing Modes ('[]') Work (not CPU32)

the expression between [] is evaluated, a long from that memory address is read,

and the rest of the expression is added (I assume this is how it works, I have

never been able to test it).

example:	A0 = 1000, D0 = 2000

memory address at 9100 contains 25000

memory address at 1100 contains 75000

preindex :- ([100.w,A0,D0.l*4],200.l)

100.w + A0 + D0.l*4 = 100 + 1000 + 8000 (2000*4) = 9100

[9100] = 25000, + 200 = 25200

memory address = 25200

postindex :- ([100.w,A0],D0.l*4,200.l)

100.w + A0 = 100 + 1000 = 1100

[1100] = 75000, + 8000 (D0.l*4, 2000*4) + 200 = 83200

memory address = 83200

������������������������������������������������������������������������������ͻ

�                         Acknowledgments, Disclaimer                          �

������������������������������������������������������������������������������ͼ

The Motorola M68000 Family Programmer's Reference Manual is highly recommended

for detailed information about the instruction sets.

All copyrights are acknowledged.

This file is part of the CrossFire package. Feel free to modify this file for

your own use, but if you distribute it, please distribute the whole unmodified

demonstration package (do NOT distribute the registered version). Add files if

you like, but try to keep them in the 'EXTRA' directory.

Disclaimer

To the best of my knowledge the information contained in this file is accurate.

However, the user takes full liability for any damage caused by the use or

misuse of software, documentation and information in the CrossFire package.