Slick is a Copy(c)right 1985,1994 by Gary Raymond, New Orleans, La.

Slick Modem Terminal Program V6.5a

Tech Help 504-288-6550

SOLVING PROBLEMS:

NO RESPONSE FROM MODEM:

If nothing at all appears on your screen, (after the LoGo

disappears and you hear the initialization beep) the fault is

likely that SLICK's default port setting (COM2) is incorrect for

your computer/modem setup. You will need to set another COM port.

The COM menu is selected by hitting Alt N (while still in regular

terminal mode) then selecting port 1, 3 or 4 depending on your

available ports. SLICK will attempt to test the port at the time

you select it, and inform you if the port is not there (or

incorrectly set on your modem card).

�����������������������������������������������ͻ

�      �����ĴSLICK Comm Port Menu�������       �

�              Port Now = COM2                  �

�       COM[1] COM[2] COM[3] COM[4] ->2         �

�             Esc or Cr to Exit!                �

�����������������������������������������������ͼ

If you need to customize a communications port, use Alt-Q while in

the default menu. The port address must be entered in decimal

form. Interrupts (IRQ's) are limited in selection from 2 to 7.

Example: Suppose you need to use IRQ 7 on COM port ONE. The hex

address of port 1 is 3F8. Converted to decimal it is 1016. Once

SLICK has been customized for a special pairing of port and IRQ,

the normal IBM default ports will be in operational and the opening

screen will remind you that you are in custom port mode. The

decimal addresses for the IBM PS\2 are listed in the self

displaying serial information text file called RS232DAT.COM

To test your new port selection, simply type ATZ at the terminal

prompt looking for both the letters to be echoed on the screen

along with an OK (or "0") reply from the modem. Once you have the

correct IBM port selected you will want to save it into the EXE

module so it will remain the default next time you run SLICK. This

is done via the default menu brought up with the Alt Z key

combination. One inside the defaults menu, use Alt P to write the

new port into EXE module. On a 4.77 mhz floppy only system, this

may take 10-20 seconds. On hard drives with fast processors, less

than a second. You may also wish to change the baud rate, parity,

data & stopbit and or modem string by this same general method.

You may also set the UpLoad and DnLoad disk/directory locations

and the default editor for SLICK's  dialing directory. SLICK comes

with SLICKED which is pre defined as the default editor. Any other

favorite text editor will work also. The Alt-Z (Defaults) Menu is

also used to change the hot key macros. (Hit F5 to see whats there

now.)

SLICK WONT HANG UP

A common question is "everything seems to be working ok but the

modem wont hang up the line after a good connect, Why?" Some

modems depend 100% on software control of important functions like

carrier and data state. Usually, "&C1" (less quotes) added to the

string will allow the modem to hang up.

WON'T DIAL USING DIAL DIRECTORY:

Some older modems require you set the character used to identify

a carriage return. SLICK uses ascii 13 (IBM default). Set S3=13

if a selection from the dial directory does not appear to dial

the modem but a manual entry in terminal modes works ok.

REPEAT DIALER ONLY DIALS ONCE:

SLICK depends on full voice replies from the modem. Example:

'BUSY' is required to enable the repeat dialer (F9) to function

correctly. This is usually controlled by setting the V and X

registers in the modem setup string (stored in macro Alt-0). Check

your modem manual for exact details. V1 and X4 usually command

most modern modems to spit out everything. Modems generally use V0

to request numeric replies and V1 for voice. Perhaps, if I get the

time, at some later date I will code SLICK to operate with either

voice or numeric responses; but, for now it MUST be voice responses.

When I dial into TeleNet, TymNet and Compuserve, I only get

garbage character, why?

These old networks are still using 7 data bits and even parity. You

must change the data bits from the default of 8 and no parity to

7 and even parity. This is done via Alt-F7 while in the terminal

mode. The line status menu that will appear is shown below.

Select number 3, then logon again.

��������������������������������������ͻ

�  * SLICK Parity/Data/StopBit Menu *  �

�            P/D/S now = 8N1           �

�   [1]=8N1 [2]=8N2 [3]=7E1 [4]=7E2    �

�   [5]=7O1 [6]=7O2 [7]=7M1 [8]=7S1    �

�  (Default=1), Select or Escape ->1   �

��������������������������������������ͼ

At any time you need a reminder of your various current settings,

hit F7 and the following line will replace line 25. Hit it again

and the original line 25 will return.

Port = COM 2 Line = 8N1 1200 bd AltC=Clr Printer Off   Echo Off     Log Off

USING ANSI COLOR/GRAPHICS:

SLICK incorporates its own internal ansi device driver. This

eliminates having to install ANSI.SYS in your DOS CONFIG file.

SLICK's internal driver will also respond correctly to any BBS

that interrogates your terminal for ANSI color capability.

----------------------------------[EOF]-----------------------------------